Stream Buffer Process for Timber Suitability Analysis

(process recaptured by sbryan on 03.07.22)

- Identify perennial and intermittent streams from USGS NHD

  - Most recent NHD uses inconsistent mapping (disclaimer on USGS website); backed up several versions to improve consistency (2012 NHD) across NP

  - Clip to NP ownership

  - USGS NHD f-codes used to identify intermittent and perennial channels (categorization still inconsistent but improved. NHD is a computer model and has not been 100% ground-truthed)

    - 46003 = intermittent

    - 46006 = perennial

    - 46007 = ephemeral

- Buffer intermittent flowlines by 50’ and dissolve into polygon. Calculate area.

  - This was originally done with smaller buffers (15’) and updated to include most recent planning framework (50’).

- Buffer perennial flowlines by 100’ and dissolve into polygon. Calculate area.

- Merge buffered intermittent and perennial flowlines into single polygon. Recalculate area.

  - This is what is used in Timber Suitability Analysis (not a spatial process but uses spatial data for the math calculations).

  - This shapefile does not represent what is recognized as streamside forests on the NP—the planning framework is to be applied at the project level, as appropriate.