FSH 2509.22 - SOIL AND WATER CONSERVATION PRACTICES HANDBOOK

R-1/R-4 AMENDMENT NO. 1

EFFECTIVE 05/88

CHAPTER 10 - SOIL AND WATER CONSERVATION PRACTICES DOCUMENTATION

Contents

10.10     Introduction

10.20     Development Considerations

10.30     Application

10.40     Feedback Mechanism

10.50     Format

11        WATERSHED MANAGEMENT

11.01     Determination of Cumulative Watershed Effects

11.02     Soil and Water Resource Monitoring and Evaluation

11.03     Watershed Improvement Planning and Implementation

11.04     Floodplain Analysis and Evaluation

11.05     Wetlands Analysis and Evaluation

11.06     Public Supply Watershed Management

11.07     Oil and Hazardous Substance Spill Contingency Planning

11.08     Control of Activities Under Special use Permit

11.09     Management by Closure to use

11.10     Water Well Construction and Management

11.11     Petroleum Storage and Delivery Facilities Management

11.12     Administrative Site Planning and Management

11.13     Sanitary Guidelines for Construction of Temporary

Labor, Spike, Logging, and Fire Camps, and Similar

Installations

11.14     Management of Snow Survey Sites

12        RECREATION

12.01     Recreation Facilities Planning

12.02     Monitoring and Evaluation of Water Quality at

Designated Swimming Sites

12.03     Sanitary Surveys to Augment and Evaluation of

Designated Swimming Areas

12.04     Providing Safe Drinking Water Supplies

12.05     Documentation of Potable Water Quality Data

12.06     Management of Sanitation Facilities

12.07     Control of Refuse Disposal

12.08     Assuring Proper Sanitation and Water Supplies for

Special Use Facilities

12.10     Management of Off-Road Vehicle Use

12.11     Protection of Water Quality Within Developed and

Dispersed Recreation Areas

12.12     Location of Pack and Riding Stock Facilities in

Wilderness, Primitive, and Backcountry Areas

13        VEGETATION MANIPULATION

13.01     Operating Seeding and Land Preparation Equipment on the

Contour

13.02     Slope Limitations for Tractor Operation

13.03     Tractor Operation Excluded from Wetlands, Bogs, and Wet

Meadows

13.04     Revegetation of Surface Disturbed Areas

13.05     Soil protection During and After Slash Windrowing

13.06     Soil Moisture Limitations for Tractor Operation

13.07     Planning and use of pesticides

13.08     Apply Pesticides According to Label and EPA

Registration Directions

13.09     Pesticide Application Monitoring and Evaluation

13.10     Pesticide Spill Contingency Plan

13.11     Cleaning and Disposal of pesticide Containers and

Equipment

13.12     Protection of Water, Wetlands, and Riparian Areas

During pesticide Spraying

13.13     Controlling pesticide Drift During Spray Application

14        TIMBER

14.01     Timber Sale Planning

14.02     Timber Harvest Unit Design

14.03     Use of Sale Area Maps for Designating Soil and Water

Protection Needs

14.04     Limiting the Operating Period of Timber Sale Activities

14.05     Protection of Unstable Areas

14.06     Riparian Area Designation

14.07     Determining Tractor Loggable Ground

14.08     Tractor Skidding Design

14.09     Suspended Log yarding in Timber harvesting

14.10     Log Landing Location and Design

14.11     Log Landing Erosion Prevention and Control

14.12     Erosion Prevention and Control Measures During Timber

Sale Operations

14.13     Special Erosion Prevention Measures on Area Disturbed

by Harvest Activities

14.14     Revegetation of Areas Disturbed by Harvest Activities

14.15     Erosion Control on Skid Trails

14.16     Meadow Protection During Timber harvesting

14.17     Streamcourse Protection (Implementation and

Enforcement)

14.18     Erosion Control Structure Maintenance

14.19     Acceptance of Timber Sale Erosion Control measures

Before Sale Closure

14.20     Slash Treatment in Sensitive Areas

14.21     Non-recurring "C" provisions That Can Be Used for Water

Quality Protection

14.22     Modification of the Timber Sale Contract

14.23     Reforestation Requirement

15        ROADS AND TRAILS

15.01     General Guidelines for Transportation Planning

15.02     General Guidelines for the Location and Design of Roads

and Trails

15.03     Road and Trail Erosion Control Plan

15.04     Timing of Construction Activities

15.05     Slope Stabilization and Prevention of Mass Failures

15.06     Mitigation of Surface Erosion and Stabilization of

Slopes

15.07     Control of Permanent Road Drainage

15.08     Pioneer Road Construction

15.09     Timely Erosion Control Measures on Incomplete Road and

Streamcrossing Projects

15.10     Control of Road Construction Excavation and Sidecast

Material

15.11     Servicing and Refueling of Equipment

15.12     Control of Construction in Riparian Areas

15.13     Controlling In-Channel Excavation

15.14     Diversion of Flows Around Construction Sites

15.15     Streamcrossings on Temporary Roads

15.16     Bridge and Culvert Installation (Disposition of Surplus

Material and Protection of Fisheries)

15.17     Regulation of Borrow Pits, Gravel Sources and Quarries

15.18     Disposal of Right-of-Way and Roadside Debris

15.19     Streambank Protection

15.20     Water Source Development Consistent with Water Quality

Protection

15.21     Maintenance of Roads

15.22     Road Surface Treatment to Prevent Loss of Materials

15.23     Traffic Control During Wet periods

15.24     Snow Removal Controls

15.25     Obliteration of Temporary Roads

15.26     surface Erosion Control at Facility Sites

15.27     Trail Maintenance and Rehabilitation

16        MINERALS

16.01     Administration of the General Mining Law (Act of May

10, 1872) for Mineral Exploration and Extraction on

National Forest System Lands

16.02     Administration of Bureau of Land Management Issued

permits or Leases for Mineral Exploration and

Extraction on National Forest System Lands

16.03     Administration of Common Variety Mineral Operations

16.04     Permits and Administration of Geophysical Operations

16.05     Mineral Activity Coordination Analysis

16.06     Reclamation of Oil and Gas Well Sites

16.07     Reserve Pit Location, Design, Operation, and

Reclamation

16.08     Oil and Gas Well Blowout Contingency Plan

16.09     Abandoned Mine Land Reclamation

17        RANGE

17.01     Range Analysis, Allotment Management Plan, Grazing

Permit System, and Permittee Operating Plan

17.02     Controlling Livestock Numbers and Season of Use

17.03     Controlling Livestock Distribution

17.04     Rangeland Improvements

18        FIRE SUPPRESSION AND FUELS MANAGEMENT

18.01     Fire and Fuel Management Activities

18.02     Formulation of Fire Prescriptions

18.03     Protection of Soil and Water from Prescribed Burning

Effects

18.04     Minimizing Watershed Impacts from Fire Suppression

Efforts

18.05     Stabilization of Fire Suppression Related Watershed

Damage

18.06     Emergency Rehabilitation of Watershed Following

Wildfires

FSH 2509.22 - SOIL AND WATER CONSERVATION PRACTICES HANDBOOK

R-1/R-4 AMENDMENT NO. 1

EFFECTIVE 05/88

ZERO CODE

01 - AUTHORITY.  The Clean Water Acto of 1972 (Public Law

92-500), as amended in 1977 (Public Law 95-217) and 1987 (Public

Law 100-4), was intended by Congress to provide a means to

protect and improve the quality of the water resources and

maintain their beneficial uses.  The clean Water Act (Sections

208 and 319) recognized the need for control strategies for

nonpoint source pollution.  To provide environmental protection

and improvement emphasis for water and soil resources and

water-related beneficial uses, the National Nonpoint Source

Policy (December 12, 1984), the Forest Service Nonpoint Quality

Strategy (January 29, 1985), and the USDA Nonpoint Source Water

conservation practices were recognized as the primary control

mechanisms for nonpoint sources of pollution on National Forest

System lands.  This perspective is supported by the Environmental

Protection Agency (EPA) in their guidance, "Nonpoint Source

Controls and Water Quality Standards" (August 19, 1987).

Federal agency compliance with water pollution control mandates

is addressed through Section 313 of the Clean Water Act and in

Executive Order 12580 of January 23, 1987.  Agency compliance is

to be consistent with requirements that apply to "any

nongovernmental entity" or private person.  Compliance is to be

in line with "all Federal, State, interstate, and local

requirements, administrative authority, and process and sanctions

respecting the control and abatement of water pollution".  To

comply with State Water Quality Standards, the Forest Service is

required to apply water quality practices in State Forest

Practices Regulations, where applicable, reasonable land, soil,

and water conservation practices, or specialized best management

practices.  All these types of practices are designed with

consideration of geology, land type, soil type, erosion hazard,

climate, cumulative effects, and other factors in order to fully

protect and maintain soil, water, and water-related beneficial

uses, and to prevent or reduce nonpoint source pollution.

02 - OBJECTIVE.  The objective of this handbook si to present a

process to develop site specific conservation practices for use

on national Forest System lands to minimize effects of management

activities on soil and water resources, and to protect

water-related beneficial uses. It describes the application,

monitoring, evaluation, and adjustment of these conservation

practices.  This handbook is also to provide examples of soil and

water conservation practices which have been tested and have

provided protection in specific situations, and that can be

utilized or adapted in developing in developing site specific

conservation practices.  Additionally, this handbook is a

supplemental document to all Forest Plans.

03 - POLICY.  The Forest Service must be responsive to the

environmental intent and directives provided in the Clean Water

Act, as amended, State water quality goals and standards, and

other environmental legislation.  As part of its land stewardship

policy, the Forest Service's management actions must e carried

out in a manner which protects the soil and water resources. The

Forest Service will continue to coordinate all management actions

affecting water quality and beneficial uses with State water

quality agencies and will provide leadership in nonpoint source

pollution control for Forest Management.

In accordance with the Multiple Use-Sustained Yield Act and other

legislation (RPA, NFMA, and so forth), National Forest System

lands are to be managed for multiple uses,  Maintenance of soil

and water resources and protection of water-related beneficial

uses are among those multiple uses.  Use of soil and water

conservation practices are a means to ensure protection of those

resources and uses, while achieving other resource objectives.

Application of soil and water conservation practices translates,

in essence, to good land stewardship.

Pursuant to Section 208 of the Clean Water Act, all agencies

which are responsible for carrying out any portion of a State

Water Quality management Plan to minimize nonpoint source

pollution must be designated as a Water Quality management

Agency.  The Forest Service has been recognized as the Designated

Water Quality Management Agency for National Forest System lands

in the Northern and Intermountain Regions. This handbook has been

prepared to provide the Forest Service with a means to meet

requirements for obtaining and maintain this designation.

04 - RESPONSIBILITY.

1.  Regional Forester.  The Regional Forester shall:

a.  Provide program guidance in soil and water

conservation practices.

b.  Provide soil and water conservation practices

training and materials.

c.  Coordinate with appropriate State and other Federal

agencies involved in water quality regulations,

management of water-related beneficial uses, watershed

management, and best management practices.

d.  Monitor and evaluate on a regional basis the

implementation and effectiveness of soil and water

conservation practices.

e.  Notify the State of the result of monitoring and

evaluation.

2.  Forest Supervisor.  The Forest Supervisor shall:

a.  Train appropriate Forest personnel in development and

use of soil and water conservation practices.

b.  Coordinate with appropriate State and other Federal

agencies involved in water quality regulation, management

of water-related beneficial uses, watershed management,

and best management practices.

c.  Develop and improve soil and water conservation

practices with respect to changing technologies and

Forest Service direction.

d.  Notify the Regional Forester of any development or

improvement of soil and water conservation practices.

e.  Monitor the implementation and effectiveness of site

specific, soil and water conservation practices and

suggest any necessary adjustments.

3.  District Ranger.  The District Ranger shall:

a.  Develop and implement site specific, soil and water

conservation practices.

b.  Develop and implement site specific, soil and water

conservation practices on all resource management

project.

c.  Monitor the implementation and effectiveness of site

specific, soil and water conservation practices and

provide any necessary adjustments.

05 - DEFINITIONS.

1.  List of Abbreviations.

APD       -   Application for Permit to Drill

BLM       -   Bureau of Land Management

BMP       -   Best Management Practices

COR       -   Contracting Officer's Representative

CFR       -   Code of Federal Regulations

EA(R)     -   Environmental Assessment (Report)

EO        -   Executive Order

EPA       -   Environmental Protection Agency

EIS       -   Environmental Impact Statement

FAR       -   Federal Acquisition Regulations

FERC      -   Federal Energy Regulatory Commission

FS        -   Forest Service

FSH       -   Forest Service Handbook

FSM       -   Forest Service Manual

ID        -   Interdisciplinary

MOU       -   Memorandum of Understanding

NEPA      -   National Environmental Policy Act

NFMA      -   National Forest Management Act and/or

Regulations

ORV       -   Off Road Vehicle

R-1       -   Region 1 (Northern Region) of the USDA Forest

Service

R-4       -   Region 4 (Intermountain Region) of the USDA

Forest Service

RPA       -   Resource Planning Act

SA        -   Sale Administrator

SCS       -   Soil conservation Service

STORET    -   A storage and retrieval computer system for

water quality data administered by the

Environmental Protection Agency

SPCC      -   Spill Prevention Control and Counter Measure

SWCP      -   Soil and Water Conservation Practice

TSA       -   Timber Sale Administrator

2.  Glossary of Terms.

Area Transportation Plan.  A plan that identifies the

transportation facilities needed to manage the lands and

resources for a given area.

Baseline Data.  Data representative of a particular base

period or concurrent control sample.  Normally representative of

the undisturbed, undeveloped state.

Beneficial Use.  Actual or potential use that may be made of

the waters of the state, including but not necessarily limited to

domestic, municipal, agricultural, and industrial supply; power

generation; recreation; esthetic enjoyment; navigation;

preservation and enhancement of fish, wildlife and other aquatic

resources.

Best Management Practice (BMP).  A practice or combination of

practices, that is determined by a State (or designated area-wide

planning agency) after problem assessment, examination of

alternative practices, and appropriate public participation to be

the most effective, practical (including technological, economic,

and institutional considerations) means of preventing or reducing

the amount of pollution generated by nonpoint sources to a level

compatible with water quality goals (40 CFR 130.2(q)).

Best Management practices as defined by State regulation or

agreement between the State and Forest Service inlcude the

following by state:

In Idaho:

-  Idaho Forest Practices Rules and Regulations.

-  Rules and Regulaitons and Minimum Standards for

StreamChannel Alternations.

-  Idaho Best Management Practices for Road Activities

Handbook, Parts I and II.

In Montana:

-  Best Management Practices for the Flathead River from the

Flathead Drainage 208 Project, Appendix A (Only applicable

to the Flathead National Forest).

-  Best Management Practices (BMP's) for Forestry in Montana.

In Nevada:

-  Handbook of Best Management Practices (Nondesignated Area,

Water Quality Management Plan).

In Utah:

-  Best Management Practices for Nonpoint Source Pollution

Reduction in Waters of Summit, Wasatch, and Utah Counties.

-  Best Management Practices for Nonpoint Source Water

Pollution Control in Utah.

-  Salt Lake County Water Quality and Pollution Control:

Erosion-Sediment Control Handbook.

-  Land Use vs. Water Quality - Wasatch Streams.

Carrying Capacity.  (Recreation):  the amount of recreation

use an area can sustain without deterioration of site quality;

(Wildlife):  the maximum number of animals an area can support

during a given period of the year; (Range):  the maximum stocking

rate possible without damaging the vegetation or related

resources.  Carrying capacity may vary from year to year on the

same area due to fluctuating forage production.

Contract Provisions.  Controls, constraints, and/or general

direction included in Contracts offered by the Forest Service.

Cross Drain/Ditch.  A man-made ditch or channel constructed

to intercept surface water runoff and divert it before the runoff

concentrates to erosive volumes and velocities.

Crowning.  Forming a convex road surface which allows runoff

to drain from the running surface to both sides of the road

prism.

Cumulative Effect.  The impact on the environment which

results from the incremental impact of the action when added to

other past, present, and reasonable foreseeable future actions

regardless of what agency or person undertakes such other action

(40 CFR 1508.7).

Degraded Watershed.  A basin which has suffered environmental

damage, resulting in accelerated soil or vegetative loss or

chemical contamination to the quantifiable detriment of other

resources.

Designated Streams.  A stream or portion of a stream

identified as warranting special consideration in management

decisions and project activities.  See also Stream or Stream

course.

Floodplain.  The lowland and relatively flat areas adjoining

inland waters that are covered by its waters during flooding.

Hazardous Substances.  Materials which by their nature are

toxic or dangerous to handle or dispose of, such as radioactive

materials, petroleum products, pesticides, chemicals, and

biological wastes.

In-Service.  Pertains to activities, actions,or personnel

within the USDA Forest Service.

Interdisciplinary Team.  A group of individuals from various

disciplines or with different skills appropriate to resolve the

issues or problems identified.  The team is assembled out of

recognition that no one scientific discipline is sufficiently

broad to adequately solve the identified issue or problem.  The

members of the team proceed to solution with frequent

interaction, so that each discipline may provide insights to any

stage of the problem and disciplines may combine to provide new

solutions.

Line Officer.  Management personnel within the Forest Service

Organization consisting of:  Secretary of Agriculture, Chief or

Forest Service, Regional Foresters, Forest Supervisors, and

District Rangers,  Refers to the line of authority and

responsibility.

Log Landing.  An area where logs are skidded or yarded prior

to loading and transportation to a mill.

Mitigate.  To offset or lessen real or potential impacts or

effects through the application of additional controls or

actions.  Counter measures are employed to reduce or eliminate

undesirable or unwanted results.

NEPA Process.  All measures necessary for compliance with the

requirements of section 2 and Title I of the National

Environmental Protection Act (NEPA).

Nonpoint Source Pollution.  Diffuse sources of water

pollution that originate from many indefinable sources and

normally include agricultural and urban runoff, runoff from

construction activities, and so forth.  In practical terms,

nonpoint sources do not discharge at a specific, single location

(such as a single pipe).  Nonpoint source pollutants are

generally carried over or through the soil and ground cover via

stormflow processes.  Unlike point sources of pollution (such as

industrial and municipal effluent discharge pipes), nonpoint

sources are diffuse and can come from any land area.  The

following silvicultural activities are considered to be nonpoint

sources of pollution: nursery operations, site preparation,

reforestation and subsequent cultural treatment, thinning,

prescribed burning, pest and fire control, harvest operations,

surface drainage, and road construction and maintenance from

which there is natural runoff (40 CFR 122.27).

Normal Operating Season.  A portion of a year when normal

timber harvesting operations are expected to take place

uninterrupted by adverse weather conditions.

Outsloping.  Shaping a road to cause drainage to flow toward

the outside shoulder (generally the fill slope), as opposed to

insloping which encourages drainage to flow to the inside

shoulder (generally the cut slope).  Emphasis is on avoiding

concentrated water flow.

Permittee.  Individual or entity that has received a grazing

or Special Use Permit from the Forest Service.

Pesticide.  A general term applied to a variety of chemical

materials including insecticides, herbicides, fungicides, and

rodenticides.

Point Source.  Originating from a discrete identifiable

source or conveyance.  Silvicultural point sources of pollution

include the following: rock crushing, gravel washing, and log

sorting and storage facilities where water is applied

intentionally to the logs (40 CFR 122.27).

Purchaser.  The entity which is awarded a USDA Forest Service

contract after bidding, usually with competition.  As used in

timber, the entity which has purchased timber as identified in a

timber sale contract.

Reclamation.  Restabilization of land denuded by land

management activites.

Reforestation.  The replacement of vegetative cover which has

been harvested or lost due to natural occurrences.  Accomplished

either through planting of nursery stock or seeding, or through

natural processes.

Revegetation.  The replacement of vegetative cover which has

been harvested or lost due to natural occurrences.  Accomplished

either through planting of nursery stock or seeding, or through

natrual processes.

Riparian Areas.  Geographically delineable areas with

distinctive resource values and characteristics that are

comprised of the aquatic and riparian ecosystems.

Riparian Ecosystems.  A transition between the aquatic

ecosystem and the adjacent terrestrial ecosystem; identified by

soil characteristics or distinctive vegetation communities that

require free or unbound water.

Rip Rapping.  The use of large rock, boulders, concrete

chunks, or similar non-erosive, heavy objects as an armoring

device.

Road Maintenance Plan.  A documented schedule and program for

upkeep of roads to provide a level of service for the user and

protection of resources.  There are five levels of maintenance:

Level I is the least intense and Level V the most intensive.

Rocking.  The application of aggregate to a roadbed to

provide strength and a more stable erosion resistent surface.

Sale Area Map.  A map of suitable scale and detail to be

legible which is part of a timber sale contract.  The map

identifies sale area boundaries and contract requirements

specific to the sale.

Significant Disturbance.  Disturbance of surface resources,

including soil, water and vegetation, which has the potential to

degrade water quality to a level requiring corrective action.

Site Preparation.  A general term for removing unwanted

vegetation, slash, and even roots and stones from a site before

reforestation.  It is generally accomplished by either

mechanical, chemical, or biological means, or controlled fire.

Site Specific.  Pertains to a discernible, definable area or

point on the ground where a project or activity will (or is

proposed) to occur.

Soil and Water Conservation Practices (SWCP).  The set of

practices which, when applied during implementation of a project,

ensures that soil productivity is maintained, soil loss and water

quality impacts are minimized, and water-related beneficial uses

are protected.  These practices can take several forms.  Some are

defined by State regulation or memoranda of understanding between

the Forest Service and the States and thus are recognized as Best

Management Practices (BMP's).  Others are defined by the Forest

interdisciplinary teams or described in Forest Service manuals

and Handbooks.  Both kinds of SWCPs are included in the Forest

Plans as Forest-wide standards or are referenced in the plans.  A

third kind of SWCP is identified by the interdisciplinary team

for application to specific management areas.  These are included

as management Area Standards in the appropriate management areas

in the Forest Plan.  A fourth kind, site specific SWCPs, are

based on project level evaluation and represent and most

effective and practical means of accomplishing the soil and water

resource goals and protecting the beneficial uses of a specific

area.  These site specific conservation practices supplement the

Forest Plan for specific projects.  In Idaho, these site specific

SWCPs are recognized as specialized Best management practices in

the State's Water Quality Regulations.  This handbook will aid in

the development of the fourth kind of SWCP.

Soil Productivity.  The capacity of a soil to produce a

specific crop such as fiber and forage, under defined levels of

management.  It is generally dependent on available soil

moisture, nutrients, texture, structure, organic matter, and

length of gorwing season.

Special Use Permit.  A permit issued under established laws

and regulaitons to an individual, orgainzation, or company fo

roccupancy or use of national Forest system lands for some

special purpose.

Specified Road.  A forest development transportationsystem

road that is identified in and to be constructed or reconstructed

under a Forest Service timber slae contract.

Stream or Streamcourse.  A natural channel with defined bed

and banks.  It may be perennial, intermittent, or ephemeral.

Streamside Management Zone (SMZ).  A designated zone that

consists of the stream and an adjacent area of varying width

where management practices that might affect water quality, fish,

or other aquatic resources are modified.  The SMZ is not a zone

of exclusion, but a zone of closely managed activity.  It is a

zone which acts as an effective filter and absorptive one for

sediment; maintains shade; protects aquatic and terrestrial

riparian habitats; protects channel and streambanks; and promotes

floodplain stability.  The SMZ may be wider than the riparian

area.

Wetlands.  Those area that are inundated by surface or

groundwater with a frequency sufficient to support, and under

normal circumstances do or would support a prevalence of

vegetation or aquatic life that requires saturated or seasonally

saturated soil conditions for growth and reproduction.  Wetlands

generally include swamps, marshes, bogs, and similar areas such

as sloughs, potholes, wet meadows, river overflows, mud flats,

and natural ponds.

Windrowing.  To pile slash or debris in a row along the

contour of the slope.

FSH 2509.22 - SOIL AND WATER CONSERVATION PRACTICES HANDBOOK

R-1/R-4 AMENDMENT NO. 1

EFFECTIVE 05/88

CHAPTER 10 - SOIL AND WATER CONSERVATION PRACTICES DOCUMENTATION

10.10 - Introduction.  Soil and water resources and water-related

beneficial uses are best protected during land disturbing

activities from nonpoint source pollution by use of site

specific, soil and water conservation practices.  These are

developed in an interdisciplinary process.  This process places

emphasis for maintenance and protection of these resources and

uses on the application of the site specific practices,

monitoring successes and failures, and adjusting the practices

and/or evaluation criteria until the resources are protected.

This handbook describes this process and provides some examples

of proven soil and water site specific practices, design

standards and risks, environmental effects, practicality, and

institutional, political, social, economic, and technical

feasibility must be considered.  The subsequent discussion the

process in 10.1-10.4 is supported and additional described by W.

C. Haper in "A Resource Agency's perspective on Nonpoint Source

management" (Symposium on Monitoring, Modeling, and Mediating

Water Quality, American Water Resources Association, May 1987,

pages 641-652).

The soil and water conservation practices that are presented in

this handbook are generally the initial development stage for

site specific practices.  They were compiled from Forest Service

manuals, handbooks, contract and permit provisions, and policy

statements. Their use or modification as more site specific

conservation practices will directly or indirectly improve water

quality, protect beneficial uses, reduce losses in soil erosion

and productivity, and abate or mitigate management effects, while

meeting other resource goals and objectives.

These soil and water conservation practices in this handbook are

of three basic forms: administrative, preventive, and corrective.

They are not detailed solutions for specific problems.  However,

in some instances, specific examples and practices are provided.

For the most part, they are purposely broad to ensure site

specific adaptation prior to their use.  In additions, they

identify management requirements and considerations to be

addressed prior to and during the formulation of alternatives and

the project implementation of land management activities.

10.20 - Development Considerations.  The effects of land

management activities on soil and water resources and

water-related beneficial uses vary considerably.  The extent of

these management effects on these resources and uses is a

function of:

a.  The physical, meteorologic, hydrologic, and biologic

environment where the activity takes place (topography,

physiography, precipitation, channel density, geology,

soil type, vegetative cover, and so forth).

b.  The specific water-related beneficial use(s), the

importance to various publics, and the sensitivity to

management influences.

c.  The type of activity imposed on a given environment

(recreation, mineral exploration, timber management, and

so forth) and it's real extent and magnitude.

d.  The method of application and the duration of the

activity (grazing system used, types of silvicultural

practice used, constant v. seasonal use, recurrent

application or one-time application, and so forth).

e.  The season of the year that the activity occurs or is

applied.

These factors vary within the National Forest System lands in the

Northern and Intermountain Regions and from site to site.  It

follows then that the extent and kind of impacts are variable, as

are the abatement and mitigation measures.  No specific practice

method, or technique is best for all circumstances.  Thus, the

soil and water conservation practices presented in this handbook

include such phrases as "according to design", "as prescribed",

"suitable for", "within acceptable limits", and similar

qualifiers.  The actual specifications, designs, and site

specific conservation practices must be the result of evaluation

and development by professional personnel through

interdisciplinary involvement in the NEPA process.  This results

in conservation practices that are tailored to meet local

resource requirements and needs for site specific conditions.

Area universities and Research have generated much information

that can be utilized for development of appropriate conservation

practices.  However, this information tends to be developed for a

specific site.  While such information is valuable, it should be

used with caution when developing conservation practices.  This

information may not adequately reflect site specific conditions

associated with other projects or locations, or consider spatial

and temporal distribution of other land management activities.

Additionally, it is important to establish an acceptable level of

risk associated with failure in developing site specific

conservation practices.  Since land managers must work with the

natural environment, all it's complexities, and therefore

uncertainty, it is necessary to accept some level of risk with

any design.  An important but difficult task is to agree on an

acceptable level of risk for given projects based on possible

impacts on other resources, and the need to conduct the

management of risk dictates that there will be some projects

which will fail to meet an absolute standard.  Through the

process of monitoring, evaluation, and adjustment of SWCP's, soil

and water resources and water-related beneficial uses can be

maintained and protected.

NEPA and Interdisciplinary Involvement.  The NEPA process and

interdisciplinary involvement is cirtical for the development of

site specific conservaiton practices.  Direction for the NEPA

process (environmental analysis and documentation) is contained

inForest Service Policy and Porceduires that are found in FSM

1950 and FSH 1909.15.  They also provide direction to incorporate

the interdisciplinary process in planning and decision-making.

The soil and water conservation practices detailed in this

handbook were utilized in Forest Planning in the Northern and

Intermountain Regions and now supplement all Forest Plans.  The

interrelationships between Forest Planning and Forest Plan

Implementation (project identification/administration) are

described in FSM 1922.5 and FSH 1909.12, and being further

refined by informational and awareness efforts in both Regions.

During Forest Plan Implementation, the Soil and Water

Conservation practices Handbook together with the Forest Plan are

used by the interdisciplinary team to develop site specific

conservation practices.

In NEPA, interdisciplinary involvement is essential to the

success of the process.  Interdisciplinary involvement is the use

of a team of individuals who represent two or more areas of

professional knowledge, learning, and/or skill.  They concentrate

on the same subject, such as development of conservation

practices, to derive a common resolution or product.  The team is

not necessarily a fixed assortment of professionals.  Team

members represent the skills necessary to provide input for

alternative formulation, evaluation, and conservation practice

development.  They are identified by either a core team or

technical staffs, the Staff Officer, the Line Officer or a

combination of all three.  The final team composition is approved

by the Line officer with project approval authority.

The Forest Service utilizes interdisciplinary involvement to

investigate problems, evaluate alternatives, analyze

environmental considerations, develop site specific practices,

and aid decision-making.  The responsibility for making the

decision lies with the Line Officers (that is, District Rangers,

Forest Supervisors, and so forth), not the team.  The team's

responsibility is to provide the responsible official with

alternatives and evaluations needed to make a reasonable

decision.

10.30 - Application.  After development of site specific

conservation practices, they are implemented on the ground along

with other specific requirements, controls, and considerations

that were built into a project activity.

A training and information program for personnel that are

involved in application of conservation practices is critical to

ensure maximum effectiveness of the practice in maintenance and

protection of soil and water resources and water-related

beneficial uses.  This training and information program should

involve Forest Service resource staff, Line Officers, and State

Water Quality personnel.  It should cover resource protection,

water quality and beneficial use management opportunities, cause

and effect relationships, and the relevance of conservation

practices, their legal aspects, and their application.  In

addition to Contracting Officers, Engineering Representatives,

and certified Sale Administrators, disciplines involved in

activities which have the potential to affect water quality,

beneficial uses, and soil resources should be offered more

intensive training.

10.40 - Feedback Mechanism.  Following application of site

specific conservation practices, they are monitored and

evaluated.  Subsequent adjustments and modifications are made tot

he conservation practices and/or water evaluation criteria until

the soil and water resources and water-related beneficial uses

are protected.  This feedback mechanism is absolutely necessary

to ensure sound land stewardship by the Forest Service.

Monitoring

Monitoring is the first step of the feedback mechanism.  It is

designed to answer questions about site specific conservation

practice development, application, and effectiveness.  Specific

questions may be:

Were the appropriate conservation practices included in

the project?

Did the project follow the plan?

Are the conservation practices technically sound and

appropriate for the specific site conditions?

Is there a better conservation practice to apply which is

technically sound, economically feasible, within

institutional authority, and protects the resources?

Were the conservation practices applied in total concept

or only partially employed?

Were personnel, equipment, funds, or training lacking

which resulted in incomplete or inadequate application?

How effective were the site specific conservation

practices in meeting the evaluation criteria?

Monitoring also is designed to seed answers about the

appropriateness of practices in maintaining or protecting soil

and water resources and water-related beneficial uses.  Some

questions may be:

Are the practices protecting the soil and water resources

and beneficial uses?

Do the parameters that are monitored establish the right

indices to indicate protection of resources or uses?

Is there a cause and effect relationship between the

conservation practices and soil and water resources and

beneficial uses?

Was the hydrologic risk that was built into the practice

exceeded?

To monitor all aspects of site specific conservation practices,

an appropriate mix of both extensive and intensive monitoring is

needed and performed at established intervals.  Extensive

monitoring is the primary means that the Forest Service will use

to evaluate the development, application, and effectiveness of

conservation practices.  It can be characterized by use of

project reviews and collection of both quantitative and

qualitative information on nearly all project activities.

Intensive monitoring tends to be more costly than extensive

monitoring, and will be restricted to the determination of cause

/effect relationships and specific conservation practice

effectiveness both on a representative sample basis.  This type

of monitoring obtains mostly quantitative information.  Both

extensive and intensive levels of monitoring are described and

further discussed in Forest Service manuals and handbooks.

Evaluation

Evaluation is the second step in the feedback mechanism.  To

evaluate monitoring information and judge the effectiveness of

site specific conservation practices, evaluation criteria must be

defined.  The Forest Plan standards and State Water Quality

Standard Criteria have been developed to serve as the evaluation

criteria.  These criteria should be defined in quantitative

terms, whenever possible.  However, they should avoid

instantaneous measurements given the dynamic nature of nonpoint

source conditions.  These criteria should recognize and consider

the attributes and characteristics of the particular resource or

use, natural variability and background, limits of acceptable

change in magnitude and duration, transport mechanisms and

pathways, time delayed effects, and risk.

In design of monitoring and during evaluation, certain elements

in the application of soil and water conservation practices must

be recognized and acknowledged.  The first of theses is that a

certain risk of failure is inherent in each site specific

conservation practice utilized.  This risk is a balance between

the value of the resource(s) to be protected and the cost of

additional units of protection.  Monitoring and evaluation must

not only measure effects but must determine when a design failure

point has been exceeded.  This has been discussed earlier in this

chapter (Development Considerations).

Another consideration is recognition that the link between land

management activities and the resulting impacts on soil and water

resources and water-related beneficial uses is not always well

understood.  Monitoring and evaluation must be designed to

improve our knowledge of this link and to provide an early

warning system where little research information exists for

guidance.  Where adequate research information exists for similar

conditions, the use of site specific conservation practices that

are designed and based on this information can be reasonably

expected to protect the soil and water resources and beneficial

uses.  In this case, monitoring and evaluation need not be so

intensive.

The use of sometimes inappropriate, State Water Quality Standard

Criteria in evaluation is another element to recognize.  With

existing technology, it is extremely difficult to determine the

background levels and variability to a level of precision and

accuracy necessary for direct control by numeric, State Water

Quality Standards.  This difficulty is particularly evident when

considering the tremendous temporal and spatial variability of

soil and water resources and water-related beneficial uses.

Because many existing water quality standards do not recognize

this variability, they may be of limited value as an evaluation

criteria for nonpoint source activities.  For this very reason,

State Water Quality Standards for nonpoint sources in conjunction

with conservation practices are also monitored, evaluated, and

adjusted, if necessary.  Without any adjustment, there is a

danger that site specific conservation practices will be required

that are technically sound and feasible but are of little or no

value in protecting soil and water resources and beneficial uses.

Adjustment

The last step of the feedback mechanism is adjustment.  If

monitoring and evaluation indicates evaluation criteria not being

met, an adjustment of the site specific conservation practices

are needed.  This adjustment will vary dependent upon the type

and severity of the impact to the soil and water resource or

beneficial use.  For minor or moderate impacts, the conservation

practice will be redesigned or upgraded to assure the criteria

are not exceeded.  When the impact is major, the project activity

will be reevaluated, redesigned, or dropped, or the application

process for practices and other project requirements revised.

Corrective actions to prevent or minimize the impact will be

initiated immediately.  Additionally, the appropriate evaluation

criteria are reviewed for adjustment.

This feedback mechanism is an iterative type process.  Through

the continuous cycle of monitoring , evaluation, and adjustment

of conservation practices and/or evaluation criteria, the site

specific conservation practices will lead to achievement of

evaluation criteria (that is, State Water Quality Standards and

Forest Plan standards) and protection of soil and water resources

and beneficial uses.

10.50 - Format.  The general format used to present each of the

conservation practices in this handbook is as follows:

Heading                         Context

PRACTICE                 Includes the sequential number of the

practice and a brief title.  The

numbering of the activities does not

have any intended significance.

OBJECTIVE                Describes the desired results or

attainment of the practice as it relates

to soil and water resource or beneficial

use protection.

EXPLANATION              Further define the brief title and

expresses how the practice is applied.

Describes criteria or standards used

when applicable.

IMPLEMENTATION           Describes where the practice is applied,

who is responsible for application,

direction and supervision, and when the

practice is employed.

REFERENCES               Identifies the Forest Service Manual,

Handbook, contract or permit provision,

Code of Federal Regulation, policy

statement and/or other references where

the practice is further documented.  The

reference listings are not all

inclusive.

Soil and water conservation practices are grouped by; management

activity for ease of presentation and understanding.  The

activities are: (11) Watershed Management, (12) Recreation, (13)

Vegetation Manipulation, (14) Timber, (15) Roads and Trails, (16)

Minerals, (17) Range, and (18) Fire Suppression and Fuels

Management.  Although a practice might be shown under only one

activity designation, it may apply to another activity.  A number

of the practices are referenced to more than one activity, and

some apply to all activities.

11 - WATERSHED MANAGEMENT.  Watershed management is the practice

of protecting and maintaining soil and water resources and

water-related beneficial uses.  Management is oriented toward

maintaining or enhancing watershed conditions favorable for

optimum water yield and timing, water quality, and soil

productivity.  Watershed management includes the improvement of

soil and water resources on National Forest lands damaged by

catastrophic events or degraded by past use.

PRACTICE:  11.01 - Determination of Cumulative Watershed Effects.

OBJECTIVE:  To determine the cumulative effects or impacts on

beneficial water uses by multiple land management activities.

Past, present, or reasonably foreseeable future actions in a

watershed are evaluated relative to natural or undisturbed

conditions.  Cumulative impacts are a change in beneficial water

uses caused by the accumulation of individual impacts over time

and space.  Recovery does not occur before the next individual

practice has begun.

EXPLANATION:  The Northern and Intermountain Regions will manage

watersheds to avoid irreversible effects on the soil resource and

to produce water of quality and quantity sufficient to maintain

beneficial uses in compliance with State Water Quality Standards.

Examples of potential cumulative effects are: 1) reduced natural

woody debris input to stream channels that may cause reductions

in fish habitat; 20 excess sediment production that may reduce

fish habitat and other beneficial uses; 30 water temperature and

nutrient increases that may affect beneficial uses; 40 compacted

or disturbed soils that may cause site productivity loss and

increased soil erosion; and 50 increased water yields and peak

flows that may destabilize stream channel equilibrium.

IMPLEMENTATION:  As part of the NEPA process, the Forest Service

will consider the potential cumulative effects of multiple land

management activities in a watershed which may force the soil

resource's capacity or the stream's physical or biological system

beyond the ability to recover to near-natural conditions.  A

watershed cumulative effects feasibility analysis will be

required of projects involving significant vegetation removal,

prior to including them on implementation schedules, to ensure

that the project, considered with other activities, will not

increase sediment or water yields beyond or fishery habitat below

acceptable limits.  The Forest Plan will define these acceptable

limits.  The Forest Service will also coordinate and cooperate

with States and private landowners in assessing cumulative

effects in multiple ownership watersheds.

REFERENCES:  40 CFR 1508.7; for protions of Montana, Montana

Department of State Lands Cumulative Watrershed Effects

Cooperative; for Idaho Forests, Idaho Forest Practices Water

Quality Management Plan, 1987; R. N. Coats and T. O. Miller.

981.  Cumulative Silvicultural Impacts on Watersheds:  A

Hydrologic and Regulatory Dilemma. Envir. Mgt. 5(2):147-160.

PRACTICE 11.02 - Soil and Water Resource Monitoring and

Evaluation

OBJECTIVE:  To determine effects of land management activities on

soil productivity and beneficial water uses; to monitor baseline

watershed conditions for comparison with State standards, Forest

Plan standards, and estimation of long-term trends; to ensure the

health and safety of water users; to evaluate SWCP's

effectiveness; and to determine the adequacy of data,

assumptions, and coefficients in the Forest Plans.

EXPLANATION:  The Northern and Intermountain Regions will manage

watersheds to avoid irreversible effects on the soil resource and

to produce water of quality and quantity sufficient to maintain

beneficial uses in compliance with State Water Quality Standards.

Monitoring and evaluation are needed as feedback mechanisms to

compare the results of management activities and SWCPs on soil

and water resources with previous conditions, desired

end-products, and State standards.  To accomplish this, a

comparison will be made, on a representative sample basis, of

effects on soils and water over time.  Previous monitoring and

evaluating has included, for example:

a.  Bulk density, soil disturbance, and/or tree growth to

evaluate soil productivity.

b.  Fecal coliform and pH to monitor swimming sites.

c.  Sediment, turbidity, and water temperature to

evaluate domestic water supplies.

d.  Sediment, dissolved oxygen, water temperature, pH,

cobble embededness, percent fines in substrate, and

channel cross sections to monitor effects on fisheries.

IMPLEMENTATION:  Forest Plans will provide watershed monitoring

plans that are focused on beneficial water uses such as domestic

supplies, recreation, and fisheries, and on soil loss and

productivity.  Regionally approved monitoring techniques will be

used.  Specific monitoring plans will be coordinated among

adjacent National Forests and with State water quality agencies.

Specific monitoring and evaluation plans will include such items

as:

a.  Monitoring objectives.

b.  Review of existing data and information.

c.  Location of monitoring sites.

d.  Soil and water quality characteristics that are to be

monitored and evaluated.

e.  Type(s) or technique(s) of monitoring.

f.  Intensity of monitoring (frequency and duration).

g.  Responsibilities and roles of monitoring personnel.

Methodology for analysis and evaluation.

i.  Estimated cost.

j.  Report preparation.

When changes and effect from management activity are detected,

the Forest Service will evaluate their significance and determine

appropriate action.  Where project level activities will not meet

Forest Plan or State standards, they will be redesigned,

rescheduled or dropped.

The EPA computerized STORET system is the accepted repository for

water quality data collected to monitor and evaluate Forest

programs and management activities.  Water quality data will be

placed in this computer system for storage, manipulation and

review.

REFERENCES:  FSM 1922, 2525, 2532, and 2554; SWCP Handbook 10.40

Feedback Mechanism; FSH 1909.12, Land and Resource Management

Planning Handbook, chapter 6; 36 CFR 219; Solomon, R. A. and

Avers, P. E., 1987.  A Water Quality Monitoring Framework to

Satisfy Legal Requirements.  AWRA Symposium on Monitoring,

Modeling, and Mediating Water Quality.  pp. 231-242; FSH 2509.18,

Soil Monitoring Handbook, State Water Quality Standards; for

Idaho Forests, Idaho Forest Practices Water Quality Management

Plan, 1987; SWCP 12.02, 12.03, and 13.09; S. L. Ponce.  1980.

Water Quality Monitoring Programs. USDA, Forest Service, WSDG

Tech. Paper - 00002.  66 pp.; for R-4, R-4 Technical Guide for

Preparing Water Quality Monitoring Plans, USDA, Forest Service,

1981.

PRACTICE:  11.03 - Watershed Improvement Planning and

Implementation

OBJECTIVE:  To improve degraded watershed conditions, to minimize

soil erosion, and to improve water availability or quality.

EXPLANATION:  Watershed improvement is a corrective measure.

Factors considered in the evaluation of soil and water resource

problems and subsequent improvement are: predicted changes in

water quality and its associated effects on beneficial uses,

downstream values, on-site productivity, threat to life and

property, direct and indirect economic returns, and social and

scenic benefits.  Examples of water she improvement measures are

stream bank stabilization, debris removal, soil ripping, seeding

, and fertilizing.

IMPLEMENTATION:  This conservation practice is typically

implemented through the development of a soil and water resource

improvement inventory, the approval of cost effective plans, and

the funding of the paln and subsequent improvement action.  If a

soil and water resource problem is observed and documented by

Line Officers, an interdisciplinary team will assess each site,

devlop the necessary actions to correct the problem and integrate

them into the Forest Planning process for funding and execution.

The NEPA process will be followed in the planning and

implementation of improvement measures.  The actual

implementation work may be done by Forest Service crews or

contract.  Effectiveness of improvement measures will be

monitored and evaluated.

REFERENCES:  FSM 2522; FSH 2509.15, Watershed Improvement

Handbook; NFMA.

PRACTICE:  11.04 - Floodplain Analysis and Evaluation

OBJECTIVE:  To protect floodplain values and avoid, where

possible, the long and short-term adverse impacts to soil and

water resources associated with the occupancy and modification of

floodplains.

EXPLANATION:  A flood hazard analysis and evaluation will be made

prior to acquisition or exchange of land within floodplains.  A

floodplain analysis and evaluation will be made when sites within

floodplains are being considered for structures, developments, or

management activities.  Environmental quality, ecological

effects, and individual safety and health are considered.  Flood

frequencies, watershed conditions, climatic and environmental

factors associated with past flood events, flood flow quantities

and specific flood boundaries are all evaluated.

IMPLEMENTATION:  The Regional Forester is responsible for

ensuring consideration of floodplain hazards and values in all

NEPA planning processes.  The Forest Supervisor, through use of

technical staffs, is responsible for:

a.  Determining if proposed facilities are within 100 and

500 year floodplain boundaries.

b.  Determining if data currently exist about floodplain

boundaries.

c.  Documenting analysis of floodplain hazards and

management options.

d.  Requiring flood hazard evaluations prior to issuance

of special-use permits.

e.  Ensuring that floodplain hazards, management

considerations, and appropriate restrictions are included

in authorizing documents.

f.  Designing, constructing, or rehabilitating National

Forest real property in accordance with criteria outlined

in the National Flood Insurance Program.

g.  Providing for conspicuous marking of highest past and

probable future flood heights, on permanent structures

including those in developed recreation sites.

REFERENCES:  EO 11988, Floodplain Management; FSM 2527; Maxwell,

J. and LaFayette, R., 1986  Guidelines for Making Floodplain and

Wetland Evaluations for Land Exchanges.  USDA, Forest Service,

Southwestern Region Hydrology Note No. 19a.

PRACTICE:  11.05 - Wetlands Analysis and Evaluation

OBJECTIVE:  To maintain wetlands functions and avoid adverse soil

and water resource impacts associated with the destruction or

maodificaiton of wetlands.

EXPLANATION:  The Forest Service does not permit the

implementation of activities and new construction in wetlands

whenever there is a practical alternative.  Through the NEPA

process, a wetland analysis and evaluation will be made prior to

acquisition or exchange of wetlands.  Evaluation of proposed

actions in wetlands will consider factors relevant to the

proposal's effective on the survival and quality of the wetlands.

Factors to be considered include water supply, water quality,

recharge areas, flood and storm hazards, flora and fauna species,

soil types, habitat diversity and stability, and hydrologic

utility.

IMPLEMENTATION:  The Regional Forester is responsible for

insuring wetland values are considered and documented as an

integral part of all planning process.  The Forest Supervisor,

through use of technical staffs, will determine whether proposed

actions should be located in wetlands and, if so, whether there

is a practicable alternative.  If there are no viable

alternatives, the Forest Supervisor must insure hat all

mitigating measures are incorporated into the plans and designs

and that the actions maintain the function of the wetlands.

Identification and mapping of wetlands are part of the Forest

Planning process.

REFERENCES:  EO 11990, Protection of Wetlands; FSM 2527; Maxwell,

J. and LaFayette, R., 1986.  Guidelines for Making Floodplain and

Wetland Evaluations for Land Exchanges.  USDA, Forest Service,

Southwestern Region Hydrology Note No. 19a.

PRACTICE 11.06 - Public Supply Watershed Management

OBJECTIVE:  To manage community and noncommunity public supply

watersheds to comply with State water quality standards.

EXPLANATION:  The Northern and Intermountain Regions will manage

public supply watersheds for multiple use with special emphasis

on providing water suitable for human consumption within the

realm of State Water Quality Standards, water supply regulations,

and Forest Plan standards.

IMPLEMENTATION:  Watersheds identified by the States as public

supply watersheds will be recognized in Forest Plans. Forest

Plans will include management goals and standards which will

guide the management of the watershed and result in compliance

with State Water Quality Standards.  All project plans will be

reviewed through the NEPA process which includes review by the

appropriate State agency and by the water users and tiered to

direction in the Forest Plans and EIS.

REFERENCES:  FSM 2542; State Drinking Standards; State Public

Water Supply Regulations; 36 CFR 251.

PRACTICE  11.07 - Oil and Hazardous Substance Spill Contingency

Planning

OBJECTIVE:  To minimize contamination of waters from accidential

spills by prior planning and development of Spill

PreventionControl and Countermeasure Plans.

EXPLANATION:  A contingency plan is an immediate reporting and

action plan that contains a predetermined organization to be

implemented in the event of a hazardous substance spill.  Factors

considered for each spill are: the specific substance spilled,

the quantity, its toxicity, proximity of spill to waters, and the

hazard to like, property, and the environment.

The Spill Prevention Control & Countermeasure (SPCC) plan is a

docuemnt which requires appropriate measures to prevent oil,

petroleum products, or known hazardous materials that could be

spilled from entering the navigable waters of the United States.

An SPCC plan is needed if the total, above-ground storage of oil,

petroleum products, or known hazardous materials exceeds the

appropriate "reportable quantity" and if these facilities could

reasonably be expected to discharge these hazardous substances

into surface waters in the event of a spill.

IMPLEMENTATION:  Each Forest is responsible for designating

emergency spill coordinators and documenting names and telephone

numbers of agencies to call regarding notification and clean-up

of spills.  Individual Forests may maintain an inventory of

materials to use during the clean-up of a spill.  Disposal sites

will be coordinated with EPA, State, and local officials

responsible for safe disposal.

If a spill is from a Forest Service facility or operation, the

Forest Service is the "person in charge" and is responsible for

all reporting and immediate response actions, as appropriate.  If

the spill is from a third party operation, the Forest Service

will only respond and report the spill if the third party fails

to take appropriate action.  The Forest Service will generally

turn its incident command role over to authorized, Federal

On-Scene Coordinators or other authorized, State or local

authorities after their arriving at the spill site, and provide

support services.

SPCC plans are required for Forest Service owned and special use

permitted facilities, and include timber sale operators and other

construction contracts.  All SPCC plans must be reviewed and

certified by a registered professional engineer.

REFERENCES:  FSH 6740, 7442, 7443, and 7460; 40 CFR 112; FSH

6709.11, Health and Safety Code Handbook; FSH 6709.12, Safety and

Health program Handbook; R-1 and R-4 Emergency and Disaster Plan;

Oil and Hazardous Substances Pollution Contingency Plan for EPA

Regions 8 and 10, 7/26/85; State Hazardous Waste management

Plans; SWCP 11.11, 13.07, and 13.10.

PRACTICE  11.08 - Control of Activities Under Special Use Permit

OBJECTIVE:  To protect surface and subsurface soil and water

resources from physical, chemical, and biological pollutants

resulting from activities that are under special use permit.

EXPLANATION:  Many activities and uses take place on National

Forest lands which are not directly related to Forest Service

management activities.  Some examples are: hydropower plants,

water diversions and water transmission systems, electronic

sites, highway and railroads rights-of-way, waste water treatment

and disposal, solid waste disposal, and power transmission lines.

There are other uses which are recognized Forest Service land

management activities which are achieved through permits to a

public or private agency, group, or individual.  Examples of

these types of uses are: organization  camps, recreation

residence tracts, and ski areas.

Management objectives and Forest Service authority may be limited

on lands withdrawn under Federal Energy Regulatory Commission

(FERC) authority.  When the FERC license is renewed, the Forest

Service makes a complete reevaluation of water quality and

quantity needs, values, and effects, and updates the management

controls within which the permittee must comply.

IMPLEMENTATION:  The special use permit under which these

agencies, groups, or individuals operate, details the conditions

that they must meet to continue operating.  The permittees are

required to conform to all applicable State and local regulations

governing water quality and sanitation.  Failure on the part of

the permittee to meet the conditions of the special use permit

may result in the permit being revoked.  Relevant SWCP's from

this and other sections may be required: 11.01, 11.04, 11.05,

11.07, 11.10, 11.11, 12.04, 12.06, 12.07, 12.08, 13.04,

13.07-13.13, 14.12-14.14, 14.20, 15.01-15.18, 15.21.

REFERENCES:  FSM 2700.

PRACTICE:  11.09 - Management by Closure to Use

OBJECTIVE:  To exclude activities that could result in damages to

facilities or degradation of soil and water resources.

IMPLEMENTATION:  Closures (seasonal, temporary, or permanent) are

made when the responsible Line Officer determines that a

particular resource or facility needs protection from use.  The

decision to close an area is made after an evaluation of

alternative methods of protection.

REFERENCES:  EO 11644, use of Off-Road Vehicles on the Public

Lands, and 11989, Off-Road Vehicles on Public Lands; SWCP 12.10.

PRACTICE:  11.10 - Water Well Construction and Management

OBJECTIVE:  To protect ground water resources from contamination

transmitted from water well developments.

EXPLANATION:  Ground water wells and facilities will be completed

in a manner that reduces the potential of contamination of the

ground water aquifer.  Properly designed well casing programs and

above ground well collars will minimize the risk of incidental

contamination from grazing livestock and human activities.

IMPLEMENTATION:  Water well design standards will contain

measures to exclude livestock from the immediate vicinity of the

well bore and to prevent the accidental or purposeful

introduction of toxic substances into the well bore by humans.

Protective measures may include grading to direct surface waters

away from the well bore, cementing or grouting around the casing

pipe, well covers and seals, fencing, and imposing minium

distances between wells and sewage systems.  Abandoned wells will

be permanently sealed in compliance with local and state

reacquaints.

REFERENCES:  FSH 2209.22, Structural Range Improvement Handbook;

FSH 7409.11, Water Developments and Sanitation Handbook;

Engineering Field Manual for Conservation Practices, 1969, U.S.

Solid Conservation Service, Chapter 12, springs and Wells';

Recommended Standards for Water Works (Ten States Standards), New

York Health Education Service; State Drinking Water Standards;

State Well Construction Standards.

PRACTICE:  11.11 - Petroleum Storage and Delivery Facilities and

Management

OBJECTIVE:  To protect surface and subsurface soil and water

resources from petroleum fluid contamination resulting from

leaking petroleum delivery systems and storage facilities.

EXPLANATION:  Petroleum delivery and storage facilities will be

located, designed, constructed, and maintained in a manner that

minimizes the potential for contamination of surface and

subsurface soil and water resources from leaking flowlines,

pipelines and storage tanks.  Roads, vegetative manipulation, and

other considerations should be evaluated in the construction and

maintenance of these facilities.

IMPLEMENTATION:  The siting and operation of petroleum delivery

systems and storage facilities will follow applicable Federal

(EPA) and state guidelines and requirements with regard to:

a.  Design/Location.

b.  Construction

c.  Installation.

d.  Oepration procedures.

e.  Testing.

f.  Release detection systems.

g.  Recordkeeping requirements.

h.  Leak.spill reportingrequirements.

i.  Abandonment.

Storage facilities and delivery systems on National Forest lands

will require a license and/or speical use permit.  Licenses and

permits require the proejct ot comply with all State and local

standards.  Relevant SWCPs from this and other secitons may be

required: 11.04, 11.05, 11.07, 11.08, 11.10, 13.04, 13.07-13.13,

14.12-14.14, 14.20, 15.01-15.18, 15.21.

REFERENCES:  Resource Conservation and Recovery Act of 1976, (90

Stat. 2795), as amended; Underground Storage Tank Regulations (40

CFR part 280); State Hazardous Waste management Plans; FSM 7460.

PRACTICE:  11.12 - Administrative Site Planning and Management

OBJECTIVE:  To locate, design and manage administrative sites to

prevent water pollution and other adverse environmental and

health impacts.

EXPLANATION:  Several types of facilities may be located on

Forest Service land to facilitate the administration of various

programs. These may include Ranger and Research stations, work

centers, fire depots and nursery offices.  Some facilities

include living quarters with wastewater systems.  Most include

potable water systems.  Water quality and potential health

impacts should be a major factor in determining the size, type,

and location of new or expanded facilities.

IMPLEMENTATION:  New or expanded facilities should not be located

within floodplains, wetlands, or riparian areas.  Individual

facilities would be located to minimize sediment production and

the movement of potential pollutants (oil, gas, and so forth)

into surface and  ground waters.  All potable water and sewage

treatment systems will meet State and local standards.  Regional

Forest Service programs that guide operation and maintenance of

wastewater facilities will be implemented by qualified sanitary

personnel.  Relevant SWCP's from this and other sections may be

required: 11.04, 11.05, 11.10, 11.11, 12.04, 12.06, 12.07, 15.26.

PRACTICE:  11.13 - Sanitary Guideline for Construction of

Temporary Labor, Spike, Logging, and Fire Camps and Similar

Installations

OBJECTIVE:  To eliminate water pollution and other potential

environmental and health impacts from the disposal of human waste

and wastewater form temporary camps of all types.

EXPLANATION:  Environmental and health impacts associated with

these camps can be adverse if care is not taken to properly plan,

locate, and design wastewater facilities.  In establishing site

locations, sewage disposal consideration should not be

overlooked.  Despite the sometimes urgent development of a site,

wastewater design should be a primary consideration.  This should

result in adequate sewage disposal facilities, and little or no

health and water pollution threats.

The same applies to the urgent establishment of fire camps.

Sufficient care should always be given to the potential impacts

associated with human waste and wastewater disposal.  Disposal

facilities at existing stations or work centers can be overloaded

by fire crews.  If this occurs, septic tanks may be overloaded,

solids may overflow and plug drainfields, and drainfields may be

damaged.  Surfacing effluent becomes a potential pollution and

health hazard.

IMPLEMENTATION:  Appropriate planning will be required and

completed for each site by competent technical resource staffs.

Systems will be designed to meet requirements of applicable

Federal, State and local regulations.  Forest Supervisor approval

will be obtained prior to construction.

Wastewater treatment and disposal systems at administrative sites

will be evaluated as to their capacity to handle fire, emergency,

or other crews.  Emergency measures such as chemical toilets and

portable showers will be considered as an alternative to use of

on-site systems.

Latrines or pits for camps will be located at least 150 feet

downstream from the camp, 100 feet from surface water, and 4 feet

above high groundwater.  Latrines will be replaced with chemical

toilets or similar units as soon as practicable.

REFERENCES:  health and Safety Code Handbook (FSH 6709.11,

chapter 7-9); FSM 7420, 7430, and 7440; SWCP 12.04, 12.06, 12.07,

and 12.09.

PRACTICE:  11.14 - Management of Snow Survey Sites

OBJECTIVE:  To protect snow courses and related data sites from

effects by land management activities.

EXPLANATION:  Snow survey sites are invaluable for forecasting

water yields from mountain watersheds and predicting the

availability and timing of streamflow.  These sites can consist

of snow courses, snow pillows, precipitation gauges, snow depth

markers, and soil moisture stations, with or without radio

telemetry and radio repeaters.  Formal Memorandum of

Understanding, special use permits, or other mechanisms have been

developed between the Soil Conservation Service (SCS) and Forest

Service to protect these sites against activities or disturbing

influences that would damage their value.

IMPLEMENTATION:  During the NEPA process for land disturbing

activities, the interdisciplinary team will consult land status

records, special use permits, and other sources to insure

identification of any snow survey sites within the assessment

area.  If sites are identified, they will be protected according

to the terms of the memorandum of understanding, special use

permit, or other mechanism.  Line Officers should request

assistance from the SCS in designing measures to protect and

buffer these sites.  The Forest Service and SCS will also

cooperate on the elimination of ineffective sites and

installation of additional snow courses or sites to improve the

reliability of runoff forecasts.  If it becomes necessary to

conduct activities which may affect a snow course or related data

site, Line Officers must consult the SCS and follow the

instruction of the agreement.

REFERENCES:  FSM 2530 and 2720; FSM 1541.1--1 to 7 (FSM 1/97 R-1

Supp. 50) or FSM 2504.2--9 to 16 (FSM 2/79 R-4 Supp. 56).

12 - RECREATION.  Recreation on National Forest System lands

falls into two general categories, developed and dispersed.

Developed recreation is the term used to describe recreation

areas that have been designed and built to provide some

facilities to the user.  These are campgrounds where tables, fire

places, toilets, and so forth, have been provided.  Recreational

residences, resorts, ski areas, and other similar facilities are

considered developed recreation.

Dispersed recreation is outdoor recreation use which occurs

outside of sites developed or managed for concentrated recreation

use.  Facilities are, however, sometimes required to safeguard

visitors, protect resources, enhance the quality of visitor

experiences, and disperse users.  Dispersed recreation also

includes the table-under-the-tree type of facility, portable

toilets in otherwise undeveloped sites, trailheads, and trails.

PRACTICE:  12.01 - Recreation Facilities Planning

OBJECTIVE:  To introduce soil and water resource considerations

into Recreation Facilities Planning.

EXPLANATION:  Recreation facilities include developed sites such

as campgrounds, picnic areas, boat ramps, and so forth, and

special use facilities such as ski areas, summer homes, and

organizational camps.  Individual site plans are prepared for

each facility.

IMPLEMENTATION:  An interdisciplinary team identifies potential

impacts to water quality and soil productivity during the NEPA

process.  The NEPA process identifies mitigating measures needed

to protect soil and water resources.  Consideration should be

given to appropriate location and design of facilities and to the

secondary impacts due to continuing use.

REFERENCES:  NFMA, MEPA, FSM 2332 and 2333.

PRACTICE:  12.02 - Monitoring and Evaluation of Water Quality at

Designated Swimming Sites

OBJECTIVE:  To insure the health and safety of water contact

recreationists at designated National Forest Swimming Sites.

EXPLANATION:  the monitoring and evaluation for bacterial

contamination of water quality is recommended at all designated

swimming sites.  Analysis values are tested against State Water

Quality Standards for primary contact recreation.

IMPLEMENTATION:  As part of SWCP 11.02, the water quality for

swimming sites is monitored and evaluated.  If State Standards

are exceeded, the area will be closed to all contact recreation

use until the cause or causes have been identified and remedied.

Closure is the responsibility of the Forest Supervisor or the

District Ranger.

REFERENCES:  FSM 2532 and 2335, State Water Quality Standards.

PRACTICE:  12.03 - Sanitary Surveys to Augment the Evaluation of

Designated Swimming Waters

OBJECTIVE:  To provide information regarding potential hazards or

the cause of an existing problem which is a health hazard.

EXPLANATION:  This practice is designed to augment the monitoring

and evaluation of swimming waters (SWCP 12.03).  Adjacent areas

and the aquatic environment are examined to detect potential or

existing health hazards.  The survey provides information needed

in defining the cause(s) of contamination, if evaluation of

swimming waters shows standards have been exceeded.

IMPLEMENTATION:  A sanitary survey will be made prior to the

development of plans for each new swimming facility.  All areas

where swimming is specifically encouraged or permitted should

have a sanitary survey as soon as practical.  Subsequent surveys

will be repeated periodically.  Results of the survey are

documented and provided to the Forest Supervisor and District

ranger for evaluation and appropriate action.

REFERENCES:  FSM 2532 AND 2335, SWCP 11.02.

PRACTICE:  12.04 - Providing Safe Drinking Water Supplies

OBJECTIVE:  To provide safe drinking water to Forest Service

facilities such as campgrounds, picnic grounds, trailheads,

Visitor Information Centers, winter sport areas, and developed

roadside facilities.

EXPLANATION:  Administrative guidelines for water source location

and development; testing frequency and limitation for

bacteriological, chemical, and physical contaminants; performance

of sanitary surveys; closing, correction, and reopening of

defective water systems; and documentation of data are provided

in the State Drinking Water Standards.

IMPLEMENTATION:  Location, design, sampling, and sanitary surveys

will be performed by qualified individuals who are familiar with

drinking water supply systems and requirements.  Coordination and

cooperation will be pursued with state and local health

department representatives in all phased of drinking  water

system management.  Preventive measures will be taken in the

location, construction operation, and maintenance of water supply

systems to minimize possibilities of contamination.

Sampling and testing of drinking water quality will be required.

If State and local health departments do not perform the water

analysis, State approved laboratories must be used.  When test

results indicate that State Standards are exceeded, the water

supply will be closed or treatment required until the problem is

corrected and satisfactory results are obtained.  Seasonal

systems will be tested and proven to be satisfactory prior to

opening.

REFERENCES:  Safe Drinking Water Act (P.L. 95-190); State Water

Quality Standards; State Drinking Water Standards; 40 CFR Parts

141, 142, and 143; FSH 6709.11, Health and Safety Code Handbook ;

FSM 2332, 2333, and 7420; SWCP 11.10 and 12.05.

PRACTICE:  12.05 - Documentation of Potable Water Quality Data

OBJECTIVE:  To assure the availability of water quality data and

related information when making analysis and interpretations with

respect to potable water systems.

EXPLANATION:  An inventory of the location of all designated

potable water supplies will be prepared and maintained

documenting pertinent site information such as dates and results

of all water quality tests and surveys.  This is an

administrative practice of record keeping to establish a record

of cause and effect to aid in identifying any sources of

contamination.

IMPLEMENTATION:  Forests will use the computer-based "Potable

Water System Management Program" for site documentation.  All

laboratory results will be filed on the Forest for a minimum of

five years.  Data will be stored, edited, and summarized

annually.

REFERENCES:  FSM 2532, 7421.24, and 7421.25.

PRACTICE:  12.06 - Management of Sanitation Facilities

OBJECTIVE:  To protect surface and subsurface soil and water

resources from bacteria, nutrients, and chemical pollutants

resulting from the collection, transmission, treatment, and

disposal of sewage at Forest Service facilities.

EXPLANATION:  Toilet facilities are provided at some recreation

sites.  The type and number depends on site utilization and the

capacity of a given site.  Sanitation facilities which may vary

from a pit toilet to a treatment plant will be planned, located,

designed, constructed, operated, inspected and maintained to

minimize the possibility of soil and water contamination.

IMPLEMENTATION:  The location, design, inspection, operation and

maintenance will be performed or controlled by qualified

operators who are familiar with sanitation systems and

guidelines.  State and local authorities should be consulted

prior to the installation of new sanitation facilities or

modifications of existing facilities to assure compliance with

all applicable State and local regulations.  Coordination and

cooperation should be pursued with State and local Health

Departments.

Disposal of sewage at designated sewage treatment plants will be

required, when feasible.  Where disposal at sewage treatment

plants is not feasible, specially designated sites will be

identified.  These areas will be identified in the disposal

contract or agreement.

Sanitary engineers will designate all areas on Forest property

where untreated effluent may be disposed of.  Sewage should be

buried at least 300 feet away from all surface water sources.

State and local health departments will be consulted as to

concerns, requirements, and regulations.

REFERENCES:  FSH 7409.11, Water Developments and Sanitation

Handbook; FSM 2331, 2332, 2333, 7430, 7440, and 7460; State and

local codes; State Water Quality Standards.

PRACTICE:  12.07 - Control of Refuse Disposal

OBJECTIVE:  To protect surface and subsurface  soil and water

resources from nutrients, bacteria, and chemicals associated with

soil waste disposal

EXPLANATION:  The users of National Forest recreation facilities

are encouraged to cooperate in the proper disposal of garbage and

trash.  Users are encouraged to burn their combustible trash in

fireplaces or stoves.  Receptacles are provided for unburnable

garbage and trash at some developed sites.  Garbage and trash

must be packed out by those who use dispersed and wilderness

areas.

IMPLEMENTATION:  The public education effort is a continuing

process accomplished through the use of signs, printed

information, mass media, and personal contact.  Public

cooperation is vital.

Each National Forest will follow their solid waste disposal plan

which describes their collection, removal, and final disposal

methods.  Garbage containers are place din areas which are

convenient for recreationists and are easily maintained.  On-site

enforcement may be necessary.

The final disposal of collected garbage will be at a properly

designed and operated sanitary landfill.  Each landfill site will

be located where groundwater and surface waters are at a safe

distance as prescribed by State and local codes.

REFERENCES:  FSM 2332, 2333, and 7460; State and local codes.

PRACTICE:  12.08 - Assuring Proper Sanitation and Water Supplies

For Special Use Facilities

OBJECTIVE:  To protect the quality of water both consumed by and

discharged from facilities under Special Use Permit.

EXPLANATION:  Special Use facilities must comply with State and

local sanitation ordinances.  Buildings and grounds will be

supplied with at least the minimum sanitary facilities required

by local codes.  Water systems must provide an adequate volume of

acceptably pure water for drinking, cooking, and general

sanitation.  Structures designed with toilets, showers, and wash

basins will be planned to serve the facility's needs and

capacity.

IMPLEMENTATION:  Management controls and requirements protecting

water quality through installation and maintenance of proper

sanitation and water supply facilities will be incorporated into

the Special Use Permit for the facility.  Permittees are required

to inspect water supply and proper sanitation.  Copies of the

test results will be provided to the appropriate Forest Officer,

as specified in the Special Use Permit.

REFERENCES:  FSM 2342 and 2343; SWCP 11.08, 12.04, 12.06 and

12.07; State Water Quality and Drinking Water Standards.

PRACTICE:  12.09 - Sanitation at Hydrants and Water Faucets

Within Developed Recreation Sites

OBJECTIVE:  To maintain water quality standards around hydrants

and faucets which provide water for consumptive use in developed

recreation sites.

EXPLANATION:  This practice prohibits the cleaning or washing of

any personal property, fish, animal, or food at a hydrant or at a

water faucet not provided for that purpose.  The public must be

informed of their responsibilities concerning sanitary

regulations.

Acceptable designated areas are those that are located away from

consumptive water sources and where effluent from the washing

operation can be disposed of properly.

IMPLEMENTATION:  The authorized Forest Officer will inform the

public of their sanitary responsibilities by posting signs on

bulletin boards or at hydrants or faucets, by notices in

newspapers, and by personal contact.  On-site enforcement by the

Forest Officer may be necessary for violators.

REFERENCES:  FSM 2334, 7410, and 7430.

PRACTICE:  12.10 - Management of Off-Road Vehicle Use

OBJECTIVE:  To control Off-Road Vehicle (ORV) use which is

causing soil erosion and adverse effects on water quality and to

identify corrective measures.

EXPLANATION:  Areas or trails where ORV use is causing

degradation of water quality or soil erosion should be

identified.  It should be determined through monitoring and

evaluated if degradation is beyond acceptable limits, as defined

by Forest Plans.

IMPLEMENTATION:  Monitoring results will be evaluated against

Forest Plan standards for soil and water resources and the

management objectives for the area.  If considerable ad adverse

effects are occurring or are likely to occur, corrective action

should be taken (SWCP 11.03).  Corrective actions may include,

but are not limited to, redistribution in the amount of ORV use,

development of a Forest-wide Travel Plan, signing or barriers to

redistribute use, partial closing of areas, rotation of use on

areas, closure to causative vehicle types or total closure, and

structural solutions such as culverts and bridges.

Closure is done by authority of the Forest Supervisor (SWCP

11.09).

REFERENCES:  SWCP 11.02; EO 11644, Use of Off-Road Vehicles on

the Public Lands, and EO 11989, Off Road Vehicles on Public

Lands; 36 CFR 295.5; FSM 2352 and 2355.

PRACTICE:  12.11 - Protection of Water Quality Within Developed

and Dispersed Recreation Areas

OBJECTIVE:  To protect water quality by regulating the discharge

and disposal of potential pollutants.

EXPLANATION:  This practice prohibits placing in or near a

stream, lake, or other waterbody, materials or substances which

may degrade water quality.  The actual safety distance from each

water body is at least 100 feet, or greater if warranted by local

conditions.

This includes, but is not limited to , human and animal waste,

oil, and other hazardous substances.  Areas may be closed in

order to restrict use in problem areas.

IMPLEMENTATION:  Within developed sites, authorized Forest

Officers will inform the public of their responsibility through

signs, bulletin boards, newspapers or personal contact.

Pamphlets, brochures and other material will be used to encourage

public cooperation in protecting water quality in dispersed

areas.  Forest Officers can issue citations to violators.

REFERENCES:  FSM 2323, 2332, 2333, 2334, 2335, and 2502; SWCP

11.02 and 11.07; FSH 2309.19, Wilderness Management Handbook.

PRACTICE:  12.12 - Location of Pack and Riding Stock Facilities

in Wilderness, Primitive, and Backcountry Areas

OBJECTIVE:  To avoid degradation of water quality from pack and

riding stock facilities.

EXPLANATION:  This practice directs the location of pack and

riding stock facilities at safe locations away from springs,

streams, lakes, wet meadows, and any other surface waters.

Public education is essential to establish acceptable use

patterns.  Signs, brochures, organized public education programs,

and personal contacts will be used to gain compliance.

Wilderness patrolmen will check compliance with use.

REFERENCE:  FSM 2323.63; FSH 2309.19, Wilderness Management

Handbook.

13 - VEGETATION MANAGEMENT. Vegetation management on National

Forest System lands is conducted in the course of forest

regeneration, brushland conversion to grass for fuels reduction,

brushland conversion to forests, utility transmission corridor

maintenance, rangeland improvement, water yield improvement, and

wildlife habitat improvements.  Means of conversion are:

chemical, mechanical, burning, and biological.  Each project is

evaluated through the NEPA process by an interdisciplinary team.

PRACTICE:  13.01 - Operation Seeding and Land Preparation

Equipment on the Contour

OBJECTIVE:  To reduce soil erosion and losses in soil

productivity and to minimize sediment production and turbidity.

EXPLANATION:  This measure is implemented to provide a means of

rapid infiltration and surface water detention, so that

infiltration can take place.  The factory evaluated are slope,

infiltration rate, permeability, soil depth, climatic variables,

and soil water holding capacity.  These field evaluations are

made by technical staffs during project planning.

IMPLEMENTATION:  Appropriate contract provisions and/or

management controls are identified in the NEPA process.  The

project supervisor is responsible for enforcing these management

requirements that deal with contour operations on In-Service

projects.  The Contracting is responsible for enforcing

provisions of contracts.  Contour operations, where limited by

steepness of slope (usually greater than 30 percent), may require

other methods of stabilization such as hydroseeding or hand

operations.

REFERENCES:  FSM 2245 (National Grasslands), see references in

"Best Management Practice" Definition (05--2 and 3).

PRACTICE:  13.02 - Slope Limitations for Tractor Operation

OBJECTIVE:  To reduce gully and sheet erosion and associated

sediment production.

EXPLANATION:  This measure limits surface disturbance and keeps

surface runoff water from concentrating.  This practice restricts

tractor operation to slopes where corrective measures for proper

drainage such as water bars are easily installed and effective.

Criteria that may be used to determine slope restrictions are

soil stability, mass stability, infiltration rate, and soil water

holding capacity.  This data may be interpreted from soil and

landtype inventories, geologic maps, and climatic and hydrologic

information.  Subsequent field verification may be necessary.

IMPLEMENTATION:  Recommended provisions and management controls

for operating machinery on steep slopes is identified during the

NEPA process.  These provisions should be included in the

contract. The contracting Officer is responsible for ensuring

implementation of these contract provisions.  For In-Service

projects, the project supervisor is responsible for enforcement

of management requirements and for identification of additional

areas where operations should be limited.

REFERENCES:  SWCP 14.07; See references in "Best Management

Practice" Definition (05--2 and 3).

PRACTICE:  13.03 - Tractor Operation Excluded from Wetlands,

Bogs, and West Meadows

OBJECTIVE:  To limit soil damage, turbidity, and sediment

production resulting from compaction, rutting, runoff

concentration, and subsequent erosion.

EXPLANATION:  This practice is designed to prevent soil puddling,

compaction and displacement, and the concentration of surface

water and soil erosion, which may lead to rill or gully erosion

and subsequent water quality degradation.  This measure is

intended to prevent or reduce the need for corrective measures to

solve water concentration problems due to tractor use.

IMPLEMENTATION:  Application is mandatory on all vegetation

manipulation projects unless specifically excluded in the NEPA

process.  Contract specifications and controls and requirements

are identified in the environmental analysis.  The project

supervisor and/or Contracting Officer are responsible for

identifying wetlands and meadows not previously recognized in the

NEPA process, and for following management controls and contract

provisions pertaining to wetlands and meadows.

REFERENCES:  EO 11990, protection of Wetlands; FSM 2527; SWCP

11.05 and 14.16; see references in "Best Management Practice"

Definition (05--2 and 3).

PRACTICE:  13.04 - Revegetation of Surface Disturbed Areas

OBJECTIVE:  To protect soil productivity and water quality by

minimizing soil erosion.

EXPLANATION:  This practice is used to stabilize the surface of

the disturbed area through the influence of vegetation.  The

vegetation will be selected to meet many or most of the

management objectives for the area; range, wildlife, timber,

fuels, minerals, aesthetics, and so forth.  Grass or browse

species may be seeded between recently planted trees for erosion

prevention, wildlife habitat enhancement, or other management

needs.

The factors evaluated are soil fertility, slope, aspect, landtype

characteristics, soil water holding capacity, climatic factors,

vegetation species characteristics, and project objectives.

These are filed determinations and office interpretations made by

an interdisciplinary team.

IMPLEMENTATION:  The identification of disturbed areas and

species mix will be determined during the NEPA process.  The

responsible Line Officer assigns specific individuals to execute

the project. Projects are subsequently monitored to assess the

revegetation effectiveness, and need for follow-up action.

REFERENCES:  FSM 2522, 2405, 2472, and 7721; SWCP 11.02, 11.03,

and 14.13; see references in "Best Management Practice"

Definition (05--2 and 3).

PRACTICE: 13.05 - Soil Protection During and Following Slash

Windrowing

OBJECTIVE:  To prevent removal or severe disruption of the

productive surface soil and to minimize losses from erosion.

EXPLANATION:  Windrowing is a common method of slash treatment

removal and surface scarification.  On slopes the material should

be windrowed on the contour to act as a filter barrier which

catches sediment and detains runoff water.  On many forest soils,

great care must be taken to preserve the surface soil layer

during the windrowing operation.

IMPLEMENTATION:  Recommendations on slash windrowing are

identified during the NEPA process.  The project supervisor is

responsible for enforcing applicable management requirements.

The Contracting Officer is responsible for enforcing contract

clauses.

REFERENCES:  SWCP 13.01; see references in "Best Management

Practice" Definition (05--2 and 3).

PRACTICE: 13.06 - Soil Moisture Limitations for Tractor Operation

OBJECTIVE:  To minimize soil compaction, puddling, rutting, and

gullying with resultant sediment production and loss of soil

productivity.

EXPLANATION:  This measure minimizes surface disturbance during

high soil moisture conditions which would result in compaction,

puddling, rutting, and gullying problems. This practice reduces

the need to correct these soil and water resource problems later.

Soil erodibility, compatibility, climatic factors, soil/water

relationships, and mass stability are factors that are used to

evaluate tractor limitations during the NEPA process.

IMPLEMENTATION:  Contract provisions and management requirements

for soil moisture limitations are identified during the

environmental analysis.  The project supervisor and/or

Contracting Officer are responsible for determining when the soil

surface is unstable and susceptible to damage and is then

responsible for suspending or terminating operations.

REFERENCES:  FSM 2522; see references in "Best Management

Practice" Definition (05--2 and 3); In R-4: R-4 Technical Guide -

Erosion Prevention and Control on Timber Sale Areas, May 1981.

PRACTICE:  13.07 - Pesticide Use Planning

OBJECTIVE:  To incorporate water quality and hydrologic

considerations into the Pesticide Use Planning Process.

EXPLANATION:  The pesticide use planning process will be used to

identify problem areas and the objectives of the project,

establish the administrative controls, identify treatments and

preventive measures, and incorporates the hydrologic

considerations contained in SWCP 13.08 through 13.13.  The NEPA

process addresses these considerations in terms of impacts,

mitigation measures, and alternative treatment measures.  Project

work and safety plans specify management direction.

Factors considered in pesticide selection are:  purpose of the

project, application methods available, target species, timing of

treatment, pest locations, size of treatment area, and need for

repeated treatment.  Practicability of application considers:

registration restrictions, form and method of application,

topographic relief and areas to be avoided, and social acceptance

of the project.  The degree of risk considers: hazard to humans,

method of application, transportation and handling hazards,

carriers needed, and chemical persistence.

IMPLEMENTATION:  The interdisciplinary team evaluates the project

in terms of potential site response, potential social and

environmental impacts, mitigating measures needed to protect

water quality, and the need and intensity of monitoring and

evaluation.  The responsible Line Officer then prepares the

necessary NEPA documentation, Project Plan, and Safety Plan.

Depending on the pesticide use, (FSM 2151.04) the Forest

pesticide-use coordinator or Integrated pest Management Working

Group or Regional IPMWG reviews the documents along with the

Pesticide-Use Proposal, form FS-2100-2, and makes recommendations

for or against approval of the project.

REFERENCES:  NFMA; NEPA; FSM 2150 and 2323; State Hazardous Waste

Management Plans; see references in "Best Management Practice"

Definition (05--2 and 3).

PRACTICE:  13.08 - Apply Pesticides According to Label and EPA

Registration Directions

OBJECTIVE:  To avoid water contamination by complying with all

label instruction and restrictions.

EXPLANATION:  Label directions for each pesticide are detailed

and specific, and include legal requirements for use.

IMPLEMENTATION:  Constraints identified on the label and other

legal requirements of application are incorporated into project

plans and contracts.  Responsibility for ensuring that label

directions and other applicable requirements are followed rests

with the Forest Supervisor or a designate such as the Forest

Pesticide Use Coordinator.  For contracted projects, it is the

responsibility of the Contracting Officer to ensure that label

directions and all other requirements are followed.

REFERENCES:  FSM 2150; see references in "Best Management

Practice" Definition (05--2 and 3).

PRACTICE:  13.09 - Pesticide Application Monitoring and

Evaluation

OBJECTIVE:  To determine and document that pesticides have been

applied safely and to provide an early warning for any

contamination of water or non-target areas or resources.

EXPLANATION:  This practice provides feedback on the placement

accuracy, application amount, and any water contamination that

might occur from pesticide use, so as to minimize or eliminate

hazards to non-target areas or resources.  Monitoring and

evaluation methods include spray cards, dye tracing, and direct

measurement of pesticide in or near water.  Type of pesticide,

equipment, application difficulty, public concern, beneficial

uses, monitoring difficulty, availability of competent laboratory

analysis and applicable Federal, State, and local laws and

regulations are factors considered when determining the

monitoring and evaluation needs.

IMPLEMENTATION:  The monitoring and evaluation of pesticide

application is a component of SWCP 11.02.  The need for a

monitoring plan is identified during the Pesticide Use Planning

Process/NEPA process.  If determined necessary, this monitoring

and evaluation plan will consider the same items as in SWCP

11.02.  A technical staff familiar in pesticide monitoring will

evaluate and interpret the monitoring results in terms of

compliance, State water quality standards and adequacy of project

specifications.

REFERENCES:  FSM 2150; see references in "Best Management

Practice" Definition (05--2 and 3).

PRACTICE:  13.10 - Pesticide Spill Contingency Planning

OBJECTIVE:  To reduce contamination of water from accidental

pesticide spills.

EXPLANATION:  A contingency plan that contains a predetermined

organization and immediate actions to be implemented in the event

of a hazardous substance spill will be prepared.  The plan lists

notification requirements, time requirements for the

notification, how spills will be handled, and who will be

responsible for clean-up.  Factors considered for each spill are:

specific substance spilled, quantity, toxicity, proximity of

spill to waters, and the hazard to life, property, and the

environment.

IMPLEMENTATION:  The Pesticide Spill Contingency Plan will be

incorporated into the Project Safety Plan.  The NEPA process will

provide the means for including public and other agency

involvement in plan preparation.  The plan will list the

responsible authorities.

REFERENCES:  SWCP 11.07; FSH 2109.12, Pesticide Storage,

Transportation, Spills, and Disposal Handbook; FSM 6740, 7442,

7442, and 7460; Oil and Hazardous Substances Pollution

Contingency Plan for EPA Regions 8 and 10, 7/26/85; R-1 and R-4

Emergency and Disaster Plan; see references in "Best Management

Practice" Definition (05--2 and 3).

PRACTICE:  13.11 - Cleaning and Disposal of Pesticide Containers

and Equipment

OBJECTIVE:  To prevent water contamination and risk to humans

from cleaning and disposal of pesticide containers.

EXPLANATION:  The cleaning and disposal of pesticide containers

and equipment must be done in accordance with Federal, State, and

local laws, regulations, and directives, and in a manner which

will safeguard public health, the beneficial uses of water,

aquatic organisms and wildlife.  Containers are rinsed three

times, the rinse water applied on the project area as soon as

practical, and the containers taken to the designated disposal

site.  Application equipment is also rinsed and rinse water

applied to the project site before the equipment is moved from

the project area.

IMPLEMENTATION:  When the pesticide is applied by In-Service

personnel, the Forest or District Pesticide Use coordinator will

locate proper rinsing and disposal sites, and will arrange for

container disposal in an approved disposal site.  When the

pesticide is applied by a contractor, the contractor is

responsible for proper clean-up and container disposal in

accordance with label directions and Federal, State, and local

laws.

The Project Contracting Officer will document that the proper

disposal methods were followed.

REFERENCES:  FSM 2150; FSH 2109.12, Pesticide Storage,

Transportation, Spills, and Disposal Handbook; FSH 6709.11,

Health and Safety Code Handbook; FSH 6709.12, Safety and Health

Program Handbook; SWCP 11.07 and 11.08; see references in "Best

Management Practice" Definition (05--2 and 3).

PRACTICE:  13.12 - Protection of Water, Wetlands, and Riparian

Areas During Pesticide Spraying

OBJECTIVE:  To minimize the risk of a pesticide entering surface

or subsurface waters or affecting riparian areas, wetlands, and

other non-target areas.

EXPLANATION:  When applying pesticides, an untreated buffer strip

will be left alongside surface waters, wetlands, and riparian

areas.  Factors considered in establishing buffer strip widths

beyond minimums established by FSM and NEPA documents are:

beneficial water uses, adjacent land use, rainfall, temperature,

wind speed, wind directions, terrain, slope, soils and geology,

vegetative type, and aquatic life.  Other considerations include:

persistence, mobility, toxicity, and formulation of the

pesticide, method of applications, equipment used, spray

patterns, droplet size, application heights, and application

pattern.

IMPLEMENTATION:  Protected areas will be identified and mapped by

an interdisciplinary team and the Forest Pesticide Use

Coordinator during the NEPA process.  Protection of untreated

areas is the responsibility of the project supervisor for

In-Service projects and the Contracting Officer for contracted

projects.  The certified commercial applicators are briefed about

location of protection areas.  These areas are flagged or

otherwise marked when necessary to aid in boundary

identification.

REFERENCES:  FSM 2526, 2527, 2245, AND 2150; see references in

"Best Management practice" Definition (05--2 and 3).

PRACTICE:  13.13 - Controlling Pesticide Drift During Spray

Application

OBJECTIVE:  To minimize the risk of pesticide contaminating

non-target areas.

EXPLANATION:  Pesticide spray applications will be accomplished

according to a prescription that specifies the following:  areas

to be left untreated, buffer areas, type of spray and associated

materials, equipment and method to be used, droplet size, spray

height, application pattern, flow rate, terrain and

meteorological considerations.  Hand spraying, with less

associated risk, will have fewer application restrictions for

drift control than aerial applications.

IMPLEMENTATION:  The prescription is prepared by an

interdisciplinary team and the Forest or District Pesticide Use

Coordinator during the NEPA process.  The Line Officer is

responsible for designating a project supervisor who is

responsible for ensuring that the prescription is followed during

application and for terminating application if the standards are

exceeded.

REFERENCES:  FSM 2150 and 2245; SWCP 13.12; see references in

"Best Management Practice" Definition (05--2 and 3).

14 - TIMBER.  Timber harvesting and reforestation are the

culmination of several years of timber resource assessment and

detailed project planning. The actual physical activities consist

of felling, bucking, skidding, yarding, loading and hauling, site

preparation, tree planting, and other activities associated with

stand establishment.

Planning generally starts 5 to 10 years before the timber is sold

for harvesting.  First, the land must be suitable for and

allocated to timber resource activities in the Forest Plan.  The

proposed sale must follow the standards, guidelines, and

direction within the Forest Plan.  Next, a cumulative effects

feasibility analysis is conducted prior to including the project

on the implementation schedule to ensure that the project will

not impact soil, water, and other resources beyond acceptable

limits.  A position statement is then prepared which documents

the intent and schedule to harvest and offers tentative

harvesting alternatives.  The harvest proposal is next considered

by an interdisciplinary team which conducts an environmental

analysis. Based on the analysis results, the appropriate NEPA

document which is tiered to the Forest Plan is prepared

documenting the estimated effects of the proposed timber project.

This is used by the appropriate Line Officer in decision making.

When the sale plan is approved, the timber project is implemented

under terms of this decision. The Timber Sale Contract and

appraisal are then prepared by using contract provisions that

were selected to satisfy management constraints and mitigation

measures in the environmental analysis.  The timber is now

advertised and sold to the successful bidder.  Finally, the terms

of the Timber Sale Contract, including harvesting, are

administered on the ground by the certified Sale Administrator

and Forest Service Representative.

Success of a good harvest is measured by comparing the

on-the-ground results to the management objectives and

constraints identified and addressed in the environmental

analysis and Forest Plan.

PRACTICE:  14.01 - Timber Sale Planning

OBJECTIVE:  To incorporate soil and water resource considerations

into Timber Sale Planning.

EXPLANATION:  Timber Sale Planning is accomplished through the

NEPA process.  The environmental analysis will evaluate the

potential for impacts to and the cumulative effects on the soil

and water resources.  If a significant potential exists, the

environmental analysis will: (1) consider how to minimize

potential effects during and following the sale layout and

subsequent logging operations; (2) include mitigation of effects

for those treated areas where impacts are unavoidable; (3) and

identify environmentally sensitive areas where impacts from

proposed treatments can not be mitigated to conform with

standards.

IMPLEMENTATION:  During the NEPA process, an interdisciplinary

team will evaluate watershed characteristics and estimate

response of soil and water resources to proposed timber harvest

and related activities.  The NEPA process identifies mitigating

measures needed to protect soil and water resources.  The

subsequent contract will include provisions to meet water

quality, soils, and other resource protection requirements as

directed by the environmental analysis.

REFERENCES:  NFMA; NEPA; FSM 1950, 2431.1, 2431.2, 2511, and

2531; the Timber Sale Contract; individual Forest Plans; SWCP

11.01, 11.02, 11.03, 11.04, 11.05, 11.06, 11.07, and 11.14; FSH

2409.18, Sale Preparation Handbook.

PRACTICE:  14.02 - Timber Harvest Unit Design

OBJECTIVE:  To ensure that timber harvest unit design will secure

favorable conditions of water flow, maintain water quality and

soil productivity, and reduce soil erosion and sedimentation.

EXPLANATION:  This is an administrative and preventive practice.

The proposed timber harvest units are evaluated to estimate the

response on the affected watersheds.  This involves field

examination, utilization of existing data, analysis of potential

watershed response (that is, water yield and sediment yield

analysis), and professional judgment.  Characteristics to be

evaluated can include: (1) the recovery from past harvests; (2)

the allowable area that can be harvested; (3) the protection of

stream channels; (4) the erosion potential of the area; (5)

landform characteristics; (6) the number, size, shape, and

location of harvest units; (7) estimated location and size of

roads and skid trails; (8) logging system design; and (9) the

potential natural recovery rate of the watershed.  Where adverse

water quality and soil productivity impacts or undesirable

streamflows may result, the harvest unit design should be

modified, individual units deleted, and/or the natural recovery

rate accelerated by using watershed improvement measures.

IMPLEMENTATION:  The watershed evaluation of proposed timber

harvest is accomplished by the interdisciplinary team during the

NEPA process of Timber Sale Planning.  Prescriptions to assure

acceptable protection of soil and water resources are

incorporated into the environmental analysis.  On-the-ground

accomplishment of the direction in the environmental analysis is

carried out by the Presale Forester, the certified Sale

Administrator, the Project Engineer, and the administrator of

post sale slash disposal and cultural activities, with review by

technical resource staffs.  The need for monitoring and

evaluation will be identified in the environmental analysis when

necessary.

REFERENCES:  NFMA (Section 3091-6 F, III-V, and Section 219.10);

FSH 2409.15, Timber Sale Administration Handbook, and FSH

2409.18, Sale Preparation Handbook; FSM 2471 and 2409.13; SWCP

11.01, 11.02, 11.03, and 11.134.

PRACTICE:  14.03 - Use of Sale Area Maps for Designating Soil and

Water Protection Needs

OBJECTIVE:  To delineate the location of protection areas and

available water sources and to insure their recognition, proper

consideration, and protection on the ground.

EXPLANATION:  The following features are designated on the Sale

Area Map (and described in associated contract provisions), which

is an integral part of the Timber Sale Contract.

a.  Location of stream courses to be protected

(perennial, intermittent, and ephemeral).

b.  Wetlands and Riparian Areas (meadows, lakes, pot

holes, and so forth) to be protected.

c.  Boundaries of harvest units.

d.  Specified roads.

e.  Roads where log hauling is prohibited or restricted.

f.  Structural improvements.

g.  Areas for different skidding and yarding methods.

h.  Sources of rock for road work, riprapping, and so

forth.

i.  Water sources available for Purchaser's use.

j.  Other features required by Division "C" contract

Provisions.

k.  Domestic or public water supply source.

IMPLEMENTATION:  The interdisciplinary team identifies and

delineates these and other features on maps which are included in

the project design along with a discussion of each feature.  The

Presale Forester includes them on the Sale Area Map at the time

of contract preparation.  The features are reviewed on the ground

by the Purchaser and the certified Sale Administrator prior to

harvesting.

REFERENCES:  Timber Sale Contract Provisions B1.1, B6.5, B6.6,

C6.51 (R-1); FSM 2431.1 - .3 and 2471; FSH 25409.15, Timber Sale

Administration Handbook and FSH 2409.18, Sale Preparation

Handbook.

PRACTICE:  14.04 - Limiting the Operating Period of Timber Sale

Activities

OBJECTIVE:  To minimize soil erosion and sedimentation and loss

in soil productivity by insuring that the Purchaser conducts

their operations, including erosion control work, road

maintenance, and so forth, within the time period specified in

the Timber Sale Contract.

EXPLANATION:  Timber is purchased by individuals or companies who

either harvest the timber themselves or contract harvest to other

parties. Therefore, it is necessary to insure that purchasers

understand and adhere to soil and water resource recommendations

determined in the NEPA process.  This is accomplished by setting

forth the Purchaser's responsibilities in the Timber Sale

Contract.

The C6.3 "Plan of Operation" provision is required in all timber

Sale Contracts.  This provision states that the Purchaser must

submit a general plan of operation which will set forth planned

periods for and methods of road construction, timber harvesting,

completion of slash disposal, erosion control work, and other

contractual requirements.  Forest Service written approval of the

Plan of Operation is a prerequisite to commencement of the

Purchaser's operation.

The contract provision B6.31 "Operation Schedule" requires that

the Purchaser shall provide an annual schedule of anticipated

activities such as road maintenance and erosion control work.

Contract provision C6.31 "Limited Operating Period" may be used

in a contract to limit the Purchaser's operations to specified

periods of the year.  Provision B6.6 can be used to control

operations because of wet weather, high water, and so forth, in

order to protect resources.

IMPLEMENTATION:  Limited operating periods are identified and

recommended during the environmental analysis by the

interdisciplinary team.  The Presale Forester prepares the

contract to include provision C6.31.  Provisions B6.3, B6.31, and

C6.3 are all mandatory provisions of the timber Sale contract.

Provision C6.3 is only mandatory for sales over a two year

contract period.  The Purchaser must submit his general plan and

annual plans to the Forest Service.  The Purchaser may commence

operations only after written Forest Service approval of the

general plan under C6.3.

REFERENCES:  Timber Sale Contract Provisions B6.3, B6.31, B6.65,

B6.6, C6.3; FSM 2451 and 2453.2; FSH 2409.18, Sale Preparation

Handbook.

PRACTICE:  14.05 - Protection of Unstable Areas

OBJECTIVE:  To protect unstable areas and to avoid triggering

mass movements of the soil mantle and resultant erosion and

sedimentation.

EXPLANATION:  This management practice is an administrative and

preventive control.  Where unstable areas cannot be managed

without irreversible effects, they are taken out of suitable

forest land base in the Forest Plan and are reclassified as

unsuitable forest land.  Using existing harvesting technologies,

these lands are not managed for timber production because

irreversible damage to soil productivity or watershed conditions

would result.  Timber harvesting is deferred until improved

harvesting technologies are developed and proven.

IMPLEMENTATION:  The interdisciplinary team during the

environmental analysis identifies unstable areas by utilizing

input provided by various technical resource staffs.  Where

unstable areas are presently classified as suitable forest lands

and harvest cannot be designed without causing irreversible

effects, they are changed to the classification of unsuitable

forest lands.  If the interdisciplinary team determines that

current or prospective logging methods would result in

unacceptable watershed impact, the harvest is deferred.

REFERENCES:  FSM 2405.13.

PRACTICE: 14.06 - Riparian Area Designation

OBJECTIVE:  To minimize the adverse effects on Riparian Areas

with prescriptions that manage nearby logging and related land

disturbance activities.

EXPLANATION:  The Riparian Area is not a zone of exclusion, but

an area of closely managed activity.  It acts as (1) an effective

filter and absorptive zone for sediment; (2) maintains shade; (3)

protects aquatic and terrestrial riparian habitats; (4) protects

channel and streambanks; and (5) promotes floodplain stability.

As a preventive measure, roads, skid trails, landings, and other

timber harvesting facilities will be kept out of these areas when

feasible or at a prescribed distance from streams and wetlands.

Factors such as stream class, channel stability, sideslope

steepness, slope stability, resources dependent on these areas,

and standards, guidelines, and direction from Forest Plans are

considered in determining the management of activities and width

of Riparian Areas.  Fisheries habitat condition and its estimated

response to the proposed timber sale are also evaluated.

IMPLEMENTATION:  The Riparian Area requirements are identified

during the environmental analysis by the interdisciplinary team.

The timber sale project is designed to include site specific

recommendations for the prevention of sedimentation and other

stream damage from logging activities.  The environmental

analysis will provide for planning of harvests to insure

long-term health and revegetation of the Riparian Areas, while

meeting shading, debris recruitment, and other management

objectives.  As appropriate, monitoring and evaluation will be

identified in the environmental analysis documentation.  The

Presale Forester is responsible for the inclusion of the Riparian

Areas in the Timber Sale Contract and on the Sale Area Map.  The

certified Sale Administrator is responsible for contract

compliance during harvest operations.

REFERENCES:  FSM 2405.13, 2453.2, 2526, and 2471, NEPA; NFMA;

Timber Sale Contract Provision C6.5 (R-1), C5.421; SWCP 11.02,

11.05, 13.03, and 14.03; FSH 2409.15, Timber Sale Administration

Handbook and FSH 2409.18 Sale Preparation Handbook; see

references in "Best Management Practice" Definition (05--2 and

3).

PRACTICE:  14.07 - Determining Tractor Loggable Ground

OBJECTIVE:  To protect water quality from degradation caused by

tractor logging ground disturbance.

EXPLANATION:  This practicer is intended to minimize soil erosion

and subsequent sedimentation and water quality degradation.

Tractor loggable ground is a product of local slope limitations,

the volume of timber to be harvested from the site, and the soil,

land type, geologic, climatic, and hydrologic characteristics of

the site.  On-the-ground reconnaissance may be necessary.

IMPLEMENTATION:  The determination of tractor loggable ground is

performed by the interdisciplinary team during the transportation

planning portion of the Timber Sale Planning Process.  The

results of this determination are presented in a NEPA document.

These results are considered during the selection of logging and

silvicultural methods and are used in determining the intensity

of and restrictions for land disturbance activities, Provisions

in the Timber Sale Contract specify the areas and conditions upon

which tractors can operate (such as, requiring tractor logging on

snow in riparian areas).

REFERENCES:  14.08 - Tractor Skidding Design

OBJECTIVE:  To minimize erosion and sedimentation and protect

soil productivity by designing skidding patterns to best fit the

terrain.

EXPLANATION:  This is a preventive practice.  The watershed

factors that are considered include slope, aspect, soil

stability, vegetative cover, Riparian Areas, meadows, and other

factors that may affect the flood and sediment yield potential of

the land.  The careful control of skidding patterns serves to

avoid onsite and downstream channel impacts, the build up of

destructive runoff flows, erosion in sensitive watershed areas

such as meadows and Riparian Areas, and a reduction in soil

productivity.

Two complementary methods of protecting soil and water resources

by tractor skid trail design are:

a.  End-Lining.  This method involves winching the log

directly out of the sensitive areas (such as meadows and

Riparian Areas) with a cable operated from outside the

sensitive area.  In this manner, logs can be removed from

the sensitive areas while avoiding encroachment by heavy

equipment and associated site damage.

b.  Felling to the Lead.  This method involves felling

trees toward a predetermined skid pattern.  This

procedure facilitates an uncomplicated approach of the

tractor operating between the log and the skid trail.

Soil disturbance and compaction are consequently lessened

and residual stand and site impacts are minimized.

IMPLEMENTATION:  For skid trail design, sensitive areas are

identified and evaluated in the environmental analysis during the

Timber Sale Planning Process.  If necessary, prescriptions can be

included in the Timber Sale Contract through the use of a special

provision.  The certified Sale Administrator then executes the

prescription on the ground by locating the skid trails with the

timber purchaser or by agreeing to the Purchaser's proposed

locations prior to construction.

REFERENCES:  Timber Sale Contract Provision B6.422, B6.424, and

C6.6; FSM 2524 and 2451; FSH 2409.18, Sale Preparation Handbook

and FSH 2409.15, Timber Sale Administration handbook; see

references in "Best Management Practice' Definition (05--2 and

3); in R-4, R-4 Technical Guide - Erosion Prevention and Control

on Timber Sale Areas, May 1981.

PRACTICE:  14.09 - Suspended Log Yarding in Timber Harvesting

OBJECTIVES:  To protect the soil from excessive disturbance and

accelerated erosion and to maintain the integrity of the Riparian

Area and other sensitive watershed areas.

EXPLANATION:  Suspended log yarding includes all yarding systems

which suspend logs either partially or wholly off of the ground.

These systems include hilead, skyline, helicopter, and balloon

yarders.  The systems are used on steep or unstable slopes and in

Riparian Areas where tractors cannot operate.  All of the systems

result in less soil disturbance since heavy machinery is not used

over the sale area.  In most cases, these systems require fewer

roads because of longer skidding distances.  Fewer roads and less

soil disturbance will result in less impact on the soil and water

resource.

IMPLEMENTATION:  Areas where suspended log yarding is to be used

are determined and identified in the environmental analysis.  The

specific systems are included in the contract and designated on

the Sale Area Map by the Presale Forester.  The certified Sale

Administrator oversees the project operation using the guidelines

and standards established in the Timber Sale Contract with

reference to the environmental analysis documentation.

REFERENCES:  Timber Sale Contract Provisions B6.42 and C6.4; FSH

2409.18, Sale Preparation handbook and 2409.15, Timber Sale

Administration Handbook; see references in "Best Management

Practice" Definition (05--2 and 3); in R-4, R-4 Technical Guide -

Erosion Prevention and Control on Timber Sale Areas, May 1981.

PRACTICE:  14.10 - Log Landing Location and Design

OBJECTIVE:  To locate landings in such a way as to avoid soil

erosion and water quality degradation.

EXPLANATION:  This practice is both administrative and

preventive.  Location of all landing clearing limits shall be

agreed to by the Forest Service and Purchaser prior to

construction.  The following criteria are used in evaluating

landings:

a.  The cleared or excavated size of landings shall not

exceed that needed for safe and efficient skidding,

decking, and loading operations.  Every landing must meet

the safety requirements of the Occupational Safety and

Health Administration.

b.  Where a choice exists, landing locations are selected

which involve the lease amount of excavation and the

least erosion potential.

c.  Where possible, landings are located near the points

of ridges so that felled timber lying between drainages

can be skidded to the lading without crossing channels or

impacting Riparian Areas.

d.  Landings are located where the lease number of skid

roads are required and sidecast will neither enter

drainages nor damage other sensitive areas.

e.  If possible, landings are positioned such that the

skid road approach will be nearly level.

f.  Locate landings to minimize the number of tractor

roads entering a given landing.

g.  Some landings are designed and constructed as part of

specified roads.

h.  Landings are shaped to drain in a planned direction

and manner to minimize erosion and sediment delivery to

stream courses.

i.  Major landings, such as those for helicopter logging,

are treated to restore soil infiltration rates when use

is completed.

IMPLEMENTATION:  Landing locations chosen by the Purchaser must

be agreed to by the certified Sale Administrator.  The Sale

Administrator can negotiate with the Purchaser's representative

to select mutually acceptable landing locations.  Acceptable

lands must meet the above criteria.  Should agreement not be

reached, the decision of the Forest Service shall prevail within

the limitations of law.

REFERENCES:  Timber Sale Contract Provisions B6.422; FSH 2409.15,

Timber Sale Administration Handbook; see references in "Best

Management Practice" Definition (05--2 and 3); In R-4: R-4

Technical Guide - Erosion Prevention and Control on Timber Sale

Areas, May 1981.

PRACTICE:  14.11 - Log Landing Erosion Prevention and Control

OBJECTIVE:  To reduce the impacts of erosion and subsequent

sedimentation from log landings through the use of mitigating

measures.

EXPLANATION:  This practice employs administrative, preventive,

and corrective controls to meet the objective.  Timber Sale

Contract requirements provide for erosion prevention and control

measures on all landings.  Provisions are made in the Timber Sale

Contract for landings to have proper drainage.  After landings

have served the Purchaser's purpose, the Purchaser shall ditch or

slope the landings to permit the drainage and dispersion of

water.  Provisions are also made for revegetation.  Other

provisions may include scarifying, smoothing and sloping

construction of drainage ditches, prevention of water draining

off roads from reaching a landing, spreading slash, covering with

wood chips, or applying straw mulch.  Unless agreed, cut and fill

banks around landings shall be sloped to remove overhangs and

otherwise minimize erosion.  The specific work needed on each

landing will depend on the actual ground conditions.

IMPLEMENTATION:  The Presale Forester and certified Sale

Administrator assess the need for stabilization with technical

resource staff input as needed.  It is the responsibility of the

certified Sale Administrator to insure that this practice is

properly implemented on the ground.

REFERENCES:  Timber Sale Contract Provisions B6.6, B6.63, B6.422,

C6.4, C6.6, C6.601; FSM 2405.13; FSH 2409.15, Timber Sale

Administration Handbook; see references in "Best Management

Practice" Definition (05--2 and 3); In R-4: R-4 Technical Guide -

Erosion Prevention and Control on Timber Sale Areas, May 1981.

PRACTICE:  14.12 - Erosion Prevention and Control Measures During

Timber Sale Operations

OBJECTIVE:  To ensure that the Purchaser's operations shall be

conducted reasonably to minimize soil erosion.

EXPLANATION:  timber is purchased by individuals or companies who

either harvest the timber themselves or contract harvest to other

parties.  Therefore, it is necessary to insure that purchasers

understand and adhere to soil and water resource prescriptions

arrived at in the Timber Sale Planning Process.  This is

accomplished by setting forth the Purchaser's responsibilities in

the Timber Sale Contract.

IMPLEMENTATION:  Equipment shall not be operated when ground

conditions are such that excessive impacts will result.  The

kinds and intensity of control work done by Purchaser shall be

adjusted to ground and weather conditions and the need for

controlling runoff.  The certified Sale Administrator is

responsible for insuring that the Purchaser conducts his

operations according to the Timber Sale contract.  Erosion

control work shall be kept current immediately preceding expected

seasonal periods of precipitation or runoff.  If the Purchaser

fails to do erosion control work prior to any seasonal period of

precipitation or runoff, the Forest Service may temporarily

assume responsibility for the work and any unencumbered deposits

(performance bonds) may be used by the Forest Service to do the

work.

REFERENCES:  Timber Sale Contract Provisions B4.225, C6.3,

C6.312, C6.6, C6.601; FSM 2451, 2453.2, and 2533, SWCP 14.04; FSH

2409.15, Timber Sale Administration Handbook; see references in

"Best Management Practice" Definition (05--2 and 3); In R-4: R-4

Technical Guide - Erosion Prevention and Control on Timber Sale

Areas, May 1981.

PRACTICE:  14.13 - Special Erosion Prevention Measures on Areas

Disturbed by Harvest Activities

OBJECTIVE:  To prevent erosion and sedimentation on disturbed

areas.

EXPLANATION:  Where soil is disturbed by Purchaser's operations

on tractor roads, skid trails, landings, temporary road fills,

and other logging sites, the purchaser shall provide adequate

treatment to protect exposed soils.  This may be accomplished by

spreading slash or wood chips, mulching, establishing an adequate

cover of grass or other vegetating acceptable to the Forest

Service, or preforming other agreed stabilization measures.  This

provision is to be used only for sales which contain special soil

stabilization problems and are not expected to be revegetated by

the normal methods prescribed under the standard Timber Sale

Contract.

IMPLEMENTATION:  The interdisciplinary team will identify areas

needing special stabilization measures during the Timber Sale

Planning Process.  Treatment areas will be verified during sale

layout and the estimate of work needed will be carried to the

timber sale appraisal.  Specific locations to be treated will be

designated on the ground by the Forest Service.  The Forest

Service, upon request, shall provide advice as to soil

preparation and the application of suitable seed mixtures, mulch,

and fertilizer, and the timing of such work.  It is the

responsibility of the certified Sale Administrator to make sure

that stabilization work is done correctly and in a timely manner.

REFERENCES:  Timber Sale Contract Provisions B6.6, C6.6, and

C6.601; SWCP 14.04; FSH 2409.15, Timber Sale Administration

Handbook and FSH 2409.18, Sale Preparation handbook.

PRACTICE:  14.14 - Revegetation of Areas Disturbed by Harvest

Activities

OBJECTIVES:  To establish a vegetative cover on disturbed sites

to prevent erosion and sedimentation.

EXPLANATION:  Where soil has been severely disturbed by

Purchaser's operations and establishment of vegetation is needed

to minimize erosion, the Purchaser shall take appropriate

measures normally used to establish an adequate cover of grass or

other vegetation acceptable to Forest Service or take other

agreed stabilization measures.  This measure is applied in

contracts where it is expected that disturbed soils in parts of

the sale area will require vegetative cover for stabilization and

the problems will not be mitigated by other contract provisions.

IMPLEMENTATION:  An estimate of the need is included in the

environmental analysis and timber sale appraisal.  The Forest

Service shall annually designate on the ground the disturbed

soils, such as logging areas and temporary roads, that must be

treated.

The Forest Service, upon request, shall provide advice as to soil

preparation and the application of suitable seed mixtures, mulch,

and fertilizer, and the timing of such work.  It is the

responsibility of the certified Sale Administrator to make sure

that revegetation work is done correctly and in a timely manner.

REFERENCES:  Timber Sale Contract Provisions B6.6, C6.6, and

C6.601; SWCP 14.14; FSH 2409.15, Timber Sale Administration

Handbook and FSH 2409.18, Sale Preparation Handbook.

PRACTICE:  14.15 - Erosion Control on Skid Trails

OBJECTIVE:  To protect water quality by minimizing erosion and

sedimentation derived from skid trails.

EXPLANATION:  This practice employs preventive controls to reach

the objective.  The Timber Sale Contract requires the

installation of erosion control measures on skid trails, tractor

roads, and temporary roads.  Normally, the work involves

constructing cross ditches and water spreading ditches.  Other

methods such as backblading may be agreed to in lieu of cross

drains.  Gras seeding may also be required by a "C" provision

which may be added to the Timber Sale Contract.  Areas in need of

erosion control measures are shown on the Sale Area Map and

designated on the ground annually as logging and temporary access

construction progresses.

IMPLEMENTATION:  Location of all erosion control measures are

designated and agreed to on the ground by the certified Sale

Administrator.  The Timber Sale Administration Handbook contains

guidelines for spacing of cross drains, construction techniques,

and cross drain heights.  The Sale Administrator can use these

guidelines on the ground to identify site specific preventive

work to be required of the Purchaser.  The Purchaser is obligated

to complete and maintain erosion control work as specified in

contract provisions.

REFERENCES: Timber Sale Contract Provisions B6.422, B6.6, B6.66,

and C6.601; FSH 2409.15, Timber Sale Administration Handbook; see

references in "Best Management Practice" Definition (05--2 and

3); In R-4: R-4 Technical Guide - Erosion Prevention and Control

on Timber Sale Areas, May 1981.

PRACTICE:  14.16 - Meadow Protection During Timber Harvesting

OBJECTIVE:  To avoid damage to the ground cover, soil, and water

in meadows.

EXPLANATION:  This is an administrative and preventive action.

Unauthorized operation of vehicular or skidding equipment on

meadows designated on Sale Area Maps and marked on the ground is

prohibited.  Vehicular or skidding equipment shall not be used on

meadows except where roads, landings, and tractor roads are

approved.  Unless otherwise agreed, trees felled into meadows

shall be removed by end-lining, and resulting logging slash shall

also be removed.

IMPLEMENTATION:  The concerns and constraints mentioned above are

set forth in Timber Sale Contract requirements.  Damage to

meadows, stream courses, and Riparian Areas caused by

unauthorized Purchaser's operations shall be repaired by the

Purchaser in a timely and agreed manner to restore and prevent

further damage.

This provision shall not apply when the damage is caused by fire

suppression activities.

REFERENCES:  Timber Sale Contract Provisions B6.61; E.O. 11990,

Protection of Wetlands; SWCP 13.03, 14.03, 14.06, 14.08, and

14.17; FSH 2409.15, Timber Sale Administration Handbook; see

references in "Best Management Practice" Definition (05--2 and

3).

PRACTICE:  14.17 - Stream Channel Protection (Implementation and

Enforcement)

OBJECTIVES:  (1)  To protect the natural flow of streams; (2) to

provide unobstructed passage of stormflows; (3) to reduce

sediment and other pollutants from entering streams; and (4) to

restore the natural course of any stream as soon as practicable

if the stream is diverted as a result of timber management

activities.

EXPLANATION:  This management practice employs administrative,

preventive, and corrective measures to meet the objectives.  The

following points are fundamental to protecting stream channels:

a.  Location and motif of stream crossings must be agreed

to prior to construction.  This is done when locations of

skid trails, tractor roads, and temporary roads are

agreed on by the Forest Service and the Purchaser.

b.  Purchaser shall repair all damage to a streamcourse

caused by Purchaser's operations, including damage to

banks and channel, to an acceptable condition as agreed

to by the certified Sale Administrator and Purchaser's

representative.

c.  All project debris shall be removed from streamcourse

and in an agreed manner that will cause the least

disturbance.

d.  Wheeled or track laying equipment shall not operate

within 50 feet slope distance of the apparent high water

mark of streamcourses designated for protection in the

Timber Sale Contract, except as agreed to by the

certified Sale Administrator and the Purchaser.

e.  When ground skidding systems are employed, logs will

be end-lined out of streamside and Riparian Areas,

Equipment is permitted to enter streamside areas only at

locations and times agreed to by the certified Sale

Administrator and the Purchaser.

f.  Water bars and other erosion control structures will

be located to prevent water and sediment from being

channeled into streamcourses, and to dissipate

concentrated flows.

g.  Material from temporary road and skid trail stream

crossings is removed and streambanks restored to an

acceptable condition, as agreed to by the certified Sale

Administrator and Purchaser's representative.

h.  Logs or products shall be fully suspended above the

ground when crossing streamcourses designated for

protection in the Timber Sale Contract.

IMPLEMENTATION:  The certified Sale Administrator works with the

Purchaser's representative to insure that the Timber Sale

Contract clauses covering the above items are carried out on the

ground.  Technical resource staffs can be consulted to help the

Sale Administrator with decisions.  In the event Purchaser causes

debris to enter streamcourses in amounts which adversely affect

the natural flow of the stream, water quality, or fishery

resources, Purchaser shall remove such debris within 48 hours and

in an agreed manner that will cause the least disturbance to

streamcourses.

REFERENCES:  FSM 2405.13 and 2452;  FSH 2409.15, Timber Sale

Administration Handbook; Timber Sale Contract Provisions B6.5,

B6.6, C6.5 (R-1), C6.6, C6.51 (R-1), and C6.53 (R-1); see

references in "Best Management Practice" Definition (05--2 and

3); In R-4: R-4 Technical Guide - Erosion Prevention and Control

on Timber Sale Areas, May 1981.

PRACTICE:  14.18 - Erosion Control Structure Maintenance

OBJECTIVE:  To insure that constructed erosion control structures

are stabilized and working effectively.

EXPLANATION:  Erosion control structures are only effective when

they are in good repair and stable condition.  Once the erosion

control structures are constructed and seeded, there is a

possibility that they may not become adequately vegetated or they

may become damaged from subsequent harvest activities.  It is

necessary to provide follow-up inspection and structural

maintenance in order to avoid these problems and insure adequate

erosion control.

IMPLEMENTATION:  During the period of the Timber Sale Contract,

the Purchaser shall provide maintenance of soil erosion control

structures constructed by the Purchaser until they become

stabilized, but not for more than one year after their

construction.  After 1 year, erosion control work needed is

accomplished through Watershed Improvement practices (SWCP

11.03).

The Forest Service may agree to perform such structure

maintenance under B4.225 (Cooperative Deposits), if requested by

the Purchaser, subject to agreement on rates.  If the Purchaser

fails to do seasonal maintenance work, the Forest Service may

assume the responsibility and charge the Purchaser accordingly.

REFERENCES:  Timber Sale Contract Provisions B6.6, B6.66, and

B4.225; FSH 2409.15, Timber Sale Administration Handbook.

PRACTICE:  14.19 - Acceptance of Timber Sale Erosion Control

Measures Before Sale Closure

OBJECTIVE:  To assure the adequacy of required erosion control

work on timber sales.

EXPLANATION:  The effectiveness of soil erosion prevention and

control measures is determined by the results found after sale

areas have been exposed one or more years to the elements.

Although a careful check is required before a timber sale is

closed to assure that planned erosion work has been completed to

the standard prescribed, the erosion prevention work done in

previous years should be periodically inspected during the life

of the timber sale.  These inspections will help determine

whether the planned work was adequate, if maintenance work is

needed, the practicability of the various treatments used, and

the necessity for modifying present standards or procedures.

IMPLEMENTATION:  "Acceptable" erosion control means only minor

deviation from established standards, provided no major or

lasting impact is caused to soil and water resources.  Certified

Sale Administrators will not accept as complete erosion control,

measures which fail to meet this criteria.

REFERENCES:  FSM 2451, 2452, 2453, and 2456; Timber Sale Contract

Provisions B6.6, B6.63, B6.64, B6.65, B6.66, and C6.6; SWCP

11.02; FSH 2409.15, Timber Sale Administration Handbook.

PRACTICE:  14.20 - Slash Treatment in Sensitive Areas

OBJECTIVE:  To protect water quality by protecting sensitive

tributary areas from degradation which would result from using

mechanized equipment for slash disposal.

EXPLANATION:  Special slash treatment may be prescribed in

sensitive areas to facilitate slash disposal without use of

mechanized equipment.  Meadows, wetlands, Riparian Areas, and

landslide areas are typically sensitive areas where equipment use

is normally prohibited.  Slash treatment methods are indicated

for each harvest unit on the Slash Treatment Map and referenced

in associated contract provisions.

IMPLEMENTATION:  Sensitive areas needing protection are

identified by the interdisciplinary team in the Timber Sale

Planning Process.  Results are documented during the

environmental analysis and identified in the Timber Sale Contract

and on the Slash Treatment Map.  The certified Sale Administrator

inspects the treatment for correct and satisfactory slash

disposal accomplishment.

REFERENCES:  Timber Sale Contract; SWCP 14.08, 14.16, and 14.17;

FSH 2409.15, Timber Sale Administration Handbook and FSH 2409.18,

Sale Preparation Handbook.

PRACTICE:  14.21 - Non-recurring "C" Provisions For Soil and

Water Protection

OBJECTIVE:  To exercise the option of inserting non-recurring

(Special) "C" provisions into the Timber Sale Contract to protect

soil and water resources, where standard "B" or "C" provisions do

not apply or are inadequate to protect watershed values.

EXPLANATION:  Non-recurring "C" provisions are sometimes needed

to meet management objectives on a particular sale area.  They

require Regional Forester approval and may only be included in

the sale for which approval was given.  This practice can be used

for a variety of special situations which may occur on any timber

sale.  There are no standard or set provisions that can be

referenced, since each Special "C" provision is unique and

specific to one sale.

IMPLEMENTATION:  The need for non-recurring "C" provisions is

identified during the Timber Sale Planning Process and

environmental analysis by the interdisciplinary team.  The

Presale Forester prepares the non-recurring "C" provision and

submits it through Line Officers to the Regional Forester for

approval.  The Regional Forester insures that the wording

complements the Timber Sale Contract and returns it to the

District with approval.  The non-recurring "C" provision is

applied by the certified Sale Administrator in the same manner as

the standard contract provisions.

REFERENCES:  FSM 2431.2, 2431.3, and 2431.4; FSH 2409.18, Sale

Preparation Handbook.

PRACTICE:  14.22 - Modification of the Timber Sale Contract

OBJECTIVE:  To modify the Timber Sale contract if new

circumstances or conditions indicate that the timber sale will

cause irreversible damage to soil, water, or watershed values.

EXPLANATION:  Once timber slaes are sold, they are havested as

described in the Timber Sale Contract.  However, it may be

necessary to modify a timber sale contract because of new

concerns about the effects of the sale on soil and water

resources.

IMPLEMENTATION:  If evidence indicates that unacceptable impacts

would occur to soil and water resources if the sale was harvested

as planned, the Forest Service Representative will request the

Contracting Officer to gain Regional Forester advice and approval

to proceed with a resource environmental modification, mutual

cancellation, or unilateral cancellation of the Timber Sale

Contract.  Once the decision to take action is approved by the

Regional Forester, the appropriate Line Officer will assign an

interdisciplinary team to make recommendations for

implementation.

REFERENCES:  NFMA, Section 6; Timber Sale Contract Provision

B8.3; SWCP Handbook 10.40, Feedback Mechanism.

PRACTICE:  14.23 - Reforestation Requirement

OBJECTIVE:  To promote prompt reforestation and to limit

disturbance on areas with limited regeneration potential.

EXPLANATION:  Forested lands will not be planned for timber

harvest production objectives unless there is reasonable

assurance that these sites can be adequately restocked within 5

years after final harvest, based on existing technology and

knowledge.  The 5-year time frame begins at different times for

different silvicultural systems. Restocking is adequate when the

cut area contains the minimum number, size, distribution, and

species composition of  regeneration as specified in the

silvicultural prescription.  Site preparation, species selection,

and seedling protection are critical factors that need

consideration fore successful regeneration.  The implementation

of this practice affects soil and water resources by stabilizing

solids, increasing ground cover, improving infiltration, and

reducing surplus water yields.

In meeting overall resource objectives, some timber stands may be

harvested to achieve multiple resource objectives other than

timber production.  Delaying immediate regeneration may be

desirable for meeting the overall objectives.

IMPLEMENTATION:  During the Timber Sale Planning Process, the

interdisciplinary team assesses the resource objectives of the

proposed areas and the capability to achieve reforestation within

the prescribed period.  The environmental analysis  contains the

interdisciplinary team determinations and recommendations.  Past

and present reforestation activities will be evaluated.

REFERENCES:  FSH 2409.25b, Reforestation Handbook; NFMA (36 CFR

219.27 c(3)); FSM 2472; SWCP 11.02.

15 - ROADS AND TRAILS.  Transportation systems are developed top

serve the transportation needs of National Forest System lands

and resource management programs.  Transportation planning is a

complex process that assures that roads and trails are planned,

located, designed, constructed, and maintained to meet these

long-term Forest management needs and objectives.  General

objectives are developed by legislation, policy, and directives

and addressed in Forest Plans.  Specific objectives are developed

by an interdisciplinary team during the NEPA process in project

planning.

Transportation planning is normally conducted on a Forest or

area-wide basis with the objective of locating roads for

individual timber sale areas and long range transportation needs.

Alternative road corridors are mapped or flagged on the ground

after consideration of management objectives and resource

information.  The interdisciplinary team reviews these corridors

and makes recommendations for road design criteria, modifications

of the corridor locations, use of existing roads, and upgrading

inadequate roads.

The environmental effects, economic analysis, and recommendations

from the interdisciplinary team are considered prior to selecting

a preferred alternative by the Line Officer.  The appropriate

NEPA document resulting from the environmental analysis

establishes design criteria which is used to develop design

standards, erosion control measures, and the road operation and

maintenance standards.  Additional interdisciplinary team input

may be required in the design phase to ensure meeting the

management objectives.

During road construction, the Contract Officer and/or Engineering

Representative shall be assigned to the project.  These personnel

assure that the project is constructed according to contract

specifications and drawings.  Interdisciplinary team members may

be requested to review proposed design modifications during

construction.

Subsequent to and upon road completion, periodic reviews of

selected projects are made by interdisciplinary team members to

evaluate the construction performance, the effectiveness of

specific design features or treatments to control erosion, and

the appropriateness of the level of maintenance.  These reviews

provide a feedback mechanism to improve future road construction

and maintenance by modifying design or erosion control practices.

PRACTICE:  15.01 - General Guidelines for Transportation Planning

OBJECTIVE:  To introduce soil and water resource considerations

into Transportation Planning.

EXPLANATION:  Transportation Planning shall be included as an

integral part of the Forest Planning process.  In some cases, a

transportation facility may itself require an appropriate NEPA

document.  Transportation systems will be planned to achieve an

optimum balance of minimum environmental effects at minimum,

overall long-term cost, while meeting the land and resource

management objectives.

Transportation Planning shall develop and evaluate alternative

methods of providing needed transport services.  Alternative

methods may include alternative modes, routes, geometric

features, materials standards, or some combination thereof.

Evaluation shall include determining the social, environmental,

and economic characteristics of each alternative.  Selection of a

recommended alternative shall be by the responsible Line Officer

and the decision shall be documented.  No implementing activities

shall be undertaken prior to the approval by the responsible

official.

IMPLEMENTATION:  An interdisciplinary team during the NEPA

process will evaluate watershed characteristics and estimate the

response of soil and water resources to proposed transportation

alternatives and activities.  The NEPA process will identify

mitigating measures needed to protect soil and water resources.

The subsequent contract will include provisions to meet water

quality, soil, and other resource protection requirements as

directed by the environmental analysis.

REFERENCES:  FSM 1950, 7700, and 7710; NFMA; NEPA; individual

Forest Plans; SWCP 11.01, 11.02, 11.03, 11.04, 11.05, 11.06,

11.09, and 11.14; see references in "Best Management Practice"

Definition (05--2 and 3); In R-4: R-4 Technical Guide - Erosion

Prevention and Control on Timber Sale Areas, May 1981.

PRACTICE:  15.02 - General Guidelines for the Location and Design

of Roads and Trails

OBJECTIVE:  To locate and design roads and trails with minimal

soil and water resource impact while considering all design

criteria.

EXPLANATION:  There are several considerations which must be

incorporated into the location and design of roads and trails.

These factors directly affect protection of water quality, soil,

and other resource values.  The following coordination

instructions apply to all transportation activities:

a.  Area Transportation Analysis and project planning

will be completed using an interdisciplinary process, and

the appropriate NEPA document will be prepared and tiered

to the Forest Plan.  Area Transportation analysis is an

extremely effective tool to reduce overall road mileages

and, thus, minimize potential resource impacts.

b.  Location, design, and construction activities shall

utilize appropriate technical resource staffs, when

needed, to evaluate effects of transportation development

and operations, and recommend mitigating measures to

minimize adverse impacts.

c.  Roads and trails will be located and designed to

facilitate completion of the transportation system, serve

specific resource management needs, fit the terrain, and

minimize damage to improvements and resources.  Fragile,

unstable, sensitive, or special areas should be avoided.

d.  Roads and trails should be designed based on traffic

and safety requirements of anticipated use and to meet

the overall transportation plan.  The design shall

incorporate features to prevent or minimize soil movement

and sedimentation as well as undue disruption of water

flow.

e.  Stream crossing structures shall be designed to

provide the most efficient drainage facility consistent

with resource protections, importance of the road, legal

obligations, and total costs.  The design may involve a

hydrologic analysis to determine runoff rates and

volumes, flood conditions, velocities, scour, open

channel shapes, approach topography, materials-foundation

condition, and fish passage, as required.  An economic

comparison of various flood frequencies versus structure

sizes and types is also considered.

f.  Locate and design roads and trails to drain naturally

by appropriate use of out-sloping or in-sloping with

cross drainage and grade changes, where possible.  Relief

culverts and roadside ditches will be designed whenever

reliance upon natural drainage would not protect the

running surface, excavation, or embankment.  Road and

trail drainage should be channeled to effective buffer

areas to maximize sediment deposition prior to entry into

live water.

IMPLEMENTATION:  during the environmental analysis, an

interdisciplinary team will be used to insure that management

needs, objectives, requirements, and controls are incorporated in

the location and design of roads and trails.  Mitigation measures

needed to protect soil and water resources will be identified in

the NEPA process.  Contract provisions will be prepared that meet

the soil and water resource protection requirements.

REFERENCES:  FSM 7710 and 7720; NFMA; FSH 7709.56, Road

Preconstruction Handbook; NEPA; SWCP 15.01; see references in

"Best Management Practice" Definition (05--2 and 3); In R-4: R-4

Technical Guide - Erosion Prevention and Control on Timber Sale

Areas, May 1981.

PRACTICE:  15.03 - Road and Trail Erosion Control Plan

OBJECTIVE:  To prevent, limit, and mitigate erosion,

sedimentation, and resulting water quality degradation prior to

the initiation of construction and maintenance activities through

effective contract administration during construction and timely

implementation of erosion control practices.

EXPLANATION:  Land disturbing activities usually result in at

least short-term erosion.  Poorly designed, located, constructed,

and maintained roads and trails are usually responsible for the

majority of stream sedimentation problems associated with forest

management practices.  By effectively planning for erosion

control, sedimentation can be minimized.

Roads and trails require a variety of erosion control measures.

Many erosion control practices will not only protect water

quality but also maintain road prism integrity, reduce

maintenance costs, and improve trafficability.  The location of

the road or trail with respect to streams, beneficial uses of

that water, soil, and geologic information and other site factors

govern the degree of stabilization required.  Stabilization

usually includes a combination of practices that promotes the

reestablishment of vegetation on exposed slopes, provides

physical protection to exposed surfaces, prevents and downslope

movement of soil, or controls road drainage.

Since a newly constructed road is most susceptible to erosion

from seasonal precipitation, the timing of erosion control

practices is of primary concern.  Those practices that can be

accomplished concurrent with road counteractions shall be favored

as a means of immediate protection of the water resource.

IMPLEMENTATION:  Erosion control objectives and detailed

mitigation measures are developed using an interdisciplinary

approach during the environmental analysis.  These measures and

objectives shall be reflected in the contract specifications and

provisions for the road or trail.  When standard specifications

do not provide the degree of mitigation required, special project

specifications will be developed by the interdisciplinary team.

Prior to the start of construction, the Purchaser shall submit a

schedule for proposed erosion control work as required in the

Standard Specifications.  The schedule shall include all erosion

control items identified in the specifications.  The schedule

shall consider erosion control work necessary for all phases of

the project.  The Purchaser's construction schedule and plan of

operation will be reviewed in conjunction with the erosion

control plan to insure their compatibility before any schedules

are approved.  No work will be permitted on the project until all

schedules have been approved by the Contracting Officer.

The Contracting Officer or Engineering Representative shall

ensure that erosion control measures are implemented according to

the approved schedule and are completed in an acceptable fashion.

Field reviews and on-site inspection by the Line Officer and/or

Forest Engineer will identify any additional erosion control

measures required to protect the streams that were not recognized

during planning or design.  Necessary correction measures shall

be implemented immeidately through normal administrative

channels.

The following items may be considered as erosion control measures

when constructed in a timely manner.  To maximize effectiveness,

erosion control measures must be in place and functional prior to

seasonal precipitation or runoff.

a.  Measures to reestablish vegetation on exposed soils.

This is usually accomplished by seeding suitable grass

and legume species in conjunction with mulching and

fertilization.  In some situations, treatments may

include tree seedling planting or sprigging of other

woody species.

b.  measures which physically protect the soil surface

from detachment or modify the topography to minimize

erosion.  These treatments may include the use of dust

oil or gravel on the road travelway and ditches and the

use of mulches, riprap, erosion mats, and terracing on

cuts, fills, and ditches.  Temporary waterbars in areas

of uncompleted roads and trails can be effectively

utilized to reduce sedimentation.

c.  Measures which physically inhabit the downslope

movement of sediments to streams.  These may include the

use of slash filter windrows on or below the fill slopes,

baled straw in ditches or below fillslopes, catch basins

at culvert inlets, and sediment basin slash filter

windrows may be utilized in live water drainages where

fish passage is not required and where peakflows are low.

d.  Measures that reduce the amount of soil disturbance

in or near streams.  These measures may include

dewatering culvert installation or other construction

sites, and immediate placement of permanent culverts

during road pioneering.  Temporary pipes should not be

allowed unless positive control of sedimentation can be

accomplished during installation, use, and removal.

e.  Measures that control the concentration and flow of

surface and subsurface water.  These may include

insloping, outsloping, ditches, cross drains, under

drains, trenches, and so forth.

REFERENCES:  FSM 7721, 7722, and 7723; Timber Sale Contract

Provisions B6.31, B6.5, B6.6, and C6.3; see references in "Best

Management Practice" Definition (05--2 and 3); In R-4: R-4

Technical Guide - Erosion prevention and Control on Timber Sale

Areas, May 1981; Cook, M.J. and J.G. King. 1983.  Construction

Cost and Erosion Control Effectiveness of Filter Windrows on Fill

Slopes.  USDA Forest Service Research Note, INT-335; SWCP

Handbook 10.40 Feedback mechanism; FSH 7709.56b, Drainage

Structures Handbook.

PRACTICE:  15.04 - Timing of Construction Activities

OBJECTIVE:  To minimize erosion by conducting operations during

minimal runoff periods.

EXPLANATION:  Erosion and sedimentation are directly related to

runoff.  Scheduling operations during periods when the

probabilities for rain and runoff are low is an essential element

of effective erosion control.  Purchasers shall schedule and

conduct operations to prevent erosion and sedimentation.

Equipment shall not be operated when ground conditions are such

that excessive impacts will result.  Such conditions are

identified by the Contracting Officer or Engineering

Representative with assistance from technical resource staffs as

needed.  Temporary erosion control measures may be required to

prevent, control, and mitigate erosion and sedimentation.

In addition, it is important to keep permanent erosion control

work as current as practicable with ongoing operations.

Construction of drainage facilities and performance of other

contract work which will contribute to the control of erosion and

sedimentation shall be carried out concurrent with earthwork

operations or as soon thereafter as practicable.  Limitation of

the amount of area being graded at a site at any one time, and

minimization of the time that an area is laid bare should be a

consideration in contract preparation.  Erosion control work must

be kept current when road construction occurs outside of the

normal operating season.

IMPLEMENTATION:  Detailed erosion control measures are developed

by an interdisciplinary team during the environmental analysis

and are incorporated into the contract specifications.

Compliance with plans, specifications, and the operating plan is

assured by the Contracting Officer and/or Engineering

Representative.

REFERENCES:  FAR 52.236-15; Timber Sale Contract Provisions C6.3,

C6.36, and B6.31; SWCP 15.03; see references in "Best Management

Practice" Definition (05--2 and 3).

PRACTICES:  15.05 - Slope Stabilization and Prevention of Mass

Failures

OBJECTIVES:  To reduce sedimentation by minimizing the chances

for road-related mass failures, including landslides and

embankment slumps.

EXPLANATION:  Road construction in mountainous terrain requires

cutting and loading natural slopes which may lead to landslides

and/or embankment failures depending on the soil strength,

geology, vegetation, aspect, and groundwater regime.  Landslides

and embankment failures are undesirable because they interrupt

traffic, are costly to repair, visually unacceptable, and

generate large quantities of erosion and sedimentation.

Roadways may drastically change the subsurface drainage

characteristics of a slope.  Since the angle and height of cut

and fill slopes increase the risk of instability, it is often

necessary to provide subsurface drainage to avoid moisture

saturation and subsequent slope failure.  Where it is necessary,

horizontal drains, drainage trenches, or drainage blankets may be

used to lower the subsurface water levels and to prevent

groundwater from entering embankments.

In areas with high landslide potential, the composition and

characteristics of embankments may be controlled since they are

essentially engineered structures.  Care must be taken to prevent

the incorporation of construction slash or other organic material

and the embankment material should be placed by one of the

following methods.

a.  Layer placement.

b.  Controlled compaction.

c.  Controlled compaction using density controlled

strips.

d.  Compaction controlled with a special project

specification.

IMPLEMENTATION:  In areas with intrinsic slope stability

problems, appropriate technical resource staffs must be involved

in an interdisciplinary approach to route location.  Sufficient

subsurface investigation and laboratory testing must be performed

to general design parameters and mitigating features which will

meet the constraints and requirements developed through the NEPA

process.

In contracted projects, compliance with environmental analysis

requirements and controls which have been provided for in the

specifications is assured by enforcement of the Timber Sale

Contract Provisions by the Contracting Officer and/or Engineering

Representative.

REFERENCES:  FSM 7706.11, 7706.12, 7710, and 7720; Standard

Specifications 203, 212, 605, 613, 619, 630, and 631; Timber Sale

Contract Provisions B6.31, B6.62, C5.2, C5.4, and C6.36; FSH

7709.11, Transportation Engineering Handbook and FSH 7709.56b,

Drainage Structures Handbook; see references in "Best Management

Practice" Definition (05--2 and 3); In R-4: R-4 Technical Guide -

Erosion prevention and Control on Timber Sale Areas, May 1981.

PRACTICE:  15.06 - Mitigation of Surface Erosion and

Stabilization of Slopes

OBJECTIVE:  To minimize soil erosion from road cutslopes,

fillslopes, and travelway.

EXPLANATION:  Road construction exposes fresh, loose soil to the

erosive force of wind, water, and traffic.  Surface erosion from

roads is greatest during the first year following construction.

If is desirable to minimize erosion due to the adverse impacts on

water quality, vehicle maintenance, road maintenance, and safety.

Erosion can occur on cutslopes, fillslopes, and/or travelway.

Each of the three surfaces has unique erosion consideration which

are outlined below:

Stabilization-Mitigation

Surface        General Characteristics               Measures

Cutslope       Steeper, undisturbed, and     Vegetative and

mechanical

more sterile soil     stabilization

Fillslope      Flatter, loose, and more Vegetative and mechanical

fertile soil          stabilization

Travelway      Flattest, compact (dueSurface Stabilization

to traffic)

Vegetative measures include seeding herbaceous species (grass,

legumes, or browse species)  or the planting of brush or trees.

Fertilization, mulching, watering, and/or erosion netting and

fabrics may be required to insure success.

Mechanical measures include construction of slash windrows, straw

bale dams, erosion netting and fabrics, terraces, or benching,

riprapping, tackifiers, and gunnite.

Surface stabilization includes watering, dust oiling, dust

pallatives, aggregate layer, bituminous surface treatment, or

asphalt paving depending on traffic, soils, and climatic factors.

An integrated system of collection control, and dispersion of

concentrated surface water is very important in order to prevent

erosion on fillslopes, travelways, and natural slopes below cross

drains and culverts.

IMPLEMENTATION:  During the NEPA process, detailed mitigation

measures and slope stabilization techniques are incorporated into

the design package by the interdisciplinary team.  Compliance

with environmental analysis controls and requirements is obtained

by the Contracting Officer and/or Engineering Representative

through the Standard Specifications and/or Timber Sale Contract

Provisions.

REFERENCES:  FSM 7706.11, 7706.12, 7706.13, and 7720; Standard

Specifications 50.4, 203, 204, 206A, 210, 212, 412, 619, 625,

626, 629, and 630; Timber Sale Contract Provisions B6.31, B6.6,

B6.62, B6.65, B6.66, C5.2, C5.23, C5.4, C5.441, C5.46, R-1 C6.36,

C6.52, C6.6, C6.601, and C6.622; SWCP 15.03 and 15.04; see

references in "Best Management Practice" Definition (05--2 and

3); In R-4: R-4 Technical Guide - Erosion prevention and Control

on Timber Sale Areas, May 1981.

PRACTICE:  15.07 - Control of Permanent Road Drainage

OBJECTIVE:  To minimize the erosive effects of concentrated water

and the degradation of water quality by proper design and

construction of road drainage systems and drainage control

structures.

EXPLANATION:  Degradation of water quality by sediment and the

erosive effects of surface runoff can be minimized by stabilizing

the road prism and adjacent disturbed areas from erosion.

Velocities in the road drainage system can be dissipated before

entry into the natural system by design and construction of

control structures.

A number of measures can be used alone or in combination to

control the detrimental effects of road drainage.  Methods used

to control water and reduce erosion may include: properly spaced

culverts, cross drains, water bars, rolling dips, energy

dissipaters, aprons, gabions, and armoring of ditches and drain

inlets and outlets.  Dispersal of runoff can also be accomplished

by rolling the grade, insloping, outsloping crowning, contour

trenching, installation of water spreading ditches, and so forth.

IMPLEMENTATION:  Project location, design criteria, drainage

control features, and detailed mitigation measures are determined

during the NEPA process by an interdisciplinary approach.

Compliance with plans, specifications, and operating plans is

assured by the Contracting Officer or Engineering Representative.

REFERENCES:  SWCP 15.02, 15.03, 15.06; Timber Sale Contract

Provisions B6.6, B6.66, C6.3, C6.6, and C6.601; FSM 7721, 7723,

7706.11, and 7706.12; FSH 7709.56b, Drainage Structures Handbook;

see references in "Best Management Practice" Definition (05--2

and 3); In R-4: R-4 Technical Guide - Erosion prevention and

Control on Timber Sale Areas, May 1981.

PRACTICE:  15.08 - Pioneer Road Construction

OBJECTIVE:  To minimize sediment production and mass wasting

associated with pioneer road construction.

EXPLANATION:  Pioneer roads are built to allow equipment access

for construction of planned roadways.  Pioneering is usually done

within the corridor of the planned road.  To meet the objective

of minimizing sediment, the following constraints should be

followed:

a.  Construction of pioneer roads shall be confined to

the roadway construction limits unless otherwise approved

by the Contracting Officer.

b.  Pioneering shall be conducted so as to prevent

undercutting of the designated final cut slope, prevent

avoidable deposition of materials outside the designated

roadway limits, and accommodate drainage with temporary

culverts or log crossings unless approved otherwise.

c.  Erosion control work will be completed concurrent

with construction activity or prior to the wet season.

d.  Live streams crossed by pioneer roads will be

dewatered by diversion devices.

IMPLEMENTATION:  The Contracting Officer or Engineering

Representative are responsible for enforcing contract

specifications, drawings, and plans.  The Purchaser is

responsible for submitting for approval an operating plan that

includes erosion control measures.

REFERENCES:  SWCP 15.02, 15.03, 15.04, 15.05, and 15.06; Standard

Specification 201, 203; FSM 7721; Timber Sale Contract Provisions

B6.6, B6.65, C6.3; see references in "Best Management Practice"

Definition (05--2 and 3).

PRACTICE:  15.09 - Timely Erosion Control Measures on Incomplete

Roads and Streamcrossing Projects

OBJECTIVE:  To minimize erosion of and sedimentation from

disturbed ground on incomplete projects.

EXPLANATION:  The best drainage design and erosion control

measure can be useless if projects are incomplete at the end of

the normal operating season.  Affected areas can include roads,

fills, tractor trails, skid trails, landings, streamcrossings,

bridge excavations, and firelines.  Preventive measures include:

a.  The removal of temporary culverts, culvert plugs,

diversion dams, or elevated streamcrossing causeways.

b.  The installation of temporary culverts, side drains,

flumes, cross drains, diversion ditches, energy

dissipaters, dips, sediment basins, berms, debris racks,

or other facilities needed to control erosion.

c.  The removal of debris, obstructions, and spoil

material from channels and floodplains.

d.  Grass seeding, planting deep rooted vegetation,

and/or mulching.

IMPLEMENTATION:  Protective measures must be applied to all areas

of disturbed, erosion-prone, unprotected ground that is not to be

further disturbed in the present year.  When conditions permit

operations outside the Normal Operating Season, erosion control

measures must be kept current with ground disturbance, to the

extent that the affected area can be rapidly closed, if the

weather conditions deteriorate.  Areas must not be abandoned for

the winter with remedial measures incomplete.

Project location and mitigative measures are developed in the

NEPA process using an interdisciplinary approach.  Compliance

with environmental analysis controls and requirements, contract

specifications, and operating plans are assured by the

Contracting Officer or Engineering Representative.

REFERENCES:  FSM 7721; Standard Specification 206; Timber Sale

Contract Provisions B6.31, B6.6, C6.6; FAR 52.213-3, 52.236-15,

and 4G-52.235-107; SWCP 15.03 and 15.04; see reference in "Best

Management Practice" Definition (05--2 and 3).

PRACTICE:  15.10 - Control of Road Construction Excavation and

Sidecast Material

OBJECTIVE:  To reduce sedimentation from unconsolidated excavated

and sidecast material caused by road construction,

reconstruction, or maintenance.

EXPLANATION:  Unconsolidated material from road construction is

frequently exposed on cut and fillslopes, can be difficult to

stabilize, and represents a major sediment source.  The area of

exposed material is often reduced when the cut and fillslopes and

roadbed are constructed to the lines, grades, and dimensions

shown on the drawings or designated on the ground.  The

Contracting Officer and/or Engineering Representative insures

that construction is within tolerances, particularly on sections

of high erosion or stability hazards.  In some cases layer

placement and/or benching may be necessary for stabilization and

to obtain the proper dimensions and fill slope ratios.  End

hauling and retaining structure may be necessary to prevent thin

layers of consolidated material from being sidecast on steep

slopes where compaction is impractical. Prior to commencing

construction, reconstruction, or maintenance activities, waste

areas should be located where excess material can be deposited

and stabilized.  If waste areas are located on steep slopes,

sidecast materials should be consolidated and stabilized.

Disposal of slide debris should be in areas where it can be

stabilized.  The purchaser may be required to remove excess

material not placed according to the contract and/or restore

damaged areas.

Normal erosion control such as seeding should be supplemented

with special mitigation measures such as jute netting, erosion

cloth, mulching, slash windrows, sediment ponds, hay bale dams,

and rock gabions, when such measures are determined necessary for

local conditions.

IMPLEMENTATION:  Project location, selected disposal areas, and

mitigative measures are developed through the NEPA process, using

an interdisciplinary approach.  Forest Service supervisors are

responsible for insuring that In-Service projects meet design

standards and project requirements.  For contracted projects,

compliance with specifications and operating plans is assured by

the Contracting Officer and/or Engineering Representative.

REFERENCES:  FSM 7720.3. 7706.11, and 7721; Standard

Specification 203; SWCP 15.03, 15.05, 15.06, and 15.09; Timber

Sale Contract Provisions C6.221 and C5.4; see reference in "Best

Management Practice" Definition (05--2 and 3).

PRACTICE:  15.11 - Servicing and Refueling of Equipment

OBJECTIVE:  To prevent contamination of waters from accidental

spills of fuels, lubricants, bitumens, raw sewage, wash water,

and other harmful materials.

EXPLANATION:  During servicing or refueling, pollutants from

logging or road construction equipment may enter a watercourse.

This threat is minimized by selecting service and refueling areas

well away from wet areas and surface watercourses and by using

berms around such sites to contain spills.

IMPLEMENTATION:  The Contracting Officer, Engineering

Representative, or certified Sale Administrator will designate

the location, size and allowable uses of service and refueling

areas.  They will also be aware of actions to be taken in cause

of a hazardous spill, as outlined in the Forest Hazardous

Substance Spill Contingency Plan (SWCP 11.07).

REFERENCES:  SWCP 11.07; Timber Sale Contract Provisions B6.34,

C6.341, and C6.34; Standard Specifications 204.42; FSH 2409.15,

Timber Sale Administration Handbook; see reference in "Best

Management Practice" Definition (05--2 and 3).

PRACTICE:  15.12 - Control of Construction in Riparian Areas

OBJECTIVE:  To minimize the adverse effects on Riparian Areas

from roads and trails.

EXPLANATION:  Except at designated stream crossings, road and

trail construction will avoid placing fill materials or

structures in Riparian Areas that will directly affect the

ecological values of the stream.  Occasionally exceptions may

occur.  These instances should be identified by the

interdisciplinary team in the NEPA process and the final location

designed to create the minimum impact possible.  Factors such as

stream class, channel stability, sideslope steepness, slope

stability, resources dependent on these areas and standards,

guidelines, and direction from Forest Plans are considered in

determining the management of activities and width of Riparian

Areas.  Mitigation measures should be used to the optimum to

insure minimum impact.

IMPLEMENTATION:  Riparian Area requirements are identified during

the environmental analysis by the interdisciplinary team.  The

road or trail project is designed to include site specific

recommendations for the prevention of sedimentation and other

stream damage from road/trail activities.  As appropriated,

monitoring and evaluation will be identified in the NEPA

documentation.  Forest Service supervisors are responsible for

insuring that In-Service projects meet design standards and

project requirements.  On contracted projects, compliance with

project requirements, contract specifications and operating plans

is assured by the Contracting Officer or Engineering

Representative.

REFERENCES:  SWCP 11.02, 14.03, and 14.06 FSM 7706.11 7706.12,

7706.14 and 7710; Timber Sale Contract Provisions B6.5, B6.61,

C6.51, and C6.52; see reference in "Best Management Practice"

Definition (05--2 and 3).

PRACTICE:  15.13 - Controlling In-Channel Excavation

OBJECTIVE:  To minimize stream channel disturbances and related

sediment production.

EXPLANATION:  During the construction of roads and the

installation of stream crossing structures, it may be necessary

for construction equipment to cross, operate in, or operate near

streamcourses.  However, this will be allowed only at crossings

designated by the Forest Service or as necessary in the

construction or removal of culverts and bridges.  Close

coordination is needed with the Purchaser to minimize damage to

the stream and aquatic resources.

Also, excavation during the installation of streamside structures

should be accomplished in the following manner in order to

protect water quality.  Unless otherwise approved, no excavation

shall be made outside of caissons, cribs, cofferdams, or sheet

piling, and the natural stream bed adjacent to the structure

shall not be disturbed without approval of the Engineering

Representative or Contracting Officer.  If any excavation or

dredging is made at the site of the structure before caissons,

cribs, or cofferdams are sunk in place, all such excavations will

be restored to the original ground surface or the stream bed will

be protected with suitable stable material.  Material deposited

within the stream area from foundation or other excavation shall

not be discharged directly into live streams but shall be pumped

to settling areas shown on the drawings or approved by the

Engineering Representative or Contracting Officer.  If the

channel is damaged during construction, it should be restored as

nearly as possible to its original configuration without causing

additional damage to the channel.  Excavations for stream

crossings should be started early enough in the summer so that

the installation is complete before winter.

IMPLEMENTATION:  Project location and mitigation measures are

developed by the interdisciplinary team during the NEPA process

and are inserted into the contract.  Compliance with the

management requirements, contract specifications, and operating

plans is assured by the Contracting Officer or Engineering

Representative.

REFERENCES:  FAR 52.213-3, 52.236-15, and 4G-52.236-107; FSM 7721

and 2502.1; Standard Specifications 206; Timber Sale Contract

Provisions C6.36, C6.52, and B6.5; EO 11988, Flood Plain

Management; SWCP 11.04, 11.05, 14.03, 14.06, and 15.12; see

reference in "Best Management Practice" Definition (05--2 and 3);

In R-4: R-4 Technical Guide - Erosion Prevention and Control on

Timber Sale Areas, May 1981

PRACTICE:  15.14 - Diversion of Flows Around Construction Sites

OBJECTIVE:  To minimize downstream sedimentation by insuring that

all stream diversions are carefully planned.

EXPLANATION:  Flow must sometimes be guided or piped around

project sites.  Typical examples are bridge and dam construction.

Flow in streamcourses will be diverted if the Forest Service

deems it necessary for the Purchaser to do the job.  Such a

diverted flow shall be restored to the natural streamcourse as

soon as practicable and, in any event, prior to the major storm

season or fish migration season.  Stream channels impacted by

construction activity will be restored to their natural grade,

conditions, and alignment as soon as possible.

IMPLEMENTATION:  The interdiscipliary team during the

environmental analysis will identify where diversions are

required and the project design will include mitigative measures

to protect fishery values and other downstream uses.  The NEPA

process may require project review by other Federal, State,

and/or local agencies and private parties, to insure that all

factors are considered.  For In-Service projects, Forest Service

supervisors are responsible for implementing design standards and

management requirements.  On contracted projects, compliance with

contract specifications and operating plans is assured by the

Contracting Officer or Engineering Representative.

REFERENCES:  Timber Sale Contract Provisions B6.5, C6.3, C6.51,

C6.52, and C6.6; FSM 2505.1 and 7721; FAR 52.213-3, 52.236-15,

and 4G-52.236-107; FSH 7709.56b, Drainage Structures Handbook; EO

11988, Flood plain Management; SWCP 11.04, 11.05, 14.03, 14.06,

15.12, and 15.13; see reference in "Best Management Practice"

Definition (05--2 and 3).

PRACTICE:  15.15 - Streamcrossings on Temporary Roads

OBJECTIVE:  To keep temporary roads from unduly damaging streams,

disturbing channels, or obstructing fish passage.

EXPLANATION:  Culverts, temporary bridges, low water crossings,

or fords will be required on temporary roads at all locations

where it is necessary to cross streamcourses.  Such facilities

shall be designed and installed to provide unobstructed stream

flow and fish passage, and to minimize damage to streamcourses.

The number of crossings shall be kept to the minimum needed for

access.  Channel crossings should generally be as perpendicular

to streamcourses as possible.  Streambank excavation shall be

kept to the minimum needed for use of the crossings.

Crossing facilities shall be removed when the facility has served

its purpose and is no longer needed.  Fills associated with these

facilities shall also be removed.

IMPLEMENTATION:  Project location and protective measures are

developed by the interdisciplinary team during the NEPA process.

Those developed by the Purchaser will be reviewed and approved by

the certified Sale Administrator or Contracting Officer.  Forest

Service supervisors are responsible for insuring that In-Service

projects meet management objectives and requirements.  For

contracted projects, compliance with specifications and operating

plans is assured by the Contracting Officer, certified Sale

Administrator, or Engineering Representative.

REFERENCES: Timber Sale Contract Provisions B6.5, B6.62, B6.65,

C6.3, C6.51, C6.52, C6.6, and C6.753; FSH 2409.15, Timber Sale

Administration Handbook, FSM 2505.1 and 7721; FAR 4G-52.236-107;

SWCP 11.04, 11.05, 14.03 14.06, 14.17, 15.12, 15.13, and 15.14;

see reference in "Best Management Practice" Definition (05--2 and

3); In R-4: R-4 Technical Guide-Erosion Prevention and Control on

Timber Sale Areas, May 1981.

PRACTICE:  15.16 - Bridge and Culvert Installation (Disposition

of Surplus Material and Protection of Fisheries)

OBJECTIVE:  To minimize sedimentation and turbidity resulting

from excavation for in-channel structures.

EXPLANATION:  Excavation in or near streamcourse is a common

requirement for the installation of bridges, culverts, and other

streamside structures such as weirs, check dams, riprapping, or

fish passage structures.  Surplus material should not obstruct

the streamcourse including the floodplain nor the efficiency of

the associated structure.  Preventive measures include:

a.  Diverting stream flow around project sites during

construction in order to minimize erosion and downstream

sedimentation.

b.  Easily erodible material shall not be deposited into

live streams.

c.  Any material stockpiled on floodplains shall be

removed before rising waters reach the stockpiled

material.

d.  During excavation in or near the streamcourse, it may

be necessary to use suitable coffer dams, caissions,

cribs or sheet piling.  This will usually be the case

where groundwater is contributing a significant amount of

water to the immediate excavation area.  If any of the

aforementioned devices are used, they will be practically

watertight and no excavation will be immediately outside

of them.  If water from subsurface strata is not

significant, pumping may be used, provided the sediment

from the pumped water can be disposed of where it will

not re-enter the stream during high flows.

e.  Water pumped from foundation excavation shall not be

discharged directly into live streams, but shall be

pumped into settling ponds.

f.  When needed, bypass roads should be located to have

the minimal disturbance on the streamcourse.

g.  The construction activity in or adjacent to the

stream will be limited to specific times to protect

beneficial water uses (such as fisheries).

IMPLEMENTATION:  Project location and detailed mitigative

measures are developed in the environmental analysis and are

detailed in the appropriate NEPA document using an

interdisciplinary team approach.  Forest Service supervisors are

responsible for insuring that In-Service projects meet the design

standards.  For contracted projects, compliance with contract

specifications and operating plans is assured by the Contracting

Officer or Engineering Representative.

REFERENCES:  FAR 52.213-3, 52.236-15, and 4G-52.235-107; Standard

Specifications 206 and 206A;  Timber Sale Contract Provision

C6.5; FSM 2505.1; see reference in "Best Management Practice"

Definition (05--2 and 3).

PRACTICE:  15.17 - Regulation of Borrow Pits, Gravel Sources and

Quarries

OBJECTIVES:  To minimize sediment production from borrow pits,

gravel sources, and quarries, and limit channel disturbance in

those gravel sources suitable for development in floodplains.

EXPLANATION:  Borrow pits, gravel sources, and quarries are often

susceptible to erosion due to steep side slopes, lack of

vegetation, and/or their proximity to water courses.  Whenever

possible, the top soil should be removed and stockpiled for use

as surface dressing during the reclamation phases, prior to

excavation of the site.

Drainage design for the excavation should consider temporary

erosion control measures during the life of the material source

and permanent drainage control measures after the site has been

rehabilitated.  When excavation of the site has been completed on

all or part of the area, and the site will not be used again, the

sides will be sloped, graded, or scaled and the general pit are

smoothed and stabilized.  Oversized material, if planned for

future use as riprap or derrick rock, should be stockpiled.  If

not, it should be scattered or buried.  Finer material, if

available, should be spread over the bottom of the pit prior to

spreading stockpiled or imported topsoil. Seeding, mulching,

and/or planting should be carried out.  If the site will be used

again, the above requirements will be limited to those essential

to resource protection between uses.  Access roads to the site

should also have temporary or permanent drainage design for

erosion control depending on the life of the pit or the roads

should be ripped, drained, blocked to traffic, and seeded,

mulched, and/or planted unless other uses are planned.

Borrow pits and gravel sources located in floodplains require

special attention.  Material deposited in floodplains or along

channel sections during storm runoff often provide excellent and

inexpensive sand and gravel.  Because of easy access, these

deposits are often in demand.  With careful planning and design,

these deposits can be removed with minimal impact on water

resources.  Under some circumstances, sand and/or gravel removal

may alter stream flow characteristics and consequently affect

stream channel stability and create a new sediment source.

Excavation of these deposits within stream channels should be

limited to those above the waterline which is normal for the

period of the excavation.  If the borrow area is subject to

periodic flooding, leveling, shaping, or other special drainage

features shall be provided.

Excavation in flood plains should not take place below the water

table unless sediment basins are built to contain or catch the

resulting sediment.  Sediment basins should not be subject to

washouts.  If excess sediment accumulates in basins, it should be

excavated to clean the basin and the sediment removed to an

approved site.

Wash water or waste from concrete batching or aggregate

operations shall not be allowed to enter streams prior to

treatment by filtration, flocculations, settling and/or other

means.  The potential pollution of adjacent water resources by

blasting agent in quarry operations shall be addressed in the pit

operation plan.

IMPLEMENTATION:  Project feasibility, location, suitability, and

the limits for disturbance and sediment production will be

identified through the NEPA process using an interdisciplinary

approach.  Detailed mitigative measures are developed by the

design engineer using criteria from the environmental analysis

and through consultation with technical resource staffs when

needed.  Development of borrow pits or gravel sources in the

floodplain will be coordinated with State and local agencies.

Special-use permits issued for borrow pits, gravel sources, and

quarries will include the above requirements and District Rangers

or their representatives are responsible for insuring compliance.

Forest Service supervisors are responsible for implementing

In-Service projects to design standards.  For contracted

projects, compliance with management requirements,

specifications, and operating plans is assured by the Contracting

Officer or Engineering Representative.

REFERENCES:  FSM 2511, 2502.1, 7706.11, 7706.12, 7721; FSH

7709.11, Transportation Engineering Handbook, and FSH 7709.56,

Road Preconstrcution Handbook; FAR 52.236-09; Standard

Specifications 203, 210, 611, 624, 625, 626, and 629;  Timber

Sale Contract Provision B6.31, B6.6, B6.62, B6.65, and B6.66,

C5.2, C5.23, C5.4, C6.36, C6.52, C6.6, C6.601, C6.622; Water

Pollution Control Act, 33 USC 466; NEPA; Montana Water Quality

Act and Hardrock Atc; Idaho Dredge and Placer Mining Act, Title

47, Ch. 13; SWCP 11.04, 11.05, 15.03; see reference in "Best

Management Practice" Definition (05--2 and 3).

PRACTICE:  15.18 - Disposal of Right-of-Way and Roadside Debris

OBJECTIVE:  To insure that debris generated during road

construction is kept out of streams and to prevent slash and

debris from subsequently obstructing channels.

EXPLANATION:  As a preventive measure, counteractions debris and

other newly generated slash developed along roads near streams

shall be disposed of by the following means as applicable:

a.  On-Site.

(1)  Windrowing (SWCP 15.03).

(2)  Scattering

(3)  Burying

(4)  Chipping

(5)  Disposal in Cutting Units

(6)  Piling and Burning

b.  Removal to agreed upon locations.

c.  A combination of the above.

d.  Large limbs and cull logs may be bucked into

manageable lengths and piled alongside the road for

fuelwood.

IMPLEMENTATION:  Criteria for the disposal of Right-of-Way and

roadside debris are established in the environmental analysis by

an interdisciplinary team.  Project location and detailed

mitigative measures are also developed.  Forest Service

supervisors are responsible for insuring that In-Service projects

meet design standards.  For contracted projects, compliance with

plans, specifications, and operating plans is assured by the

Contracting Officer, Engineering Repreeesentative, or certified

Sale Administrator.

REFERENCES:  Timber Sale Contract; SWCP 13.05, 14.20, and 15.03;

see reference in "Best Management Practice" Definition (05--2 and

3).

PRACTICE:  15.19 - Streambank Protection

OBJECTIVE:  To minimize sediment production from streambanks and

structural abutments in natural waterways.

EXPLANATION:  The stabilization of stream embankments disturbed

by the construction of a water crossing or a raodway fill

parallel to a streamcourse, is necessary to prevent erosion of

the material during natural stream flow.  To reduce sediment and

channel bank degradation, it is necessary to incorporate

"armoring" in the design of a structure to allow the water course

to stabilize after construction.  Riprap, gabion structures, and

other measures are commonly used to armor stream banks and

drainage ways from the erosive forces of flowing water.  These

measures must be sized and installed in such a way that they

effectively resist erosive water velocities.  Stone used for

riprap should be free from weakly structured rock, soil, organic

material and materials of insufficient size, all of which are not

resistant to stream flow and would only serve as sediment

sources.  Outlets for drainage facilities in erodible soils

commonly require riprapping for energy dissipation.'

IMPLEMENTATION:  Project location and detailed mitigative

measures are developed through the NEPA process to meet the

objectives and requirements of the management.  Forest Service

supervisors are responsible for implementing In-Service projects

to design standards and management requirements.  For contracted

projects, compliance with contract specifications and operating

plans is assured by the Contracting Officer or Engineering

Representative.

REFERENCES:  SWCP 15.03; see references in "Best Management

Practice" Definition (05--2 and 3); In R-4: R-4 Technical

Guide-Erosion Prevention and Control on Timber Sale Areas, May

1981.

PRACTICE:  15.20 - Water Source Development Consistent With Water

Quality Protection

OBJECTIVE:  To supply water for road construction and maintenance

and fire protection while maintaining water quality.

EXPLANATION:  Water source development is normally needed to

supply water for road construction, dust control, mixing surface,

compaction, planting and for fire control requirements of the

timber Purchaser.  Water source development should aim toward the

construction of durable, long term water sources rather than the

construction of hasty, expedient developments.  Permanently

designed sources, such as tanks, will result in the lowest, long

term impact to the affected streams.

Other considerations in the development of water sources should

be:

a.  Downstream flow should not be reduced so as to

detrimentally affect aquatic resources, fish passage, or

other uses.

b.  Temporary cofferdams should be constructed of

sandbags containing sand or clean gravel, or of other

materials and means which will not induce sediment in the

stream.

c.  Overflow should go directly back into the stream.

d.  All temporary facilities for gathering water will be

removed prior to causing any resource damage.

IMPLEMENTATION:  Certified Sale Administrators and Engineering

Representatives in conjunction with technical resource staffs

should evaluate streams in which water developments may be

constructed.  Project location and detailed mitigative measures

are developed by the interdisciplinary approach during the

environmental analysis.  Forest Service supervisors are

responsible for insuring that In-Service projects meet design

standards and management requirements.  For contracted projects,

compliance with contract specifications and the operating plan is

assured by the Contracting Officer and/or engineering

Representative.

Any damage to resources caused by Purchaser's operations or fire

suppression activities shall be retired by purchaser or fire

suppression crews in a timely and agreed manner to the extent

practical to restore and prevent further resource damage.

REFERENCES:  Standard Specification 207; Timber Sale Contract

Provisions; SWCP 14.03; Timber Sale Administration Handbook (FSH

2409.150; see references in "Best Management Practice" Definition

(05--2 and 3).

PRACTICE:  15.21 - Maintenance of Roads

OBJECTIVE:  To maintain all roads in a manner which provides for

soil and water resource protection by minimizing rutting,

failures, sidecasting, and blockage of drainage facilities.

EXPLANATION:  Roads normally deteriorate because of use and

weather impacts.  This deterioration can be minimized through

proper and timely maintenance and/or restriction of use (SWCP

11.09).  All system roads will be maintained to at lease the

following level:  Provide the basic custodial care required to

protect the road investment and to insure that damage to adjacent

land and resources is held to minimum.  This level of maintenance

often requires an annual inspection to determine what work, if

any, is needed to keep drainage functional and the road stable.

This level is the normal prescription for roads that are closed

to traffic.  As a minimum measure, maintenance must protect

drainage facilities and runoff patterns.  Higher levels of

maintenance may be chosen to reflect greater use or resource

administrative needs.  Additional maintenance measures could

include resurfacing, outsloping, clearing debris from dips and

cross drains, armoring of ditches, spot rocking, and drainage

improvement.

Maintenance needs will be reflected in an annual road maintenance

plan developed to include all roads under Forest Service control.

Individual maintenance plans will be developed annually for each

timber sale and for each cost share area outlining performance

standards, responsibilities, and timing.

For maintenance of roads on active timber sales, the Forest

Service and the Purchaser shall annually agree at the beginning

of the operating season on an Annual Road Maintenance Plan

outlining responsibilities and timing.  If the road is subjected

to commercial use, the Forest Service may collect deposits to

facilitate road maintenance and to equitably assess maintenance

cost of each user.

In addition to timely performance of regular maintenance, each

Forest should have an emergency action plan which identifies

procedures to be used during periods of high runoff to protect

facilities and reduce resource damage.

IMPLEMENTATION:  The work is controlled through the Forest

Engineer who is responsible for the development of the annual

road maintenance plan based on condition surveys.  Maintenance

levels are established for each road and maintenance performed in

accordance with standards.  On timber sales, maintenance is a

Purchaser responsibility and compliance with standards is assured

by the Contracting Officer, Engineering Representative, or

certified Sale Administrator.  On system roads outside of active

timber sales, road maintenance is insured by the Engineering

Representative or Contracting Officer.

REFERENCES:  FSM 7730.2, 7732, and 7735; FSH 2409.15, Timber Sale

Administration Handbook and FSH 7709.15, Transportation System

Maintenance Handbook; Timber Sale Contract provision C5.4; SWP

11.09; see references in "Best management Practice" Definition

(05--2 and 3).

PRACTICE:  15.22 - Road Surface Treatment to Prevent Loss of

Materials

OBJECTIVE:  To minimize the erosion of road surface materials and

consequently reduce the likelihood of sediment production.

EXPLANATION:  Unconsolidated road surface material is susceptible

to erosion during precipitation events.  Likewise, dust derived

from road use may settle onto adjacent water bodies.  On timber

sale roads, the Purchaser shall undertake measures to prevent

excessive loss of road material if the need for such action has

been identified.  Road surface treatments may include:  water,

dust, oiling, penetration oiling, sealing aggregate surfacing,

chip-sealing, or paving.

IMPLEMENTATION:  Project location and detailed mitigative

measures are developed by an interdisciplinary approach to meet

environmental analysis criteria.  Forest Service supervisors are

responsible for insuring that In-Service projects meet design

standards and management requirements.  On contracted projects,

compliance with contract specifications, and operating plans is

assured by the Contracting Officer or Engineering Representative.

REFERENCES:  Timber Sale Contract; FSH 2409.15, Timber Sale

Administration Handbook.

PRACTICE:  15.23 - Traffic Control During Wet Periods

OBJECTIVES:  To reduce the potential for road surface disturbance

during wet weather and to reduce sedimentation probability.

EXPLANATION:  The unrestricted use of many National Forest roads

during wet weather often results in rutting and churning of the

road surfaces.  Runoff from such disturbed road surfaces often

carries a high sediment load.  The damage/maintenance cycle for

roads that are frequently used during wet periods can create a

disturbed road surface and sediment source.

Roads that must be used during wet periods should have stable

surface and sufficient drainage to allow such use with a minimum

of resource impact.  Rocking, oiling, paving, and armoring are

measures that may be necessary to protect the road surface and

reduce erosion potential.  Roads not constructed for all weather

use should be closed during the wet season.  Where winter field

operations are planned, roads may need to be upgraded and

maintenance intensified to handle the traffic without creating

excessive erosion and damage to the road surfaces.

IMPLEMENTATION:  Road closures (SWCP 11.09) and traffic control

measures should be implemented on all roads when damage would

occur as a result of use during wet weather. Project-associated

implementation procedures can be enforced by District personnel.

Hauling activity can be controlled by the certified Sale

Administrator within active timber sales.  The decision for

closure is made when the responsible Line Office determines that

a particular resource or facility needs protection from use.

Detailed mitigative measures are developed by an

interdisciplinary approach as necessary.  Forest Service

supervisors are responsible for implementing In-Service projects

according to design standards.  For contracted projects,

compliane with plans, specifications, and operating plans is

assured by the Contracting Officer or Engineering Representative.

REFERENCES:  FSM 7731.4, SWCP 11.09, 13.06, and 14.04; Timber

Sale Contract provisions B5.12, B5.22, and C5.12; FSH 2409.15,

Timber Sale Administration Handbook; see references in "Best

Management Practice" Definition (05--2 and 3).

PRACTICE:  15.24 - Snow Removal Controls

OBJECTIVE:  To minimize the impact of snow melt on road surfaces

and embankments and to reduce the probability of sediment

production resulting from snow removal operations.

EXPLANATION:  This is a preventive measure used to protect

resources and indirectly to protect water quality.  Forest roads

are sometimes used throughout the winter for a variety of

reasons.  For such roads, the following measures are employed to

meet the objectives of this practice:

a.  The Purchaser is responsible for snow removal in a

manner which will protect roads and adjacent resources.

b.  Rocking or other special surfacing and/or drainage

measures may be necessary, before the operator is allowed

to use the roads.

c.  During snow removal operations, banks shall not be

undercut nor shall gravel or other selected surfacing

material be bladed off the roadway surface.  Ditches and

culverts shall be kept functional during and following

roadway use.  If the road surface is damaged, the

purchaser shall replace lost surface material with

similar quality material and repair structures damaged in

blading operations.

d.  Snow berms shall not be left on the road surface or

shall be placed to avoid channelization or concentration

of melt water on the road or erosive slopes.  Berms left

on the shoulder of the road shall be removed and/or

drainage holes opened at the end of winter operations and

before the spring breakup.  Drainage holes shall be

spaced as required to obtain satisfactory surface

drainage without discharge on erodable fills.  On

insloped roads, drainage holes shall also be provided on

the ditch side, but care taken to insure that culverts

and culvert inlets are not damaged.

IMPLEMENTATION:  Project location and detailed mitigative

measures are developed by the interdisciplinary team

during the NEPA process.  Contracted projects are

implemented by the Purchaser.  Compliance with criteria

in the contract specifications and operating plan is

assured by the contracting Officer or Engineering

Representative.

REFERENCES:  Timber Sale Contract provisions C5.46; Standard

Specifiation 203.09; see references in "Best Management Practice"

Definition (05--2 and 3).

PRACTICE:  15.25 - Obliteration of Temporary Roads

OBJECTIVE:  To reduce sediment generated from temporary roads by

obliterating them at the completion of their intended use.

EXPLANATION:  Temporary roads are constructed for a specific

short-term purpose, such as, ski area development roads, logging

spurs on a timber sale, and so forth.  In order to prevent

continued low level casual use, such roads are obliterated at the

completion of their intended use.  Due to short-term nature of

temporary roads, continued maintenance funds can not be used for

work on temporary roads.  Temporary roads that are allowed to

remain in use beyond their prescribed time are subject to

continued, uncorrected damage, and they can become chronic

sediment sources.

Effective obliteration is generally achieved through a

combination of the following measures:

a.  Road effectively drained and blocked.

b.  Temporary culverts and bridges removed and natural

drainage on figuration re-established.

c.  Road returned to resource production through

revegetation (grass, browse, or trees).

d.  Sideslopes reshaped and stabilized.

IMPLEMENTATION:  For timber sales, temporary road closure,

stabilization and removal of temporary structures are

accomplished by the Timber Purchaser.  Compliance with plans and

the Timber Sale Contract is assured by the certified Sale

Administrator.  Forest Service supervisors are responsible for

insuring that other temporary roads developed by the Forest

Service met design standards and management requirements.

Temporary road development on Forest Service lands that are

allowed through special use permits and/or easements are subject

to the same  obliteration requirements as temporary roads on

timber sales.  District Rangers or their representatives are

responsible for assuring the obliteration of such roads is

accomplished.

REFERENCES:  Timber Sale Contract provisions B6.62, B6.5, C6.6,

and C6.601; FSM 2522; SWCP 11.03, 11.08, 11.09, 13.04, 14.12 -

14, 14.19, and 15.03; NFMA; FSH 2409.15, Timber Sale

Administration Handbook; see references in "Best Management

Practice" Definition (05--2 and 3).

PRACTICE:  15.26 - Surface Erosion Control at Facility Sites

OBJECTIVE:  To minimize the amount of erosion and sedimentation

at developed sites.

EXPLANATION:  On lands developed for administrative sites, ski

areas, campgrounds, parking areas, or waste disposal sites much

ground is cleared of vegetation.  Erosion control mehtods need to

be implemented to stabilize the soil and to reduce the amount of

stream sedimentation.  Some examples of erosion control methods

that could be applied:  grass seen, jute mesh, tackifiers,

hydromulch, paving or rocking of roads, water bars, cross drains,

or retaining walls.

To control erosion and sedimentation, the natural drainage

pattern of the area should not be changed.  Sediment basins and

sediment filters should be established to filter surface runoff.

Diversion ditches and berms should be built to divert surface

runoff aroung bare areas.  Construction activiteis should be

scheduled to avoid periods ofheavy precipitation or runoff.

IMPLEMENTATION:  Mitigative measures are developed by the

interdisciplinary team during the NEPA process and incorporated

in the project by the design engineer.  Forest Service

supervisors are responsible for implementing In-Service  projects

to design standards and management requirements.  For contracted

projects, compliance with plans, specifications, and operating

plans is assured by the Contracting Officer or Engineering

Representative.

REFERENCES:  SWCP 11.08, and 11.12; FSM 2522; see references in

"Best Management Practice" Definition (05--2 and 3).

PRACTICE:  15.27 - Trail Maintenance and Rehabilitation

OBJECTIVE:  to minimize soil erosion and water quality problems

resulting from trail erosion.

EXPLANATION:  Trails often have erosion problems due to poor

location, improper maintenance, and the amount or type of use.

This deterioration can often be minimized by proper maintenance,

restriction of certain types of use, and/or relocation.

Mainline and heavy use trails should have a functional drainage

systems (waterbars, culverts at small stream crossings, corduroy,

puncheon or boardwalks in boggy areas).  Additional measures

(lateral ditching, trail relocation, reconstruction, and so

forth) may be required in heavy sue or problem areas.

IMPLEMENTATION:  Each District will develop a trail maintenance

plan which determines level, timing and frequency of maintenance.

The need for closures will be identified through Forest

Transportation Planning.  Closure is done by authority of the

Forest Supervisor (SWCP 11.09).

REFERENCES:  SWCP 11.03, 11.09, 15.01, 15.02, and 15.03; FSH

7709.56b, Drainage Structures Handbook; see references in "Best

Management Practice" Definition (05--2 and 3).

16 - MINERALS.  Minerals (including oil, gas and geothermal

resources) exploration and development activities on National

Forest System lands fall into generally one of the following

categories:

A.  Locatable.  The General Mining Law of 1872, as amended,

governs the prospecting for and the appropriation of metallic and

most non-metallic minerals with a distinct and special value on

National Forest System lands that were reserved from the public

domain.  This applies to most hard rock and placer mineral

deposits.

B.  Leasable.  The Mineral Leasing Act of February 25, 1920,

as amended and supplemented, subject certain mineral and energy

resources to disposal through leasing actions.  These energy and

mineral resources include, but are not limited to, coal, oil,

gas, geothermal, oil shale, potassium, sodium, and phosphate.

The Mineral Leasing Act for Acquired Lands of August 7, 1947,

makes all minerals on acquired (purchased) National Forest System

lands, unless otherwise reserved or held as outstanding rights,

subject to the provisions of the 1920 Minerals Leasing Act.

C.  Common Variety Minerals.  The Materials Act of July 31,

1947, provides for the disposal and use of common variety mineral

materials such as sand, stone, gravel, pumice, cinders and clay

located on National Forest System lands.  Disposal can be by sale

or free use permit to private entities or Federal, State, and

local units of government, when consistent with good public land

management and in the public interest.

PRACTICE:  16.01 - Administration of the General Mining Law (Act

of May 10, 1872) for Mineral Exploration and Extraction on

National Forest System Lands

OBJECTIVE:  To protect water quality from degradation by physical

and chemical constituents which may result from mining and

associated activities.

EXPLANATION:  Six instruments are involved in analyzing and

approving locatable mining activities which could affect water

quality on National Forest System lands.  Instruments d., e., and

f. may not be necessary in every case.  The instruments are

listed in sequential order if all are needed:

a.  Notice of Intent to Operate.

b.  Plan of Operations.

c.  Environmental Analysis.

d.  Special Use Permit(s).

e.  Road Use Permits.

f.  State and/or other Federal Agency

permit(s)/Certification.

The Notice of Intent to Operate is required from those who

propose to conduct mining related activities which might cause

disturbance of surface resources on National Forest System lands.

The notice is submitted to the appropriate responsible official,

normally the District Ranger.  The proposed operations described

in the notice must be evaluated by the District Ranger.  The

proposed operations described in the notice must be evaluated by

the District Ranger, who will inform the operator within 15 days

after the notice is received that either his operation is exempt

from the requirement for an operating plan or one is required.

If the District Ranger determines that significant disturbance of

surface resources will likely result from the proposed

operations, the District Ranger will inform the operator to

prepare a Plan of Operations.

A written Plan of Operations is required from all operators who

will likely cause a significant disturbance of surface resources.

Prior to approval of the Plan of Operations, the operator may be

required to furnish a guarantee in the form of an approved surety

bond or other security to perform reclamation work.  If hazardous

material are to be use or generated, documentation that

compliance with applicable State or other Federal agency

permits/certification have been met is required.  The operating

plan shall be submitted to the District Ranger who will review

the plan and prepare an environmental analysis within 30 days

after the receipt of the plan.  The environmental analysis either

results in the plan of operations not being required, being

approved, needing changes or additions, needing more review time

(Environmental Assessment) but not exceeding an additional 60

days, or being deferred until an Environmental Impact Statement

(EIS) has been prepared and filed by the Forest Service.

Plans of Operation and/or Special Use Permits may be required and

issued for construction or reconstruction of roads for access to

mining claims or across National Forest system lands on which the

operator has no mining claim.  However, when an operating plan is

required, and does not include access, the plan must be approved

prior to issuance of a special use permit regulating access.

Special use permits may also be required and issued for water

diversions, water transmission facilities, and electric

transmission lines needed for mining activities.

Road Use Permits may be required and issued for commercial use of

National Forest System roads.

State and/or other Federal Agency Permits/Certification may be

required and issued for air quality, water quality, reclamation,

disposal and treatment of solid wastes, and so forth.  When

required, the Forest Service will advise the operator to obtain

the appropriate permits or certification.  If the proposed

operation will involve the use or generation of hazardous

substances, the operator will be required to incorporate the

permitting requirements of the appropriate regulatory agencies

before approval of the Plan of Operations.

IMPLEMENTATION:  A Notice of Intent or Plan of Operation is

required to be submitted by the operator prior to operations.

Preventive measures should be set forth within the notice or plan

which will control sediment from land disturbing activities,

control chemical seepages from mines and tailings, and prevent

and correct hazardous substance spills.  Locations for spoil and

tailing disposal are also specified.  The plan is reviewed by an

interdisciplinary team.  Any need changes are conveyed to the

operator and negotiated prior to final plan approval.

Through the use of the Notice of Intent, Plan of Operations, and

provisions in any Special Use Permit issued, the responsible

Forest Officer, usually the District Ranger, checks for

compliance with prescribed measures.  Legal remedies are

available if mutual cooperation fails.  A court may grant

injunctive or mandatory relief, and award damages to the extent

of property damaged.  There may be other remedies for violation

of Federal, State and local standards for air and water quality

and for the disposal of solid wastes.

REFERENCES:  36 CFR 228, 36 CFR 251, and 30 USC 612; NEPA; FSM

1950, 2725, 2726, 2730.3, 2734.3, 7720, 2810, 2817, and 2850; FSH

2809.11, Land Managers Handbook on Minerals Management; FSH

2809.12, Minerals Planning Handbook; SWCP 11.01, 11.02, 11.04,

11.05, 11.06, 11.07, 11.08, 11.11, 11.13, 12.06, 12.07, 12.08,

13.04, 15.01, 15.02, and 15.03.

PRACTICE:  16.02 - Administration of Bureau of Land management

Issued permits, Licenses, or Leases for Mineral Exploration and

Extraction on National Forest System Lands

OBJECTIVE:  To protect soil and water resource values during

mineral exploration, extraction, processing and reclamation

activities that are conducted on National Forest System lands

under the terms of Bureau of Land Management prospecting permits,

coal exploration licenses, and mineral leases.

EXPLANATION:  Through the NEPA process, the Forest Service (FS)

and Bureau of Land Management (BLM) make a determination as to

whether or not a permit, license, or lease should be issued by

the Bureau of Land Management.  The Forest Service and Bureau of

Land Management develop the permit, license, or lease

stipulations needed to protect water quality and other resource

values.

IMPLEMENTATION:  Detailed mitigative measures are developed by an

interdisciplinary team during the environmental analysis and are

written into the special stipulations section of the permit,

license, or lease.  Conditions of approval are also developed by

the interdisciplinary team to be included in the operating plan.

By interdepartmental agreement, all applications to lease lands

under Forest Service jurisdiction are referred to the Forest

Service for review, recommendations, and development of special

stipulations to protect the surface resources.  Technical

administration of Bureau of Land Management permits, licenses,

and leases is the responsibility of the Bureau of Land

Management.  Therefore, compliance inspections are the

responsibility of the Bureau of Land Management unless the Forest

Service is authorized to conduct compliance inspections through

an interagency agreement or MOU.  The Forest Service may inspect

and refer situations of non-compliance with operating plans to

the Bureau of Land Management for action.

REFERENCES:   FSH 2809.11, Land Managers Handbook on Minerals

Management; FSH 2809.12, Minerals Planning Handbook; FSM 2725,

2726, 2734, and 2820; Mineral Leasing Act (41 Stat. 437, as

amended; 30 USC 181); Federal Coal Leasing Amendments Act (90

Stat. 1083; 30 USC 201 (b) and 207; 16 USC 1276); Act of March 4,

1917 (30 Stat. 1150, as supplemented; 16 USC 520); Section 402 of

Reorganization Plan No. 3, of July 16, 1946 (60 Stat. 1097, 1099;

5 USC Appendix); Act of August 7, 1947 (61 Stat. 913; 30 USC 351,

352, 354, 359) as amended by PL 167 and the Geothermal Steam Act

(PL 91-581); SWCP 11.01, 11.02, 11.04, 11.05, 11.06, 11.07,

11.08, 11.11, 11.13, 12.06, 12.07, 12.08, 13.04, 15.01, 15.02,

and 15.03.

PRACTICE:  16.03 - Administration of Common Variety Mineral

Operations

OBJECTIVE:  To assure protection of water quality and other

resource values when common variety mineral materials are used

for both In-Service and Out-Service.

EXPLANATION:  Common variety mineral materials such as sand,

stone, gravel, pumice, cinders and clay may be disposed of and

developed when their use is consistent with good public land

management and in the public interest.  Use authorizations will

require reasonable erosion control, and rehabilitation and

revegetation of the surface.  Removal may be approved if adequate

measures can be accomplished to prevent erosion or stream

pollution and satisfactory arrangements can be made for

rehabilitation and restoration as outlined here and in SWCP

15.13, 15.1, and 15.17.

IMPLEMENTATION:  A project plan or Mineral Material permit

identifies the location and conditions of mineral material

removal and disposal.  Both will be preceded by an environmental

analysis.  Project location, the scope of the proposal, and

detailed mitigative measures are developed using an

interdisciplinary approach. The project or permit is approved by

the District Ranger or Forest Supervisor.  Compliance with the

project design standards, the terms and conditions of the permit,

and applicable Federal and State regulations is assured by the

District Ranger or Forest Service representative.

REFERENCES:  36 CFR 228, Subpart C;  FSM 2725, 2726, 2734, 2814,

2817, and 2850; FSH 2809.13, Minerals Program Handbook; FSH

7709.11, Transportation Engineering Handbook; Act of July 31,

1947 (61 Stat. 681), as amended by the Act of August 31, 1950 (64

Stat. 571), and the Act of July 23, 1955 (60 Stat. 367; 30 USC

601-603), and pursuant to the Acto of June 11, 1960 (74 Stat.

205), and the Acto of September 25, 1962 (76 Stat. 587); SWCP

11.04, 11.05, 11.07, 11.08, 15.01, 15.02, 15.03, 15.13, 15.14,

and 15.17.

PRACTICE:  16.04 - Permits and Administration of Geophysical

Operations

OBJECTIVE:  To protect the quality of surface and ground water

from degradation resulting from geophysical activities on

National Forest System lands.

EXPLANATION:  Geophysical activities will be managed in a manner

that is both timely and offers protection to other multiple use

values and management objectives.  Many activities have no

effects.  However, if effects are identified, standard seismic

hole plugging procedures will be followed to prevent

contamination of ground water resources, and shot hole placement

will be examined for potential impacts to other resource values

(SWCP 11.10).  New road construction, if allowed, will be

located, constructed, and maintained to protect the soil and

water resources (SWCP 15.01, 15.02, and 15.03).

IMPLEMENTATION:  During the environmental analysis, an

interdisciplinary team will be assembled to prepare the

appropriate NEPA document that evaluates potential impacts,

including cumulative, and any needed mitigation measures for the

geophysical prospecting permit.  The use of water resources for

the prospecting activities may require non-Forest Service

authorizations or permits.

REFERENCES:  Organic Act of 1897 (30 Stat. 34, as amended, 16 USC

472, 475-478, 480-482, 551); Multiple Use-Sustained-Yield Act of

1960 (74 Stat. 215, 16 USC 528-531); RPA, as amended (88 Stat.

476; 16 USC 1600-16140;  FSM 2860; FSH 2809.13, Minerals program

Handbook; In R-1: A Procedural Guide for Oil and Gas

Administration , USDA-Forest Service, R-1, Custer NF; SWCP 11.01,

11.07, 11.08, 11.10, 13.04, 15.01, 15.02, and 15.03.

PRACTICE:  16.05 - Mineral Activity Coordination Analysis

OBJECTIVE:  To protect the soil and water resource from

degradation during development of minerals resources.

EXPLANATION:  One essential function of a minerals management

program is to forecast what, where, and when mineral activity

will occur.  While the Forest Service has limited ability to

control the pace and location of mineral development, the Forest

Service has the authority to influence the process considerably.

The Activity Coordination Analysis approach for mineral

exploration, development and production is a prudent approach

that assists in minimizing adverse impacts to the soil, water,

and other resources.

IMPLEMENTATION:  The specific land area requiring an Activity

Coordination Analysis will be identified.  This will necessitate

the delineation of the study area based on typical oil and gas

spacing intervals, pipeline, primary and alternative recovery

installation locations, directional drilling opportunities,

sensitive areas for water and other resources, and other

environmental concerns.  An interdisciplinary team will collect

and analyze data, prepare an environmental analysis, and develop

a management design that will best mitigate impacts to soil and

water resources as well as other resource values.  Technical

staffs familiar with the components of construction, development,

and production of an oil and gas field will be utilized.

REFERENCES:  In R-1: A Procedural Guide for Oil and Gas

Administration, USDA-Forest Service, R-1, Custer NF; FSH 2809.13,

Minerals Program Handbook: NEPA; SWCP 11.01, 11.02, 11.04, 11.05,

11.07, 11.10, 11.11, 13.04, 15.01, 15.02, and 15.03.

PRACTICE:  16.06 - Reclamation of Oil and Gas Well Sites

OBJECTIVE:  To protect soil and water resources through the

development of reclamation plans prior to the approval of an

Application for Permit to Drill.

EXPLANATION:  Reclamation of oil and gas well sites is necessary

to protect the soil and water resources, both on- and off-site.

Revegetation with plants or grasses, forbs, shrubs and trees that

provide the best protective ground cover should be utilized to

prevent erosion and stream sedimentation.  Revegetation species

should, however, be selected that provide for an immediate ground

cover and allow for the reestablishment of the desirable species

over the long-term.  For both producing and non-producing well

sites, reclamation will normally commence within the first

growing season following completion of the drilling activities.

In the case of a producing well site, only those portions of the

site not required for production need be reclaimed.  Any

necessary drainage or erosion control structures to prevent

erosion and degradation of water quality should be installed

during rehabilitation of the site.  Temporary roads used to

access oil and gas well sites are also reclaimed and obliterated

upon completion of their intended use.

IMPLEMENTATION:  Detailed erosion control and mitigative measures

are developed by an interdisciplinary team during the

environmental analysis and are incorporated into the Plan of

Operations.  Compliance with permits and the operating plan is

assured by the District Ranger or Forest Service representative.

Non-compliance with approved operating plans are referred to the

Bureau of Land Management for action.

REFERENCES:  43 CFR 3164; FSM 2840; In R-1:  A Procedural Guide

for Oil and Gas Administration, USDA Forest Service, R-1, Custer

NF; SWCP 13.01, 13.04, 15.05, 15.06, and 15.25; Surface Operating

Standards for Oil and Gas Exploration and Development, 1978,

Bureau of Land Management, U.S. Geologic Survey, and USDA, Forest

Service; Surface Environment and Mining (SEAM) REclamation User

Guides.

PRACTICRE:  16.07 - Reserve Pit Location, Design, Operation, and

Reclamation

OBJECTIVE:  To protect the quality of surface and ground water

from degradation by physical and chemical contaminants

originating from the construction and operation of reserve pits

at drilling sites.

EXPLANATION:  Under current technology, a reserve pit is normally

utilized to collect, store, and recycle drilling fluids in oil

and gas well drilling operations.  Reserve pits must be located,

operated, and relcaimed so as to prevent adverse effects on the

surface and ground water resources.  REserve pits should be

designed and construced utilizing the following standards:

a.  Reserve pits should be located in the cut portion of

the drill pad and not at the edge of steep slopes.

b.  Special measures may be required to insure the

containment of drilling fluids where the reserve pit must

be placed in a sensitive location or in porous material.

These measures may consist of lining the reserve pit with

an impermeable material substance such as plastic or

bentonite or the use of a closed mud system.

c.  The pit should be constructed so as to prevent

sloughing and to maintain the integrity of the liner.

Fill material may be used and laid down to provide better

consolidation than from natural, in-place materials.

d.  The use of non-toxic drilling fluids should be

encouraged whenever possible.

e.  Dumping trash in the reserve pit will be prohibited.

f.  Reserve pit reclamation methods will be utilized that

provide positive protection to both ground and surface

water.

IMPLEMENTATION:  An interdisciplinary approach during the NEPA

process will develop and specify the project location, design

features, operating requirements, mitigative measures, and

reclamation needed.  Compliance with permits and the operating

plan is assured by the District Ranger or Forest Service

representative.

REFERENCES:  In R-1:  A Procedural Guide for Oil and Gas

Administration, USDA-Forest Service, R-1, Custer NF; Surface

Operating Standards for Oil and Gas Exploration and Development,

1978, Bureau of Land Management, U.S. Geologic Survey, and

USDA-Forest Service.

PRACTICE:  16.08 - Oil and Gas Well Blowout Contingency Plan

OBJECTIVE:  To protect the soil, water and other resources from

oil and gas well blowout impacts.

EXPLANATION:  Although infrequent, uncontrolled emissions from

oil and gas wells can occur.  Fluids emitted can be oil, fresh

water, salt water, or a combination thereof.  A Well Blowout

Contingency Plan is necessary in order to minimize impacts to

soil and water resources.

IMPLEMENTATION:  Individual operators are required to provide

Blowout Contingency Plans for emergency situations.  The plans

must meet the following items:

a.  List of agencies, institutions and persons to notify

(SWCP 11.07).

b.  A "plan of attack" to handle the various levels of

emergencies.

c.  Soil and water protection measures to be instituted

while the blowout is occurring and immediately following

the control of the well blowout.

d.  Established blowout or toxic spill damage assessment

time gates and goals (SWCP 11.07).

e.  The frame work necessary to establish an

interdisciplinary team to develop and implement a

reclamation plan.

REFERENCES:  43 CFR 3160; In R-1:  A Procedural Guide for Oil and

Gas Administration, USDA-Forest Service, R-1, Custer NF; FSM 7442

and 7443; 40 CFR 112; Oil and Hazardous Substances Pollution

Contingency Plan for EPA Regions 8 and 10, 7/26/85; SWCP 11.07

and 11.11; Surface Environment and Mining (SEAM) Reclamation User

Guides.

PRACTICE:  16.09 - Abandoned Mine Land Reclamation

OBJECTIVE:  To reduce erosion and water quality degradation by

sediment and toxic substances from abandoned mined lands through

reclamation of these lands.

EXPLANATION:  Abandoned mined lands are frequently erosive, bare

of vegetation, or are exuding toxic substances and/or sediment

into nearby streams.  Some sites may pose a threat to public

health or safety.  Reclamation plans for reducing impacts to soil

and water resources are needed for each abandoned mine.  Specific

practices may vary from site tot site, ranging from simple

revegetation or reshaping with earth-moving equipment, to

restoration to pre-disturbance conditions.  Al ground-disturbing

activities should comply with SWCP 11.03, 13.01, 13.04, 15.05,

and 15.06.

It is important that the site be revegetated with plant species

that accomplish the purposes of reclamation.  Species may be

native or introduced and may be both live plants or seed.

Fertility of soil/spoil materials and climate will affect species

selection and survival, and soil amendment recommendations.

IMPLEMENTATION:  This practice is typically implemented through

the development of an inventory of all abandoned mined lands, and

for inclusion of those lands needing reclamation within SWCP

11.03.  If a soil and water resource problem area is observed and

documented, an interdisciplinary team will assess that abandoned

mine site, develop the necessary actions to correct the problem,

and integrate then into the Forest Planning process for funding

and execution.  The NEPA process will be followed in the planning

and implementation of reclamation measures.  The Forest Service

should work toward inclusion of the more important abandoned

mined lands in State inventories and reclamation plans, since

both the State and the Office of Surface Mining (OSM) can provide

funding for State projects.

REFERENCES:  FSM 2522, 6740, 7442, 7443, and 7460; FSH 2509.15,

Watershed Improvement Handbook; NFMA; SWCP 11.03, 13.01, 13.04,

15.05, and 15.06; Abandoned Mine Lands Reclamation Control

Handbook, Office of Surface Mining; Surface Environment and

Mining (SEAM) Reclamation User Guides.

17 - RANGE.  Range management involves range and resource

analysis, allotment management planning, and a grazing permit

system.  It includes controlling overall livestock numbers and

season of use, improvements, providing for wildlife needs, and

restoration of deteriorated range lands.

The historical use of national Forest System lands in the

Northern and Intermountain Regions for grazing generally predates

the actual establishment of individual Forests.  During this

early period, grazing use was typically uncontrolled with

generally an excessive number of animals using these lands and

producing numerous soil and water resource problems.  The current

situation is much improved over that which existed historically.

Most allotments maintain the productive status of the land and

protect underlying soils.  There are, however, some allotments

where the range remains in a deteriorated condition.  These

SWCP's are designed to restore deteriorated range an maintain all

range in a productive state.

PRACTICE:  17.01 - Range Analysis, Allotment Management Plan,

Grazing Permit System and Permittee Operating Plan

OBJECTIVE:  To maintain and protect soil and water resources

through sustained forage production and managed multiple use of

range forage.

EXPLANATION:  An analysis of potential and/or existing range is

conducted by an interdisciplinary team to evaluate productive

capabilities, inherent hazards, resource values, and uses.  Based

on this analysis, the Forest Service, in cooperation with the

permittee and other users, prepares a written allotment

management plan and issues a permit to authorize livestock

grazing as per stipulations in the plan.  These documents include

measures to protect other resource values, such as water quality

and riparian areas, and to coordinate livestock grazing with

other resource uses.  Specific methods for controlling when,

where, amount of utilization, and numbers of livestock to be

grazed are covered in the plan.  Rangeland improvements and an

implementation schedule are also included.  Permittees are

required to contribute a portion of the cost of new improvements

and to maintain the existing improvements on an allotment.

A permittee operating plan is prepared, reviewed and revised

annually to reflect direction in the allotment management plan,

to account for current allotment conditions and trends, and to

adjust for unexpected resource problems (such as drought).  The

amount of livestock use is determined primarily through

measurement of vegetative utilization.  Allowable use is

determined by research, vegetative trends, and experience.

IMPLEMENTATION:  The District Ranger is responsible for analysis

of range allotments, completion of the environmental analysis,

preparation of allotment management plans, and processing grazing

applications.  The Forest Supervisor approves allotment

management plans and issues grazing permits with required

stipulations and conditions.  Most permits are issued for

ten-year terms.  Allotment management plans are revised as

needed.  Permittee operating plans are prepared or revised

annually, to adjust for current allotment conditions and trends,

and to incorporate seasonal instructions.  The permittee carries

out the plans under the direction and supervision of the District

Ranger or Forest Service representative.  Corrective action is

taken if a permittee does not comply with grazing permit

conditions designed to protect the soil and water resources.

REFERENCES:  FSM 2203, 2204, and 2323.2 - 24; FSH 2209.12, Range

Management Information System Handbook, FSH 2209.13, Grazing

Permit Administration Handbook, and FSH 2209.14, Service-wide

Range Analysis and Management Handbook; NEPA; SWCP 11.01, 11.02,

11.03, and 11.05.

PTRACTICE:  17.02 - Controlling Livestock Numbers and Season of

Use

OBJECTIVE:  To maintain and protect soil and water resoruces

through management of livestock numbers and season of use.

EXPLANATION:  In addition to the proper stocking rate and season

of use specified in the grazing permit, annual field checks are

made to identify needed adjustments in distribution, season of

use, and livestock numbers.  Analysis includes:

a.  Range readiness evaluations to assure that the soil

is not too wet and that sufficient forage growth has

occurred.

b.  Livestock counts to assure that only the permitted

livestock enter the allotment.

c.  Forage and browse utilization measurements to provide

data for improved livestock distribution and stocking.

d.  Periodic assessment of rangelands to verify soil and

vegetative condition and trend.

Standard measurement techniques for allowable utilization have

been established for key vegetative types.  Specific standards

and guidelines have been established prescribing proper

utilization levels by vegetative type.  Livestock numbers and

season of use are adjusted to reflect the results of these field

checks.

IMPLEMENTATION:  Allotments are administered by the District

ranger or Forest Service representative.  Permit provisions are

carried out by the grazing permittee, as directed in the permit

and annual operating plan.  Appeals are made to the Forest

Supervisor.  Field checks and measurements are made periodically

by the Forest Service.  Numbers, seasons, and use patterns may be

changed annually to reflect current allotment conditions and

trends and, if necessary, the permit may be modified, cancelled,

or suspended in whole or in part.

REFERENCES:  FSM 2210, 2230,  2240, 2250, and 2323.25; FSH

2209.14, Service-wide Range Analysis and Management Handbook; FSH

2209.21, Range Analysis and Management Handbook; SWCP 11.02.

PRACTICES:  17.03 - Controlling Livestock Distribution

OBJECTIVE:  To maintain and protect soil and water resources

including riparian areas through controlling livestock

distribution.

EXPLANATION:  Livestock use within allotments is typically not

uniform due to variations in topography, water availability,

vegetation type, and forage condition.  Several techniques are

used to achieve proper livestock distribution and reduce the

impact on areas which are sensitive or naturally overused.  These

techniques include:

a.  Construction of fences and implementation of seasonal

or pasture systems of management.

b.  Placing of water developments in areas that receive

little use and closure of water developments when proper

use has been achieved.

c.  Riding and herding to shift livestock locations.

d.  Placing salt or supplements away from water in forage

areas with light grazing use to attract livestock.

e. Installation of range improvements such as

construction of shade structures, fertilization,

prescribed burning, or seeding.

f.  Moving livestock when prescribed utilization levels

are reached.

Open herding, limiting trailing, and use of new bed grounds

nightly are additional techniques used for goats and sheep.

Developing sufficient watering places is one way to limit the

amount of trailing.  Livestock distribution needs are determined

through evaluations of range conditions and trends, including

utilization studies.

IMPLEMENTATION:  Livestock distribution practices are carried out

by the permittee under supervisor of the District Ranger or

Forest Service representative.  Direction is incorporated into

the allotment management plan and the annual operating plan.  The

annual operating plan becomes an integral part of the grazing

permit and  provides current Forest Service instructions.  These

instructions reflect current allotment conditions and vegetative

trends.

REFERNECES:  FSM 2210, 2230, 2240, 2250, and 2323.25; FSH

2209.14, FSH Service-wide Range Analysis Handbook; and FSH

2209.21, Range Analysis and Management Handbook; SWCP 11.02.

PRACTICE:  17.04 - Rangeland Improvements

OBJECTIVE:  To maintain and protect soil and water resources

through the use of rangeland improvements.

EXPLANATION:  Rangeland improvements are used to improve

management and restore or improve forage quality, quantity, or

availability.  These may consist of providing rangeland rest

and/or deferment through rotation grazing, fencing, or lighter

grazing use by changing the grazing season, kind, class, or

permitted number of livestock.  Other measures may include stream

channel stabilization efforts such as riprapping, gully plugging,

planting, or mechanical treatments such as pitting, chiseling, or

furrowing.  Reseeding and/or fertilization may be done

individually or in conjunction with any of these measures.  Water

developments are often included in rangeland improvement

projects.  Improvement efforts are directed at increasing the

ability of the range to produce forage and protect and provide

for other resources at a specified ecological serial stage.

Practices used for improvement of watershed conditions, which may

include the exclusion of livestock, are described in SWCP's 11.03

(Watershed Improvement Planning and Implementation) and 11.09

(Management by Closure to Use).  All range water improvements

constructed should protect the water quality of both surface and

ground water sources.

IMPLEMENTATION:  The permittee is a cooperator in rangeland

improvements and may complete the work under Forest Service

direction.  Implementation may also be done by Forest crews.

Range improvement needs are recognized in the Range Allotment

Planning Process and are scheduled for implementation in the

allotment management plan.  An interdisciplinary team provides

consultation and help in the development of improvement programs.

REFERENCES:  FSM 2210, 2240, 2250, and 2323.26; FSH 2209.22,

Structural Range Improvement Handbook; FSH 2209.23, Nonstructural

Range Improvement Handbook; SWCP 11.02, 11.03, and 11.09.

18 - FIRE SUPPRESSION AND FUELS MANAGEMENT.  Emergency fire

suppression activities on National Forest System lands are

conducted to reduce erosion and the loss of soil productivity,

degradation of water quality, and threats to life and property

both on-site and off-site.  Suppression activities include

fireline and access road construction, firing operations, and

fire retardant drops.  Water quality and soil erosion and

productivity objectives are weighed with the need for rapid

suppression during the development of suppression strategies.

Since some watershed damage will likely result from suppression

activities, an objective of the fire suppression program is to

rehabilitate suppression-related damage.

Fuels management activities are intended to reduce the size,

cost, and damage of wildfire.  Fuels management is a form of

Vegetation Manipulation (ch. 13).  Vegetation is manipulated by

changing fuel type, creating fuel breaks, or by reducing or

altering fuels over extensive areas.  Fuels management is also

concerned with the manipulation of dead fuels such as cull logs

and stash.  These materials may be utilized, removed, or burned

to reduce fuel loading.

PRACTICE:  18.01 - Fire and Fuel Management Activities

OBJECTIVE:  These administrative, corrective and preventive

measures include: (1) fuelbreak construction; (2) type

conversions; (3) greenbelt establishment to separate urban areas

from wildlands; (4) fuel reduction blocks; (5) access roads for

rapid ingress and egress; (6) fire suppression activities; (7)

fuel utilization and modification programs; and (8) public

information and education programs.

IMPLEMENTATION:  Fuel management is implemented through normal

program planning and budgeting, Forest Planning, and NEPA

processes.  Other resource areas such as Timber, Range, and

Wildlife may initiate projects that also benefit Fire Management

through fuel modification.  Fuel management projects are

evaluated by an interdisciplinary team.  The management

objectives and requirements, and multiple resource protection

prescriptions are documented through the NEPA process.

Application of controls and prescriptions are the responsibility

of the project officer.  Suppression activities are conducted in

response to fires as they occur.

REFERENCES:  FSM 1950, 5102, 5103, 5121, 5150.2, 5150.3, and

5151; NEPA; NFMA; SWCP 11.02 and 11.06.

PRACTICE:  18.02 - Formulation of Fire Prescriptions

OBJECTIVE:  to provide for soil and water resource protection

while achieving the management objective through the use of

prescribed fire.

EXPLANATION:  prescription elements will include such factors as

fire weather, slope, aspect, soil moisture, and fuel moisture

which influence the fire intensity.  These elements have a direct

effect on whether or not a litter layer remains after burning and

whether or not a water repellent layer is formed.  The amount of

remaining latter and induced water repellency can significantly

affect erosion rats, water quality, and runoff volumes.

IMPLEMENTATION:  The prescription elements are defined by the

interdisciplinary team durng the environmental analysis.  Field

investigations are conducted to identify site specific conditions

which may affect the prescription.  Both the optimum and

tolerable limits for soil and water resource needs should be

established.

REFERENCES:  FSM 5140, 5150.3, 5151, AND 5153.

PRACTICE:  18.03 - Protection of Soil and Water from Prescribed

Burning Effects

OBJECTIVES:  To maintain soil productivity, minimize erosion, and

prevent ash, sediments, nutrients, and debris from entering

surface water.

EXPLANATION:  Some of the techniques used to prevent soil erosion

and water quality degradation are: (1) construct water bars in

fire lines; (2) reduce fuel loadings in drainage channels; (3)

maintain the integrity of the Riparian Area; (4) avoid intense

fires, which may promote water repellency, nutrient leaching, and

erosion; (5) retain or plan for sufficient ground cover to

prevent erosion of the burned sites; and (6) removal of all

debris added to stream channels as a result of prescribed

burning, unless debris is prescribed to improve fisheries

habitat.

IMPLEMENTATION:  Forest Service and/or other crews are used to

prepare the units for burning.  This includes water barring

firelines and reducing fuel concentrations. The interdisciplinary

team identifies Riparian Areas and soils with water repellent

tendencies as part of the environmental analysis.

REFERENCES:  FSM 5140, 5142, 5150.3, 5151, and 5153; SWCP 11.02,

11.05, 11.06, and 13.04.

PRACTICE:  18.04 - Minimizing Watershed Impacts from Fire

Suppression Efforts

OBJECTIVES:  To avoid watershed impacts in excess of that which

would be caused by the fire itself.

EXPLANATION:  Heavy equipment operation on fragile soils,

sensitive areas, and steep slopes should be avoided when

possible.  Major project fires utilize a Resource Advisor to

advise the Incident Commander on resource values during the

suppression effort.  National fire management policies were

changed in 1978, to provide in part that an Escaped Fire

Situation Analysis shall be prepared for all fires which escape

initial suppression action.  The analysis will be prepared by a

Line Officer with Incident Management Team input.  Watershed

considerations must be part of the analysis.

IMPLEMENTATION:  A Resource Advisor is assigned by the Forest

Supervisor and works for the Incident Management Team.  Technical

resource staffs are normally available to identify fragile soils,

sensitive areas, and unstable areas and would be assigned to the

fire as technical experts.

REFERENCES:  FSH 5109.19, Fire Management Analysis and Planning

Handbook; FSM 5130.3 and 5132; see references in "Best Management

Practice" Definition (05--2 and 3).

PRACTICE:  18.05 - Stabilization of Fire Suppression Related

Watershed Damage

OBJECTIVE:  To stabilize all areas that have had their erosion

potential significantly increased, or their drainage pattern

altered by suppression related activities.

EXPLANATION:  Treatments for fire-suppression damages include,

but are not limited to: (1) installing water bars and other

drainage diversions in fire roads, firelines, and other cleared

areas; (2) seeding, planting and fertilizing to provide

vegetative cover; (3) spreading slash or mulch to protect bare

soil; (4) repairing damaged road drainage facilities; and (5)

clearing stream channels of debris that is deposited by

suppression activities.

IMPLEMENTATION:  This work is done by the fire fighting forces

either as a part of the suppression effort or before crews and

equipment are taken off the fire lines.  The Incident Commander

is responsible under the directin of the local Line Officer for

repair of suppression related resource damage.

REFERENCES:  FSH 209.13, Burned-Area Emergency Rehabilitation

Handbook; FSM 2523 and 5130.2; SWCP 11.13, 13.04,   15.03, 15.06,

and 15.11; see references in "Best Management Practice"

Definition (05--2 and 3).

PRACTICE:  18.06 - Emergency Rehabilitation of Watersheds

Following Wildfires

OBJECTIVES:  To minimize the loss of soil and on-site

productivity, the deterioration of water quality, and threats to

life and property, both on-site and off-site.

EXPLANATION:  Emergency rehabilitation is a corrective measure

that involves a variety of treatments.  Treatments may include:

(1) seeding grasses or other vegetation to provide a protective

cover as soon as possible; (2) fertilizing; (3) fencing to

protect new vegetation from livestock; (4) clearing debris from

stream channels; and (5) constructing trash racks, channel

stabilization structures and debris retention structures.

Treatments are selected on the basis of on-site values,

downstream values, probability of successful implementation,

social and environmental considerations, and cost as compared to

benefits.

IMPLEMENTATION:  Burned-area surveys shall be performed promptly

to determine if watershed emergency rehabilitation treatment is

needed.  Surveys of all fires that exceed 300 acres shall be

conducted by an interdisciplinary team.  The responsible

official, typically the District Ranger, may request a

burned-area survey by an interdisciplinary team for smaller fires

if significant resource damage has occurred.  Team members

normally include a hydrologist, a soil scientist, and other

technical resource staffs, as needed.  They are typically

designated prior to the fire season to become reacquainted with

Forest Service policy and direction.  The survey and proposed

rehabilitation treatment measures will be delivered to the

Regional Office and/or the Washington Office for approval within

three days of control of the fire.  If the rehabilitation project

is funded, a project supervisor and restoration team will begin

work with the objective of project completion before damaging

storms occur.  Rehabilitation projects are evaluated following

major storms and runoff events and at least annually until the

watershed is stabilized.  The evaluation determines the

effectiveness of the rehabilitation measures and indicates if

follow-up actions are needed.

In situations where National Forest System and other intermingled

lands are involved in similar emergency rehabilitation efforts

from large wildfires, interagency rehabilitating teams, if

applicable, may be requested to conduct burned area surveys.

Prior coordination and agreements are necessary to insure each

land owner's needs and objectives are met.

REFERENCES:  FSM 2523 and 2323.43b; FSH 2509.13, Burned-Area

Emergency Rehabilitation Handbook; SWCP 11.02, 11.03, and 11.06;

In Montana: Interagency Emergency Watershed Rehabilitation

Procedures, Draft, July 1987.s